3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230205

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0176 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR POS testcase 9.3.4.4.19s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add POS test case 9.3.4.4.19s to the IWD_22wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.3.4.4.19s required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.4.19s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Verification Test Summary	2

2	Changes required to verify test case 9.3.4.4.19s	2

2.1 Correction to f_TC_9_3_4_4_MultiRTT_NR5GC_TestBody	2

3	Branches executed	3

4	Execution Log Files	3

4.1	Mediatek MT6879 UE	3

5	References	4

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of POS test case 9.3.4.4.19s which is part of the POS test suite in iWD_TTCN3-B2022-09_D22wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	9.3.4.4.19s

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Mediatek MT6879 UE

Verification Status:	PASS

# Changes required to verify test case 9.3.4.4.19s

## 2.1 Correction to f_TC_9_3_4_4_MultiRTT_NR5GC_TestBody

| Function name | f_TC_9_3_4_4_MultiRTT_NR5GC_TestBody |
| --- | --- |
| Reason for change | The template cr_LppProvideLocationInformation_MultiRTT used to be compared o the LPP Provide Location Information message from the UE leads to wrong “LPP Message do not match to template” fail verdicts, even if UE reports measurements. The template cr_LppProvideLocationInformation_MultiRTT requires the cr_CommonIEsProvideLocationInformation to be present, which is not the case for certain UEs. This lead into the wrong fail verdicts described above. |
| Summary of change | Change the way the LPP Provide Locatrion information message is matched to a template, in a similar way as done for TC 9.3.4.2.19s. |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, cr_LppProvideLocationInformation_MultiRTT(tsc_POS_TransactionNumber));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 9.3.4.4 Step 5");

//@siclog "Step 5a" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 5

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber)));

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.nr_Multi_RTT_ProvideLocationInformation_r16)) {

if (match (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.nr_Multi_RTT_ProvideLocationInformation_r16, cr_NR_Multi_RTT_ProvideLocationInformation)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 9.3.4.4 Step 5");

}

else {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Wrong Multi-RTT Location Information");

}

}

else {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Multi-RTT Location Information not present");

}

//@siclog "Step 5a" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 5

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);<<SKIPPED CODE>>

# Branches executed

## This POS TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n71” using NIA1 Integrity and NEA1 ciphering algorithms

# Execution Log Files

## Mediatek MT6879 UE

The Mediatek MT6879 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_9_3_4_4_19s_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230011: Supporting information for addition of NR POS testcase 9.3.4.4.19s |
| --- | --- |