3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230262

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3009 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MDT testcase 8.1.6.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the configuration of NR Cell 1 SIB5, E-UTRA Cell 1 is the high priority Inter-RAT cell and the power level setting in T1 will satisify the I-RAT cell reselection criteria as defined in TS 38.304. Srxlevl = -90(Qrxlevmeas) - (-110) (q-RxLevMin) = 20 dB (threshX-High). Considering the SS signal level uncertainty, UE may reselect to E-UTRA Cell 1 in step 2 and then no response the paging in NR cell in Step 4. #draft proseCR shall be raised in RAN5#99 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed power level of EUTRA cell at step 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | According to the configuration of NR Cell 1 SIB5, E-UTRA Cell 1 is the high priority Inter-RAT cell and the power level setting in T1 will satisify the I-RAT cell reselection criteria as defined in TS 38.304. Srxlevl = -90(Qrxlevmeas) - (-110) (q-RxLevMin) = 20 dB (threshX-High). Considering the SS signal level uncertainty, UE may reselect to E-UTRA Cell 1 in step 2 and then no response the paging in NR cell in Step 4. #draft proseCR shall be raised in RAN5#99 |
| Summary of change | Changed power level of EUTRA cell at step 1. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //Anritsu TTCN CR R5s230112 //var template (value) B28_Type v_EUTRA_CellId; //var template (value) CoOrd_EUTRASysInfo_Type v_CoOrd_SysInfo := f_EUTRA_CoOrdSysInfo(eutra_Cell1); // @sic R5s220801 sic@ //v_EUTRA_CellId := int2bit(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1),28); //v_CoOrd_SysInfo.CellId := v_EUTRA_CellId; //f_EUTRA_NR_SendSysinfoCoOrd(NR, cs_EUTRA_NR_CoOrdSysInfo_Eutra({v_CoOrd_SysInfo})); f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, -90, -90); //FR2 is FFS }

After Change:

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //Anritsu TTCN CR R5s230112 //var template (value) B28_Type v_EUTRA_CellId; //var template (value) CoOrd_EUTRASysInfo_Type v_CoOrd_SysInfo := f_EUTRA_CoOrdSysInfo(eutra_Cell1); // @sic R5s220801 sic@ //v_EUTRA_CellId := int2bit(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1),28); //v_CoOrd_SysInfo.CellId := v_EUTRA_CellId; //f_EUTRA_NR_SendSysinfoCoOrd(NR, cs_EUTRA_NR_CoOrdSysInfo_Eutra({v_CoOrd_SysInfo})); f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, -96, -96); //FR2 is FFS }