3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230513

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0186 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR_ConfigUlSrsMultiRTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After R5-233395, the function f_NR_ConfigUlSrsMultiRTT includes a Cell Config Request message based on the template cas_NR_CellConfig_REQ. Such template populates several IEs that were already configured in the initial Cell Config Req message, adding then duplicate configurations. This CR proposes to use a new template different from cas_NR_CellConfig_REQ that takes care of SRS configuration only, removing the extra content that is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New template cs_NR_SRS_Config of type NR_UplinkBWP_Type is created and used in f_NR_ConfigUlSrsMultiRTT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN is not optimized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.19s, 9.3.4.2.19s, 9.3.4.4.19s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Adding cs_NR_SRS_Config template	2

2.2 Correction to f_NR_ConfigUlSrsMultiRTT	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Adding cs_NR_SRS_Config template

| Function name | NR_CellCfg_Templates |
| --- | --- |
| Reason for change | After R5-233395, the function f_NR_ConfigUlSrsMultiRTT includes a Cell Config Request message based on the template cas_NR_CellConfig_REQ. Such template populates several IEs that were already configured in the initial Cell Config Req message, adding then duplicate configurations. This CR proposes to use another template different from cas_NR_CellConfig_REQ that takes care of SRS configuration only, removing the extra content that is not needed. |
| Summary of change | Create the template cs_NR_SRS_Config for configuring SRS only |
| TTCN module | NR_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

template(value)  NR_UplinkBWPs_Type cs_NR_UplinkBWPs(NR_UplinkBWP_List_Type  p_NR_UplinkBWP_List) :=

{ /* @status    APPROVED (NR5GC) */

ActiveBWP  := omit,

BwpArray   := p_NR_UplinkBWP_List,

InitialBWP_RedCap := omit

};

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

template(value)  NR_UplinkBWPs_Type cs_NR_UplinkBWPs(NR_UplinkBWP_List_Type  p_NR_UplinkBWP_List) :=

{ /* @status    APPROVED (NR5GC) */

ActiveBWP  := omit,

BwpArray   := p_NR_UplinkBWP_List,

InitialBWP_RedCap := omit

};

template (value) NR_UplinkBWP_Type cs_NR_SRS_Config (NR_CellInfo_Type  p_NR_CellInfo) :=

{

Id := p_NR_CellInfo.PhysicalParameters.UL_BWPs[0].Id,

Common := omit,

Dedicated := {

R15 := {

pucch_Config := omit,

pusch_Config := omit,

configuredGrantConfig := omit,

srs_Config:={

setup := p_NR_CellInfo.PhysicalParameters.UL_BWPs[0].Dedicated.R15.srs_Config.setup

},

beamFailureRecoveryConfig := omit,

sl_PUCCH_Config_r16 :=omit,

cp_ExtensionC2_r16 := omit,

cp_ExtensionC3_r16 := omit,

useInterlacePUCCH_PUSCH_r16 := omit,

pucch_ConfigurationList_r16 := omit,

lbt_FailureRecoveryConfig_r16 := omit,

configuredGrantConfigToAddModList_r16 := omit,

configuredGrantConfigToReleaseList_r16 := omit,

configuredGrantConfigType2DeactivationStateList_r16 := omit,

ul_TCI_StateList_r17 := omit,

ul_powerControl_r17 := omit,

pucch_ConfigurationListMulticast1_r17 := omit,

pucch_ConfigurationListMulticast2_r17 := omit,

pucch_ConfigMulticast1_r17 := omit,

pucch_ConfigMulticast2_r17 := omit

}

}

};

<<SKIPPED CODE>>

## 2.2 Correction to f_NR_ConfigUlSrsMultiRTT

| Function name | f_NR_ConfigUlSrsMultiRTT |
| --- | --- |
| Reason for change | After R5-233395, the function f_NR_ConfigUlSrsMultiRTT includes a Cell Config Request message based on the template cas_NR_CellConfig_REQ. Such template populates several IEs that were already configured in the initial Cell Config Req message, adding then duplicate configurations. This CR proposes to use another template different from cas_NR_CellConfig_REQ that takes care of SRS configuration only, removing the extra content that is not needed. |
| Summary of change | Use the template cs_NR_SRS_Config (from point 1.1) and cads_NR_UplinkBWP_ConfigCommon_REQ instead of cas_NR_CellConfig_REQ |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_NR_ConfigUlSrsMultiRTT (NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC

{ //@sic R5-233395 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(p_NR_CellId);

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) SRS_PeriodicityAndOffset_r16 v_SRS_PeriodicityAndOffset;

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var boolean v_IsFDD := f_NR_CellInfo_GetIsFDD(p_NR_CellId);

var NR_CellInfo_Type v_NR_CellInfo;

var template (value) NR_SYSTEM_CTRL_REQ v_NR_SYSTEM_CTRL_REQ;

if(f_NR_CellInfo_GetIsFR1(p_NR_CellId))

{

if ((pc_halfDuplexFDD_TypeA_RedCap_r17 or not(v_IsFDD)) and (v_SCS == kHz15)) {

v_SRS_PeriodicityAndOffset := {sl160 := 23};

}

else if (not(v_IsFDD) and (v_SCS == kHz30)) {

v_SRS_PeriodicityAndOffset := {sl160 := 27};

}

else {

v_SRS_PeriodicityAndOffset := {sl160 := 20};

}

}

else

{

v_SRS_PeriodicityAndOffset := {sl1280 := 163};

}

v_SpCellConfig.spCellConfigDedicated := cs_NR_ServingCellConfigDef(-,-,-, cs_NR_UplinkConfigSrs(cs_SRS_Config_Pos(v_SRS_PeriodicityAndOffset)));

v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG,

-,-,-,-,

v_SpCellConfig);

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)));

f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext);

//Store UL-SRS in cell info

f_NR_CellInfo_SetUL_SRS(p_NR_CellId, cs_SRS_Config_Pos(v_SRS_PeriodicityAndOffset), 0);

//Configure SS with SL-SRS

v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId);

v_NR_SYSTEM_CTRL_REQ := cas_NR_CellConfig_REQ(p_NR_CellId,

v_NR_CellInfo,

cs_TimingInfo_Now,

-,

cs_NR_CellConfigPhysicalLayerUplink(v_NR_CellInfo));

f_NR_SS_CommonCellConfig(p_NR_CellId,

v_NR_SYSTEM_CTRL_REQ);

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_NR_ConfigUlSrsMultiRTT (NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC

{ //@sic R5-233395 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(p_NR_CellId);

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) SRS_PeriodicityAndOffset_r16 v_SRS_PeriodicityAndOffset;

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var boolean v_IsFDD := f_NR_CellInfo_GetIsFDD(p_NR_CellId);

var NR_CellInfo_Type v_NR_CellInfo;

var template (value) NR_SYSTEM_CTRL_REQ v_NR_SYSTEM_CTRL_REQ;

var template (value) NR_UplinkBWP_List_Type v_BwpArrayList;

if(f_NR_CellInfo_GetIsFR1(p_NR_CellId))

{

if ((pc_halfDuplexFDD_TypeA_RedCap_r17 or not(v_IsFDD)) and (v_SCS == kHz15)) {

v_SRS_PeriodicityAndOffset := {sl160 := 23};

}

else if (not(v_IsFDD) and (v_SCS == kHz30)) {

v_SRS_PeriodicityAndOffset := {sl160 := 27};

}

else {

v_SRS_PeriodicityAndOffset := {sl160 := 20};

}

}

else

{

v_SRS_PeriodicityAndOffset := {sl1280 := 163};

}

v_SpCellConfig.spCellConfigDedicated := cs_NR_ServingCellConfigDef(-,-,-, cs_NR_UplinkConfigSrs(cs_SRS_Config_Pos(v_SRS_PeriodicityAndOffset)));

v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG,

-,-,-,-,

v_SpCellConfig);

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)));

f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext);

//Store UL-SRS in cell info

f_NR_CellInfo_SetUL_SRS(p_NR_CellId, cs_SRS_Config_Pos(v_SRS_PeriodicityAndOffset), 0);

//Configure SS with SL-SRS

v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId);

v_BwpArrayList[0] := cs_NR_SRS_Config(v_NR_CellInfo);

v_NR_SYSTEM_CTRL_REQ := cads_NR_UplinkBWP_ConfigCommon_REQ(p_NR_CellId, cs_TimingInfo_Now, v_BwpArrayList);

f_NR_SS_CommonCellConfig(p_NR_CellId,

v_NR_SYSTEM_CTRL_REQ);

}

<<SKIPPED CODE>>