3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230692

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3199 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC R17 eNS testcase 9.1.12.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 9.1.12.1 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.1.12.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required.	3

2.1	Change 1	3

3	Branches executed.	5

4	Execution Log Files	5

4.1	Samsung Exynos2400 UE	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.12.1 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.1.12.1

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos2400 UE

Verification Status:	PASS

# Corrections required.

## Change 1

| Function name | fl_TC_9_1_12_1_TestBody () |
| --- | --- |
| Reason for change | - Registration procedure does not happen during the preamble due to state 0-A. In test body, initial registration would trigger without initial security. #draft proseCR would be raised for RAN5#101 meeting - ER-NSSAI bit to be enable for the test procedure |
| Summary of change | - Changed the initial security to no initial security. - Added check for ER-NSSAI bit |
| TTCN module | 9.1.12\ NSAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_12_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); …. //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message including ER-NSSAI bit? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_ERNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI_Step12, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_ExtdRejectedNSSAI); ….. }//End of fl_TC_9_1_12_1_Body }

After Change:

function fl_TC_9_1_12_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); …. //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message including ER-NSSAI bit? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF,Initial_NoSecurity); if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_ERNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "ERNSSAI not present in GMM capability"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI_Step12, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_ExtdRejectedNSSAI); ….. }//End of fl_TC_9_1_12_1_Body }

# Branches executed.

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Samsung Exynos2400 UE

The Samsung Exynos2400 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_1_12_1_LOG.xml

- PICS/PIXIT parameter file:

TC_9_1_12_1_PIXIT.txt

# References

R5s230693: Supporting information for addition of NR5GC R17 eNS testcase 9.1.12.1 in FR1