3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230845

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3187 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on selection expressions in control modules and applicability conditions for NR Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inconsistencies between TTCN and prose regarding test case selection expressions in NR Test Suites remain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction for the selection expressions generating control modules on several ATSs listed in clauses 1.1, 1.2, 1.3 and 1.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent conditions in the selection expressions for several NR test cases will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ENDC, NR5GC and NR5GC_IRAT Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	2

1	List of changes	3

1.1	On ENDC Test Suite	3

1.2	On NR5GC Test Suite	4

1.3	On NR5GC_IRAT Test Suite	5

1.4	TTCN issues affecting only selection expressions.	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# 1	List of changes

## 1.1	On ENDC Test Suite

The following conditions set in each ENDC Testsuite in ttcn shall be updated to its value specified in Prose as shown below. This can affect control modules and selection expressions.

| Testsuite | Spec | Testcase | Condition |  |
| --- | --- | --- | --- | --- |
|  |  |  | Prose | value in ttcn |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_1_3_2b_ENDC | C175 | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_1_3_3_ENDC | C53 | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_2_3_2_ENDC | C07A | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_2_3_4_ENDC | C07A | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_2_3_5a_ENDC | C07A | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_3_1_2_ENDC | C08A | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_3_5_3_ENDC | C01 | R |
| ENDC | 38523-2 Table 4.1-2a | TC_7_1_3_5_5_ENDC | C62 | R |
| ENDC | 38523-2 Table 4.1-3a | TC_8_2_2_4_1_ENDC | C01 | R |
| ENDC | 38523-2 Table 4.1-3a | TC_8_2_6_3_1_ENDC | C255 | C225 |

example of change

- in module ENDC_Testsuite.ttcn

Before Change:

if (f_NR_ExecutionGuideline("TC_8_2_6_3_1_ENDC") and f_SelectionExpr(NR_C225)) { execute(TC_8_2_6_3_1_ENDC()); }

After Change:

if (f_NR_ExecutionGuideline("TC_8_2_6_3_1_ENDC") and f_SelectionExpr(NR_C255)) { execute(TC_8_2_6_3_1_ENDC()); }

- in module NR_38523_SelectionExpressions.ttcn

Added condition:

.. case(NR_C52){ //UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement v_ApplCond := pc_5GCN and pc_eventA_MeasAndReport and pc_intraAndInterF_MeasAndReport and (pc_csi_RSRP_AndRSRQ_MeasWithSSB or pc_csi_RSRP_AndRSRQ_MeasWithoutSSB); } case(NR_C53){ // UEs supporting 5GS and Logical Channel SR-Delay Timer v_ApplCond := pc_logicalChannelSR_DelayTimer; } case(NR_C54){ //UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback v_ApplCond := pc_5GCN and (pc_eFDD or pc_eTDD) and pc_VoLTE and pc_EPS_fallback; } ..

## 1.2	On NR5GC Test Suite

The following conditions set in each NR5GC Testsuite in ttcn shall be updated to its value specified in Prose as shown below. This can affect control modules and selection expressions.

| Testsuite | Spec | Testcase | Condition |  |
| --- | --- | --- | --- | --- |
|  |  |  | Prose | value in ttcn |
| NR5GC | 38523-2 Table 4.1-1a | TC_6_5_2_4_NR5GC | C168 | C132 |
| NR5GC | 38523-2 Table 4.1-2a | TC_7_1_1_3_3_NR5GC | C53 | R |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_1_7_1_NR5GC | C41 | C44 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_1_7_2_NR5GC | C42 | C45 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_1_7_3_NR5GC | C43 | C46 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_4_1_NR5GC | C116 | C21 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_4_2_NR5GC | C115 | C21 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_4_3_NR5GC | C117 | C21 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_4_4_4_NR5GC | C115 | C21 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_6_1_2_1_NR5GC | C123 | C21 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_6_1_2_15_NR5GC | C267 | C268 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_6_1_2_3_NR5GC | C123 | C21 |
| NR5GC | 38523-2 Table 4.1-3a | TC_8_1_6_1_2_9_NR5GC | C124 | C129 |

example of change

- in module NR5GC_Testsuite.ttcn

Before Change:

if (f_NR_ExecutionGuideline("TC_8_1_6_1_2_9_NR5GC") and f_SelectionExpr(NR_C129, false)) { execute(TC_8_1_6_1_2_9_NR5GC()); }

After Change:

if (f_NR_ExecutionGuideline("TC_8_1_6_1_2_9_NR5GC") and f_SelectionExpr(NR_C124, false)) { execute(TC_8_1_6_1_2_9_NR5GC()); }

- in module NR_38523_SelectionExpressions.ttcn

Added condition:

.. case(NR_C116){ // UEs supporting 5G Core and conditional handover and 2 trigger events for same execution condition v_ApplCond := pc_5GCN and pc_condHandover_r16 and pc_condHandoverTwoTriggerEvents_r16; // @sic R5-213513 sic@ } case(NR_C117){ // UEs supporting 5G Core and conditional handover and conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover v_ApplCond := pc_5GCN and pc_condHandover_r16 and pc_condHandoverFailure_r16; } case(NR_C118){ // @sic R5-225298 sic@ // UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band contiguous CA and UL NR CA with 2 carriers and two PUCCH group in CA with a same numerology v_ApplCond := pc_longDRX_Cycle and pc_DRX_Adaptation and pc_scellDormancyOutsideActiveTime_r16 and pc_UL_NR_CA_2CC and pc_twoPUCCH_group and (pc_IntraBand_Contiguous_CA_WithinFR1 or pc_IntraBand_Contiguous_CA_WithinFR2); } ..

## 1.3	On NR5GC_IRAT Test Suite

The following conditions set in each NR5GC_IRAT Testsuite in ttcn shall be updated to its value specified in Prose as shown below.

| Testsuite | Spec | Testcase | Condition |  |
| --- | --- | --- | --- | --- |
|  |  |  | Prose | value in ttcn |
| NR5GC_IRAT | 38523-2 Table 4.1-3a | TC_8_1_1_3_4_NR5GC | C26 | C32 |
| NR5GC_IRAT | 38523-2 Table 4.1-3a | TC_8_2_3_17_3_NEDC | C268 | C267 |

- in module NR5GC_IRAT_Testsuite.ttcn

Before Change:

.. if (f_NR_ExecutionGuideline("TC_8_1_1_3_4_NR5GC") and f_SelectionExpr(NR_C32)) { execute(TC_8_1_1_3_4_NR5GC()); } .. if (f_NR_ExecutionGuideline("TC_8_2_3_17_3_NEDC") and f_SelectionExpr(NR_C267)) { execute(TC_8_2_3_17_3_NEDC()); } ..

After Change:

.. if (f_NR_ExecutionGuideline("TC_8_1_1_3_4_NR5GC") and f_SelectionExpr(NR_C26)) { execute(TC_8_1_1_3_4_NR5GC()); } .. if (f_NR_ExecutionGuideline("TC_8_2_3_17_3_NEDC") and f_SelectionExpr(NR_C268)) { execute(TC_8_2_3_17_3_NEDC()); } ..

## 1.4	TTCN issues affecting only selection expressions.

- in module NR_38523_SelectionExpressions.ttcn

| Condition |  | Testcase selection expression |
| --- | --- | --- |
| C81 | TTCN | (pc_IntraBand_Contiguous_CA_WithinFR1 or pc_IntraBand_Contiguous_CA_WithinFR2) and pc_UL_NR_CA_2CC |
|  | Expected | pc_5GCN and (pc_IntraBand_Contiguous_CA_WithinFR1 or pc_IntraBand_Contiguous_CA_WithinFR2) and pc_UL_NR_CA_2CC |
| C82 | TTCN | (pc_InterBand_CA_WithinFR1 or pc_InterBand_CA_WithinFR2 or pc_InterBand_CA_BetweenFR1_FR2) and pc_UL_NR_CA_2CC |
|  | Expected | pc_5GCN and (pc_InterBand_CA_WithinFR1 or pc_InterBand_CA_WithinFR2 or pc_InterBand_CA_BetweenFR1_FR2) and pc_UL_NR_CA_2CC |
| C83 | TTCN | (pc_IntraBand_NonContiguous_CA_WithinFR1 or pc_IntraBand_NonContiguous_CA_WithinFR2) and pc_UL_NR_CA_2CC |
|  | Expected | pc_5GCN and (pc_IntraBand_NonContiguous_CA_WithinFR1 or pc_IntraBand_NonContiguous_CA_WithinFR2) and pc_UL_NR_CA_2CC |
| C85A | TTCN | (pc_5GCN and pc_TransferEmergencyPDUS1toN1noN26) and (pc_nrFDD or pc_nrTDD) and pc_EPS_IMS_EmergencyCall and pc_NR_5GC_EmergencyServices |
|  | Expected | (pc_5GCN and pc_TransferEmergencyPDUS1toN1noN26) and (pc_eFDD or pc_eTDD) and pc_EPS_IMS_EmergencyCall and pc_NR_5GC_EmergencyServices |
| C85B | TTCN | (pc_5GCN and pc_TransferEmergencyPDUN1toS1noN26) and (pc_nrFDD or pc_nrTDD) and pc_EPS_IMS_EmergencyCall and pc_voiceOverNR |
|  | Expected | (pc_5GCN and pc_TransferEmergencyPDUN1toS1noN26) and (pc_eFDD or pc_eTDD) and pc_EPS_IMS_EmergencyCall and pc_voiceOverNR |
| C93 | TTCN | pc_EN_DC and pc_eventA_MeasAndReport and pc_intraAndInterF_MeasAndReport and ((pc_nrFDD_MultiBand or pc_nrTDD_MultiBand) or (pc_nrFDD or pc_nrTDD)) |
|  | Expected | pc_EN_DC and pc_eventA_MeasAndReport and pc_intraAndInterF_MeasAndReport and (pc_nrFDD_MultiBand or pc_nrTDD_MultiBand or (pc_nrFDD and pc_nrTDD)) |
| C94 | TTCN | pc_5GCN and ((pc_nrFDD_MultiBand or pc_nrTDD_MultiBand) or (pc_nrFDD or pc_nrTDD)) |
|  | Expected | pc_5GCN and (pc_nrFDD_MultiBand or pc_nrTDD_MultiBand or (pc_nrFDD and pc_nrTDD)) |
| C99 | TTCN | pc_5GCN and (pc_nrFDD_MultiBand or pc_nrTDD_MultiBand) and (pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 or pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 or pc_eutra_EPC_HO_ToNR_TDD_FR2_r15) |
|  | Expected | pc_5GCN and (pc_eFDD or pc_eTDD) and (pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 or pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 or pc_eutra_EPC_HO_ToNR_TDD_FR2_r15) |
| C105 | TTCN | (pc_IntraBand_Contiguous_CA_WithinFR1 or pc_IntraBand_Contiguous_CA_WithinFR2) and pc_NR_ehc_r16 |
|  | Expected | (pc_um_WithLongSN or pc_um_WithShortSN) and pc_NR_ehc_r16 |
| C151 | TTCN | pc_EN_DC and (pc_SFTD_MeasPSCell_MRDC_FDD and pc_SFTD_MeasPSCell_MRDC_TDD) and (pc_SFTD_MeasNR_Cell_FDD or pc_SFTD_MeasNR_Cell_TDD) |
|  | Expected | pc_EN_DC and (pc_SFTD_MeasPSCell_MRDC_FDD or pc_SFTD_MeasPSCell_MRDC_TDD) and (pc_SFTD_MeasNR_Cell_FDD or pc_SFTD_MeasNR_Cell_TDD) |
| C156 | TTCN | pc_5GCN and (pc_InterBand_ENDC_IncludingFR1_FR2 or pc_InterBand_NRDC_BetweenFR1_FR2 or pc_InterBand_CA_BetweenFR1_FR2) and pc_inactiveState and pc_directMCG_SCellActivationResume_r16 |
|  | Expected | pc_5GCN and (pc_InterBand_CA_WithinFR1 or pc_InterBand_CA_WithinFR2) and pc_inactiveState and pc_directMCG_SCellActivationResume_r16 |
| C170 | TTCN | pc_5GCN and (pc_eFDD or pc_eTDD) and pc_5GS_IMS_eCall_Only |
|  | Expected | pc_5GCN and (pc_eFDD or pc_eTDD) and pc_5GS_IMS_eCall_Only and pc_eCall_manual_Initiated |
| C171 | TTCN | pc_5GCN and (pc_eFDD or pc_eTDD) and pc_5GS_IMS_eCall_Only and pc_eCall_Test_Call |
|  | Expected | pc_5GCN and (pc_eFDD or pc_eTDD) and pc_5GS_IMS_eCall_Only and pc_eCall_manual_Initiated and pc_eCall_Test_Call |
| C173 | TTCN | pc_5GCN and (pc_eFDD or pc_eTDD) and pc_IMS_emergency_call |
|  | Expected | pc_5GCN and (pc_eFDD or pc_eTDD) and pc_NG114_v2_0 |
| C189 | TTCN | pc_5GCN and (pc_UTRA or pc_GERAN) and pc_5GS_IMS_eCall_Only and pc_eCall_manual_Initiated |
|  | Expected | pc_5GCN and (pc_UTRA or pc_GERAN) and pc_5GS_IMS_eCall_Capable and pc_eCall_manual_Initiated |
| C199 | TTCN | pc_EN_DC and pc_directSCG_SCellActivation_r16 and (pc_IntraBand_Contiguous_ENDC or pc_IntraBand_Contiguous_CA_WithinFR2) |
|  | Expected | pc_EN_DC and pc_directSCG_SCellActivation_r16 and (pc_IntraBand_Contiguous_CA_WithinFR1 or pc_IntraBand_Contiguous_CA_WithinFR2) |
| C201 | TTCN | pc_EN_DC and pc_directSCG_SCellActivation_r16 and (pc_InterBand_ENDC_IncludingFR1_FR2 or pc_InterBand_NRDC_BetweenFR1_FR2 or pc_InterBand_CA_BetweenFR1_FR2) |
|  | Expected | pc_EN_DC and pc_directSCG_SCellActivation_r16 and (pc_InterBand_CA_WithinFR1 or pc_InterBand_CA_WithinFR2 or pc_InterBand_CA_BetweenFR1_FR2) |
| C223 | TTCN | pc_5GCN and (pc_InterBand_CA_WithinFR1 or pc_InterBand_CA_WithinFR2 or pc_InterBand_CA_BetweenFR1_FR2) and pc_inactiveState and pc_directSCG_SCellActivationResume_r16 and pc_InterBand_NRDC_BetweenFR1_FR2 |
|  | Expected | pc_5GCN and (pc_InterBand_CA_WithinFR1 or pc_InterBand_CA_WithinFR2) and pc_inactiveState and pc_directSCG_SCellActivationResume_r16 and pc_InterBand_NRDC_BetweenFR1_FR2 |
| C225 | TTCN | pc_EN_DC and pc_idleInactiveNR_MeasReport |
|  | Expected | pc_EN_DC and (pc_nrIdleInactiveNRFR1_MeasReport or pc_nrIdleInactiveNRFR2_MeasReport) |
| C250 | TTCN | pc_NE_DC and pc_inactiveState and pc_multiple_CEF_Report_r17 |
|  | Expected | pc_5GCN and pc_multiple_CEF_Report_r17 |