3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230474

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3110 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR emergency services test case 11.4.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the step 2-4, when UE send TAU request to the LTE, the count is incorrectly calculated as previous count is not updated from NR. Function f_EUTRA38_TAUReqFromN1 returns NAS count which is return value from f_EUTRA_RRC_ConnEst_DefWithNas and also, 5G context is mapped to 4G at the end of the f_EUTRA38_TAUReqFromN1. Therefore, at the step 5-8, latest NAS count is applied to generate based 5G context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At the step 5-8 use latest NAS count instead of old LTE value from preamble |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_11_4_12_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | At the step 2-4, when UE send TAU request to the LTE, the count is incorrectly calculated as previous count is not updated from NR. Function f_EUTRA38_TAUReqFromN1 returns NAS count which is return value from f_EUTRA_RRC_ConnEst_DefWithNas and also, 5G context is mapped to 4G at the end of the f_EUTRA38_TAUReqFromN1. Therefore, at the step 5-8, latest NAS count is applied to generate based 5G context. |
| Summary of change | At the step 5-8 use latest NAS count instead of old LTE value from preamble |
| TTCN module | Emergency_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Alternative solution implemented: store the 5G UL NAS count received in the TAU Request message rather than estimating the next EPS count value. |

Before Change

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_AnyproSeCap('58'O))){ // @sic R5s221058 sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability indication is not correct at step 4"); } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); }; v_NasCount := v_NasInd.SecurityProtection.NasCount; //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ } }

After Change

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; var NasCountInfo_Type v_NASCountInfo; //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_AnyproSeCap('58'O))){ // @sic R5s221058 sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability indication is not correct at step 4"); } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); }; v_NASCountInfo := f_EUTRA_NB_SS_NAS_CountGet(NASCTRL); //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NASCountInfo.UL); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ } }

#### MCC160 Implementation

function f_DecipherAndDecodeNasPdu_EUTRA38(inout NasSecurity_Type p_NasSecurityByRef,

octetstring p_NAS_DedicatedInformation) runs on NASEMU_EUTRA38_IRAT_PTC return template (value) NasEmu_DecodingInfo_Type

{

var template (value) NasEmu_DecodingInfo_Type v_DecodingInfo := cs_DecodingInfo_Init; // NasIndication := omit, IntegrityError := false

var NasEmu_DecodedNasPduUL_Type v_DecodedNasPduUL;

var octetstring v_IntegrityProtectedOctets;

var SECURITY_PROTECTED_NAS_MESSAGE v_SecurityProtectedNasMsg;

var MessageAuthenticationCode v_ContainedMac;

var NasEmu_SN_Type v_SequenceNumber;

var NAS_Message v_EncodedNasPdu;

var NasCount_Type v_CountUL;

var SecurityHeaderType v_SecurityHeader;

var NASEmu_CoOrd_CNF v_NASEmu_CoOrd_CNF;

// Decode NAS message: message is either SECURITY_PROTECTED_NAS_MESSAGE or non-protected NAS message

v_DecodedNasPduUL := f_NasEmu_DecodeMsg(p_NAS_DedicatedInformation);

if (ischosen(v_DecodedNasPduUL.Invalid)) {

return v_DecodingInfo;            // v_DecodingInfo is {omit, false}

}

if (ischosen(v_DecodedNasPduUL.Msg.sECURITY_PROTECTED_NAS_MESSAGE)) {     // SECURITY_PROTECTED_NAS_MESSAGE

v_SecurityProtectedNasMsg := v_DecodedNasPduUL.Msg.sECURITY_PROTECTED_NAS_MESSAGE;

v_SecurityHeader      := v_SecurityProtectedNasMsg.securityHeaderType;

v_ContainedMac        := v_SecurityProtectedNasMsg.messageAuthenticationCode;

v_SequenceNumber.Long := oct2int(v_SecurityProtectedNasMsg.sequenceNumber);

v_EncodedNasPdu       := v_SecurityProtectedNasMsg.nasMessage;

v_CountUL             := f_NasSecurity_EstimateCOUNT(p_NasSecurityByRef.NasCount.UL, v_SequenceNumber);

if ((not p_NasSecurityByRef.SecurityStarted) and (v_SecurityHeader != tsc_SHT_IntegrityProtected)) { /* @sic R5s100024 tsc_SHT_EnabledSecurityProtection replaced by tsc_SHT_IntegrityProtected sic@ */

/* Note: A UE is allowed to transmit a security protected Attach request (Security header set to '0001'B)

*       at the initial registration in case the USIM used has a valid EPS security context  */

f_NasEmulationError(__FILE__, __LINE__, "security protected PDU received but security is not started");

return v_DecodingInfo;            // v_DecodingInfo is {omit, false}

} else {

if (p_NasSecurityByRef.SecurityStarted) {

/* Note: acc. 24.301 cl. 4.4.1 "When both ciphering and integrity protection are activated, then the NAS message is first encrypted

and then the encrypted NAS message and the SN are integrity protected by calculating the MAC"

=> we apply integrity protection on the ciphered ciphering data */

// CHECK INTEGRITY

if (p_NasSecurityByRef.IntegrityCheck5G) {  /* Only set during 4G/5G InterSystem tests @sic 5G Integrity in 4G sic@ */

v_IntegrityProtectedOctets := v_SecurityProtectedNasMsg.sequenceNumber & v_EncodedNasPdu;

// ask the 5G NAS emulator to check the integrity of this message

NG.send (cs_NASEmu_CoOrd_REQ (v_IntegrityProtectedOctets, oct2int(v_SecurityProtectedNasMsg.sequenceNumber)));

NG.receive(cr_NASEmu_CoOrd_CNF) -> value v_NASEmu_CoOrd_CNF;

if (v_ContainedMac == v_NASEmu_CoOrd_CNF.MAC) {

// Sequence number is only one octet long

v_CountUL := '000000'O & v_SecurityProtectedNasMsg.sequenceNumber;

p_NasSecurityByRef.NasCount.UL := v_CountUL;   /* write back the latest 5G count value */

} else {

// UL NAS COUNT is not updated in case of integrity error

v_DecodingInfo.IntegrityError := true;

}

// This path will only be taken for the TAU Req which is using 5G integrity

// Decode NAS message

v_DecodedNasPduUL := f_NasEmu_DecodeMsg(v_EncodedNasPdu);

if (ischosen(v_DecodedNasPduUL.Invalid)) {

// Do nothing: v_DecodingInfo.NasIndication remains omit

} else {

v_DecodingInfo.NasIndication := cs_NAS_MSG_Indication(v_SecurityHeader,

v_CountUL,

v_DecodedNasPduUL.Msg);

}

return v_DecodingInfo;

} else { // just call the normal function as usual

return f_DecipherAndDecodeNasPdu(p_NasSecurityByRef, p_NAS_DedicatedInformation);

}

} else { // just call the normal function as usual

return f_DecipherAndDecodeNasPdu(p_NasSecurityByRef, p_NAS_DedicatedInformation);

}

}

} else { // just call the normal function as usual

return f_DecipherAndDecodeNasPdu(p_NasSecurityByRef, p_NAS_DedicatedInformation);

}

}