3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250059

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4884 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RACS test case 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At Step5, the total UL grant size of 8241 bytes provided is not sufficient when each layer headers, RLC status PDU, BSR and powerheadroom reports are considered. #associated proseCR(R5-251249) raised during RAN5#106 meeting - As per cl. 11.2 TS 36.331, 80ms RRC procedure delay is allowed for response UECapabiltiyEnquiry<>ULDedicatedMessageSegment. Currently, prose provides the grant after 15ms for FDD and 20ms for TDD. Since BSR and PHR report affect overall grant size, its better to define periodicity so that number of BSR reports(BSR timer 20ms) can be calculated accurately as it will affect grant size. #associated proseCR(R5-251249) raised during RAN5#106 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | - At step 5, modified single UL grant size of 225 bytes to 249 bytes. - Allocate UL grants periodically from 80 subframes after successful completion of step 4 & periodicity of 5ms is made common for FDD & TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS 	iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_5_5_1_EUTRA |
| --- | --- |
| Reason for change | - At Step5, the total UL grant size of 8241 bytes provided is not sufficient when each layer headers, RLC status PDU, BSR and powerheadroom reports are considered. #associated proseCR(R5-251249) raised during RAN5#106 meeting - As per cl. 11.2 TS 36.331, 80ms RRC procedure delay is allowed for response UECapabiltiyEnquiry<>ULDedicatedMessageSegment. Currently, prose provides the grant after 15ms for FDD and 20ms for TDD. Since BSR and PHR report affect overall grant size, its better to define periodicity so that number of BSR reports(BSR timer 20ms) can be calculated accurately as it will affect grant size. #associated proseCR(R5-251249) raised during RAN5#106 meeting |
| Summary of change | - At step 5, modified single UL grant size of 225 bytes to 249 bytes. - Allocate UL grants periodically from 80 subframes after successful completion of step 4 & periodicity of 5ms is made common for FDD & TDD. |
| TTCN module | LTE_A_PRO\8_5\RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC { /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */ var PhysCellId v_PhysicalCellIdentity_Cell1; var ShortMAC_I v_ShortMAC_I; var EUTRA_SecurityParams_Type v_AuthParams; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var SRB_COMMON_IND v_Received_DedicatedMessageSegment; var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded; var integer v_Index := 0; var SubFrameTiming_Type v_TimingInfoStep4; var SubFrameTiming_Type v_TimingInfoStep5; var SubFrameTiming_Type v_TimingInfoStep6; var integer v_GrantPeriod; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; f_EUTRA_Init(c1); // SKIPPED CODE //@siclog "Step 5" siclog@ // The SS allocates 17 UL grants periodically from 15(FDD)/20 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. if (v_EUTRA_FDD_TDD_Mode == FDD) { v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily. } else { v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 20); // 20 subframes = 1208 ms @sic R5-247692/R5s240685 sic@ v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily. } //@sic R5-231397 sic@ f_EUTRA_PeriodicCyclicULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep5), 9, // Imcs => Acc. 36.523-3, 4008 bits TBS 25, // Nprb => Acc. 36.523-3, 4008 bits TBS v_GrantPeriod, 16); //@sic R5s240175 sic@ //@sic R5s240175 sic@ v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,16*v_GrantPeriod); //@sic R5s240560 sic@ f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 18, 5); // Imcs & Nprb => Acc. 36.523-3, 225 bytes TBS @sic R5-247692/R5s240685 sic@ //@siclog "Step 6" siclog@ // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0)))); f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now); // SKIPPED CODE f_EUTRA_TestBody_Set(false); /* Postamble */ f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC { /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */ var PhysCellId v_PhysicalCellIdentity_Cell1; var ShortMAC_I v_ShortMAC_I; var EUTRA_SecurityParams_Type v_AuthParams; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var SRB_COMMON_IND v_Received_DedicatedMessageSegment; var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded; var integer v_Index := 0; var SubFrameTiming_Type v_TimingInfoStep4; var SubFrameTiming_Type v_TimingInfoStep5; var SubFrameTiming_Type v_TimingInfoStep6; var integer v_GrantPeriod := 5; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; f_EUTRA_Init(c1); // SKIPPED CODE //@siclog "Step 5" siclog@ // The SS allocates 17 UL grants periodically from 15(FDD)/20 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 80); //@sic R5-231397 sic@ f_EUTRA_PeriodicCyclicULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep5), 9, // Imcs => Acc. 36.523-3, 4008 bits TBS 25, // Nprb => Acc. 36.523-3, 4008 bits TBS v_GrantPeriod, 16); //@sic R5s240175 sic@ //@sic R5s240175 sic@ v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,16*v_GrantPeriod); //@sic R5s240560 sic@ f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 15, 7); // Imcs & Nprb => Acc. 36.523-3, 225 bytes TBS @sic R5-247692/R5s240685 sic@ //@siclog "Step 6" siclog@ // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0)))); f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now); // SKIPPED CODE f_EUTRA_TestBody_Set(false); /* Postamble */ f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }