3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250062

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3623 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current implementation of TTCN for 6.3.2.1, the function fl_TC_6_3_2_1_TestBodyf_DomainName_Encode f_DomainName_EncodeLabels is called. This encodes the DNN criterion incorrectly. According to 23.003 cl 9.1, Quote The encoding of the APN shall follow the Name Syntax defined in RFC 2181 [18], RFC 1035 [19] and RFC 1123 [20]. The APN consists of one or more labels. Each label is coded as a one octet length field followed by that number of octets coded as 8 bit ASCII characters. Following RFC 1035 [19] the labels shall consist only of the alphabetic characters (A-Z and a-z), digits (0-9) and the hyphen (-). Following RFC 1123 [20], the label shall begin and end with either an alphabetic character or a digit. The case of alphabetic characters is not significant. The APN is not terminated by a length byte of zero. NOTE 2: A length byte of zero is added by the SGSN/MME at the end of the APN before interrogating a DNS server. For the purpose of presentation, an APN is usually displayed as a string in which the labels are separated by dots (e.g. "Label1.Label2.Label3"). Unquote If the Label is “internet”, then according to the highlighted text above it shall be encoded as 08696E7465726E6574. Now according to 24.501 cl 9.11.3.51.3, Quote For "DNN", the criterion value field shall be encoded as a DNN length-value pair field. For "S-NSSAI SST", the criterion value field shall be encoded as one octet SST field. For "S-NSSAI SST and SD", the criterion value field shall be encoded as a sequence of one octet SST field and three octets SD field. The SST field shall be transmitted first. The DNN length-value pair field shall be encoded as a sequence of one octet DNN value length field and a DNN value field. The DNN value length field shall be transmitted first. The DNN value length field indicates the length in octets of the DNN value field. The DNN value field contains an APN as specified in 3GPP TS 23.003 [4]. Unquote Here, the interpretation of highlighted would be that : DNN value = 08696E7465726E6574 DNN length = Length of DNN value = Length of (08696E7465726E6574) = 09 Thus, the SORCMCI for criterion DNN would be encoded as DNN length concatenated with DNN value = 01+09 + 08696E7465726E6574 = 010908696E7465726E6574 This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the function f_DomainName_Encode and f_DomainName_EncodeLabels to address the above. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Parikshit Bhise
	Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_DomainName_EncodeLabels |
| --- | --- |
| Reason for change | In the current implementation of TTCN for 6.3.2.1, the function fl_TC_6_3_2_1_TestBodyf_DomainName_Encode f_DomainName_EncodeLabels is called. This encodes the DNN criterion incorrectly. According to 23.003 cl 9.1, Quote The encoding of the APN shall follow the Name Syntax defined in RFC 2181 [18], RFC 1035 [19] and RFC 1123 [20]. The APN consists of one or more labels. Each label is coded as a one octet length field followed by that number of octets coded as 8 bit ASCII characters. Following RFC 1035 [19] the labels shall consist only of the alphabetic characters (A-Z and a-z), digits (0-9) and the hyphen (-). Following RFC 1123 [20], the label shall begin and end with either an alphabetic character or a digit. The case of alphabetic characters is not significant. The APN is not terminated by a length byte of zero. NOTE 2: A length byte of zero is added by the SGSN/MME at the end of the APN before interrogating a DNS server. For the purpose of presentation, an APN is usually displayed as a string in which the labels are separated by dots (e.g. "Label1.Label2.Label3"). Unquote If the Label is “internet”, then according to the highlighted text above it shall be encoded as 08696E7465726E6574. Now according to 24.501 cl 9.11.3.51.3, Quote For "DNN", the criterion value field shall be encoded as a DNN length-value pair field. For "S-NSSAI SST", the criterion value field shall be encoded as one octet SST field. For "S-NSSAI SST and SD", the criterion value field shall be encoded as a sequence of one octet SST field and three octets SD field. The SST field shall be transmitted first. The DNN length-value pair field shall be encoded as a sequence of one octet DNN value length field and a DNN value field. The DNN value length field shall be transmitted first. The DNN value length field indicates the length in octets of the DNN value field. The DNN value field contains an APN as specified in 3GPP TS 23.003 [4]. Unquote Here, the interpretation of highlighted would be that : DNN value = 08696E7465726E6574 DNN length = Length of DNN value = Length of (08696E7465726E6574) = 09 Thus, the SORCMCI for criterion DNN would be encoded as DNN length concatenated with DNN value = 01+09 + 08696E7465726E6574 = 010908696E7465726E6574 This needs to be corrected. |
| Summary of change | Updated the implementation of f_DomainName_EncodeLabels. |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment |  |

Before Change

function f_DomainName_EncodeLabels(CharStringList_Type p_LabelList) return octetstring { var octetstring v_EncodedAPN := ''O; var octetstring v_LabelOctets; var integer v_Length; var integer i; for (i := 0; i < lengthof(p_LabelList); i := i + 1) { v_LabelOctets := char2oct(p_LabelList[i]); v_Length := lengthof(v_LabelOctets); v_EncodedAPN := v_EncodedAPN & int2oct(v_Length, 1) & v_LabelOctets; } return v_EncodedAPN; }

After Change

function f_DomainName_EncodeLabels(CharStringList_Type p_LabelList, boolean p_CriterionValueDNN := false) return octetstring { var octetstring v_EncodedAPN := ''O; var octetstring v_LabelOctets; var integer v_Length; var integer i; for (i := 0; i < lengthof(p_LabelList); i := i + 1) { v_LabelOctets := char2oct(p_LabelList[i]); v_Length := lengthof(v_LabelOctets); v_EncodedAPN := v_EncodedAPN & int2oct(v_Length, 1) & v_LabelOctets; } if (p_CriterionValueDNN) { v_Length := lengthof(v_EncodedAPN); v_EncodedAPN := int2oct(v_Length, 1) & v_EncodedAPN; } return v_EncodedAPN; }

## Change 2

| Function name | f_DomainName_Encode |
| --- | --- |
| Reason for change | Please see change 1 above. |
| Summary of change | Added p_CriterionValueDNN as additional argument to function , and passed it to function f_DomainName_EncodeLabels |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment |  |

Before Change

function f_DomainName_Encode(charstring p_DomainName) return octetstring { var CharStringList_Type v_LabelList := f_StringSplit(p_DomainName, {"."}); return f_DomainName_EncodeLabels(v_LabelList); }

After Change

function f_DomainName_Encode(charstring p_DomainName, boolean p_CriterionValueDNN := false) return octetstring { var CharStringList_Type v_LabelList := f_StringSplit(p_DomainName, {"."}); return f_DomainName_EncodeLabels(v_LabelList, p_CriterionValueDNN); }

## Change 3

| Function name | fl_TC_6_3_2_1_TestBody |
| --- | --- |
| Reason for change | Please see change 1 and 2 above |
| Summary of change | Updated the call to function f_DomainName_Encode and passed “true” as parameter. |
| TTCN module | SOR_CMCI_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_6_3_2_1_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; // var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233456 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var octetstring v_Length;//@sic R5s240594 sic@

After Change

function fl_TC_6_3_2_1_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; // var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233456 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet, true); var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var octetstring v_Length;//@sic R5s240594 sic@