3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250067

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4886 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 11.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of testcase 11.2.3, at Step 27, the function f_EUTRA_SwitchOnUEandStartIP f_EUTRA_ResetIPandIMS f_IP_Handling_StartPDN is called with PDN_1. However, prior to this, the PDN_1 is not stopped. This needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called the function f_IP_Handling_StopPDN(IP); before Step 27. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | Correction to function fl_TC_11_2_3_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 11.2.3, at Step 27, the function f_EUTRA_SwitchOnUEandStartIP f_EUTRA_ResetIPandIMS f_IP_Handling_StartPDN is called with PDN_1. However, prior to this, the PDN_1 is not stopped. This needs to be implemented. |
| Summary of change | Called the function f_IP_Handling_StopPDN(IP); before Step 27. |
| TTCN module | IMS_EmergencyCall.ttcn |
| MCC160 Comments |  |

Before Change:

function fl_TC_11_2_3_Body() runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NasInd; var GutiParameters_Type v_GutiParamsA := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParamsA.PLMN_Identity); var template (value) CellPowerList_Type v_CellPowerList; var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var EUTRA_SecurityParams_Type v_AuthParams_A := f_EUTRA_Security_Get(); var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (); var template (value) TrackingAreaId v_Tai := cr_TAI(v_PLMN, tsc_TAC1); var NAS_KsiValue v_NAS_KSIasme := v_AuthParams.KSIasme; var float v_EnterLimService := 60.0; var GutiParameters_Type v_GutiParamsB := f_EUTRA_CellInfo_GetGuti(eutra_CellB); var template LocAreaId v_LAI_A := omit; var NAS_Lac v_LAC_A := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA); var NAS_AttDetValue_Type v_AttachType; var boolean v_Detached := false; var EPS_BearerIdentity v_EpsBearerId; var EPS_BearerIdentity v_EpsDedicatedBearerId; timer t_WaitforDetach := 5.0; <CODE SKIPPED> //@siclog "Step 25" siclog@ //If pc_SwitchOnOff then the UE transmits a DETACH REQUEST with the Detach Type IE indicating "switch off". if (v_Detached == false) { // @sic R5-169119 sic@ f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellB, RRC_IDLE, -, cr_DetachType('1'B, ?)); // @sic R5-115675, R5s141339 change 1.3, R5s160347, R5-169119 sic@ } else { // UE is deregistered if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE (UT, false, CNF_REQUIRED); // @sic R5s180030 sic@ } else { f_UT_PowerOffUE (UT); } } // @sic R5s170053 sic@ //The SS configures: cell A as a " Serving Cell", cell B as a "Non-suitable "Off" cell". //@siclog "Step 26" siclog@ v_CellPowerList := { cs_CellPower(eutra_CellA, tsc_ServingCellRS_EPRE), cs_CellPower(eutra_CellB, tsc_NonSuitableOffCellRS_EPRE) }; f_EUTRA_SetCellPowerList(v_CellPowerList); //@siclog "Step 27" siclog@ //The UE is brought back to operation (i.e. powered up, switched on, and the USIM inserted if it was removed in step 24). f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); // @sic R5s120065 sic@ @sic R5s141339 change 1.3, R5s169119 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s160033 r1 sic@

After Change:

function fl_TC_11_2_3_Body() runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NasInd; var GutiParameters_Type v_GutiParamsA := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParamsA.PLMN_Identity); var template (value) CellPowerList_Type v_CellPowerList; var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var EUTRA_SecurityParams_Type v_AuthParams_A := f_EUTRA_Security_Get(); var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (); var template (value) TrackingAreaId v_Tai := cr_TAI(v_PLMN, tsc_TAC1); var NAS_KsiValue v_NAS_KSIasme := v_AuthParams.KSIasme; var float v_EnterLimService := 60.0; var GutiParameters_Type v_GutiParamsB := f_EUTRA_CellInfo_GetGuti(eutra_CellB); var template LocAreaId v_LAI_A := omit; var NAS_Lac v_LAC_A := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA); var NAS_AttDetValue_Type v_AttachType; var boolean v_Detached := false; var EPS_BearerIdentity v_EpsBearerId; var EPS_BearerIdentity v_EpsDedicatedBearerId; timer t_WaitforDetach := 5.0; <CODE SKIPPED> //@siclog "Step 25" siclog@ //If pc_SwitchOnOff then the UE transmits a DETACH REQUEST with the Detach Type IE indicating "switch off". if (v_Detached == false) { // @sic R5-169119 sic@ f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellB, RRC_IDLE, -, cr_DetachType('1'B, ?)); // @sic R5-115675, R5s141339 change 1.3, R5s160347, R5-169119 sic@ } else { // UE is deregistered if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE (UT, false, CNF_REQUIRED); // @sic R5s180030 sic@ } else { f_UT_PowerOffUE (UT); } } // @sic R5s170053 sic@ f_IP_Handling_StopPDN(IP); //The SS configures: cell A as a " Serving Cell", cell B as a "Non-suitable "Off" cell". //@siclog "Step 26" siclog@ v_CellPowerList := { cs_CellPower(eutra_CellA, tsc_ServingCellRS_EPRE), cs_CellPower(eutra_CellB, tsc_NonSuitableOffCellRS_EPRE) }; f_EUTRA_SetCellPowerList(v_CellPowerList); //@siclog "Step 27" siclog@ //The UE is brought back to operation (i.e. powered up, switched on, and the USIM inserted if it was removed in step 24). f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); // @sic R5s120065 sic@ @sic R5s141339 change 1.3, R5s169119 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s160033 r1 sic@