3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250089

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3636 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC TBS testcases 7.1.1.4.1.1 and 7.1.1.4.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per prose CR R5-244415_38.523-1 whenever TBS test cases goes beyond 20 PDCP SDUs for certain mcx Nprb values, it needs to ensure that >20 PDCP SDUs are still following the 1500 octet limitation.Current TTCN code is violating the prose which is leading to size of generated SDUs more than 1500 octets when No PDCP SDU>20 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update in function fl_NR_GetPdcpSduList_1TB () |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase incorrectly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.1.NR5GC, 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.4.NR5GC, 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

3	References	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_NR_GetPdcpSduList_1TB |
| --- | --- |
| Reason for change | When No of PDCP SDU’s > 20, incorrect octet conversion leading to PDCP SDU size more than 1500 octets. |
| Summary of change | Update in function fl_NR_GetPdcpSduList_1TB () |
| TTCN module | MAC_TC_Common_NR.ttcn |

Before Change:

function fl_NR_GetPdcpSduList_1TB(integer p_TBSize) return NR_PDCP_SDUList_Type {//@sic R5-227169 sic@ var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24200) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36272) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48344) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; //@sic R5s190927 sic@ } else if (p_TBSize <= 60416) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72488) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84560) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96632) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108704) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120776) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132848) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144920) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156992) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; //@sic R5-190676 R5s210884 sic@ } else if (p_TBSize <= 169064) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 181136) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 193208) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205280) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217352) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229424) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else if (p_TBSize <= 241496) { //@sic R5-244415 sic@ v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } else if (p_TBSize <= 253568) { //@sic R5-244415 sic@ v_N := 21; v_PdcpSduSize := (p_TBSize - 1568)/160; } else if (p_TBSize <= 265640) { //@sic R5-244415 sic@ v_N := 22; v_PdcpSduSize := (p_TBSize - 1640)/160; } else if (p_TBSize <= 277712) { //@sic R5-244415 sic@ v_N := 23; v_PdcpSduSize := (p_TBSize - 1712)/160; } else if (p_TBSize <= 289784){ //@sic R5-244415 sic@ v_N := 24; v_PdcpSduSize := (p_TBSize - 1784)/160; } else if (p_TBSize <= 301856) { //@sic R5-244415 sic@ v_N := 25; v_PdcpSduSize := (p_TBSize - 1856)/160; } else if (p_TBSize <= 313928){ //@sic R5-244415 sic@ v_N := 26; v_PdcpSduSize := (p_TBSize - 1928)/160; } else { //@sic R5-244415 sic@ v_N := 27; v_PdcpSduSize := (p_TBSize - 2000)/160; } } else //Redcap UE { if (p_TBSize <= 12112) { v_N := 1; v_PdcpSduSize := (p_TBSize - 112)/8; //@sic R5s230222 sic@ } else if (p_TBSize <= 24168) { v_N := 2; v_PdcpSduSize := (p_TBSize - 168)/16; } else if (p_TBSize <= 36224) { v_N := 3; v_PdcpSduSize := (p_TBSize - 224)/24; } else if (p_TBSize <= 48280) { v_N := 4; v_PdcpSduSize := (p_TBSize - 280)/32; } else if (p_TBSize <= 60336) { v_N := 5; v_PdcpSduSize := (p_TBSize - 336)/40; } else if (p_TBSize <= 72392) { v_N := 6; v_PdcpSduSize := (p_TBSize - 392)/48; } else if (p_TBSize <= 84448) { v_N := 7; v_PdcpSduSize := (p_TBSize - 448)/56; } else if (p_TBSize <= 96504) { v_N := 8; v_PdcpSduSize := (p_TBSize - 504)/64; } else if (p_TBSize <= 108560) { v_N := 9; v_PdcpSduSize := (p_TBSize - 560)/72; } else if (p_TBSize <= 120616) { v_N := 10; v_PdcpSduSize := (p_TBSize - 616)/80; } else if (p_TBSize <= 132672) { v_N := 11; v_PdcpSduSize := (p_TBSize - 672)/88; } else if (p_TBSize <= 144728) { v_N := 12; v_PdcpSduSize := (p_TBSize - 728)/96; } else if (p_TBSize <= 156784) { v_N := 13; v_PdcpSduSize := (p_TBSize - 784)/104; } else if (p_TBSize <= 168840) { v_N := 14; v_PdcpSduSize := (p_TBSize - 840)/112; } else if (p_TBSize <= 180896) { v_N := 15; v_PdcpSduSize := (p_TBSize - 896)/120; } else if (p_TBSize <= 192952) { v_N := 16; v_PdcpSduSize := (p_TBSize - 952)/128; } else if (p_TBSize <= 205008) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1008)/136; } else if (p_TBSize <= 217064) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1064)/144; } else if (p_TBSize <= 229120) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1120)/152; } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1176)/160; } } return f_NR_Build_N_PDCP_SDUList(v_N, v_PdcpSduSize); }

After Change:

function fl_NR_GetPdcpSduList_1TB(integer p_TBSize) return NR_PDCP_SDUList_Type {//@sic R5-227169 sic@ var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24200) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36272) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48344) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; //@sic R5s190927 sic@ } else if (p_TBSize <= 60416) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72488) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84560) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96632) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108704) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120776) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132848) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144920) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156992) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; //@sic R5-190676 R5s210884 sic@ } else if (p_TBSize <= 169064) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 181136) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 193208) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205280) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217352) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229424) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else if (p_TBSize <= 241496) { //@sic R5-244415 sic@ v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } else if (p_TBSize <= 253568) { //@sic R5-244415 sic@ v_N := 21; v_PdcpSduSize := (p_TBSize - 1568)/168; } else if (p_TBSize <= 265640) { //@sic R5-244415 sic@ v_N := 22; v_PdcpSduSize := (p_TBSize - 1640)/176; } else if (p_TBSize <= 277712) { //@sic R5-244415 sic@ v_N := 23; v_PdcpSduSize := (p_TBSize - 1712)/184; } else if (p_TBSize <= 289784){ //@sic R5-244415 sic@ v_N := 24; v_PdcpSduSize := (p_TBSize - 1784)/192; } else if (p_TBSize <= 301856) { //@sic R5-244415 sic@ v_N := 25; v_PdcpSduSize := (p_TBSize - 1856)/200; } else if (p_TBSize <= 313928){ //@sic R5-244415 sic@ v_N := 26; v_PdcpSduSize := (p_TBSize - 1928)/208; } else { //@sic R5-244415 sic@ v_N := 27; v_PdcpSduSize := (p_TBSize - 2000)/216; } } else //Redcap UE { if (p_TBSize <= 12112) { v_N := 1; v_PdcpSduSize := (p_TBSize - 112)/8; //@sic R5s230222 sic@ } else if (p_TBSize <= 24168) { v_N := 2; v_PdcpSduSize := (p_TBSize - 168)/16; } else if (p_TBSize <= 36224) { v_N := 3; v_PdcpSduSize := (p_TBSize - 224)/24; } else if (p_TBSize <= 48280) { v_N := 4; v_PdcpSduSize := (p_TBSize - 280)/32; } else if (p_TBSize <= 60336) { v_N := 5; v_PdcpSduSize := (p_TBSize - 336)/40; } else if (p_TBSize <= 72392) { v_N := 6; v_PdcpSduSize := (p_TBSize - 392)/48; } else if (p_TBSize <= 84448) { v_N := 7; v_PdcpSduSize := (p_TBSize - 448)/56; } else if (p_TBSize <= 96504) { v_N := 8; v_PdcpSduSize := (p_TBSize - 504)/64; } else if (p_TBSize <= 108560) { v_N := 9; v_PdcpSduSize := (p_TBSize - 560)/72; } else if (p_TBSize <= 120616) { v_N := 10; v_PdcpSduSize := (p_TBSize - 616)/80; } else if (p_TBSize <= 132672) { v_N := 11; v_PdcpSduSize := (p_TBSize - 672)/88; } else if (p_TBSize <= 144728) { v_N := 12; v_PdcpSduSize := (p_TBSize - 728)/96; } else if (p_TBSize <= 156784) { v_N := 13; v_PdcpSduSize := (p_TBSize - 784)/104; } else if (p_TBSize <= 168840) { v_N := 14; v_PdcpSduSize := (p_TBSize - 840)/112; } else if (p_TBSize <= 180896) { v_N := 15; v_PdcpSduSize := (p_TBSize - 896)/120; } else if (p_TBSize <= 192952) { v_N := 16; v_PdcpSduSize := (p_TBSize - 952)/128; } else if (p_TBSize <= 205008) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1008)/136; } else if (p_TBSize <= 217064) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1064)/144; } else if (p_TBSize <= 229120) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1120)/152; } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1176)/160; } } return f_NR_Build_N_PDCP_SDUList(v_N, v_PdcpSduSize); }

# References