3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250100

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3643 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 7.1.2.3.9a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 7.1.2.3.9a to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.2.3.9a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.9a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required to test case 7.1.2.3.9a	5

2.1	Change 1	5

2.2	Change 2	6

2.3	Change 3	7

2.4	Change 4	8

2.5	Change 5	10

2.6	Change 6	11

2.7	Change 7	12

2.8	Change 8	12

2.9	Change 9	13

2.10	Change 10	14

2.11	Change 11	15

2.12	Change 12	16

2.13	Change 13	19

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.2.3.9a which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.2.3.9a.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

# Corrections required to testcase 7.1.2.3.9a

## Change 1

| Function name | f_NR5GC_CellInfo_SetSIB19 |
| --- | --- |
| Reason for change | According to TS 38.508-1 Table 4.4.3.1.2-1, The NR-28 combination currently lists only SIB1 and SIB19. However, the way in which the TTCN architecture is done, this results in a RUNTIME exception in the function f_NR_GetSIB1_SiSchedulingInfo. Furthermore, according to TS 38.331 section 5.2.2.1, it is stated as below: “The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED. The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB19 (if UE is accessing NR via NTN access). The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20 and SIB21, regardless of the RRC state the UE is in. The UE shall ensure having a valid version of the posSIB requested by upper layers.” Hence, for the NR-28 combination, SIB2 is added. |
| Summary of change | For NR-28 combination, add SIB2 in the system information list, correction the index of SIB19 for NR-28 combination to overcome the RUNTIME exception. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_CellInfo_SetSIB19(NR_CellId_Type p_NR_CellId, template (value) SIB19_r17 p_Sib19) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (f_NR5GC_IsSib19InSysinfoCombination(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700 := valueof(p_Sib19); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } };

After Change:

function f_NR5GC_CellInfo_SetSIB19(NR_CellId_Type p_NR_CellId, template (value) SIB19_r17 p_Sib19) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (f_NR5GC_IsSib19InSysinfoCombination(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700 := valueof(p_Sib19); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } };

## Change 2

| Function name | f_NR_GetSIB19_SI_Index |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 in the system information list, correction the index of SIB19 for NR-28 combination. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_GetSIB19_SI_Index (NR_SysinfoCombination_Type p_NR_SysinfoCombination) return integer { var integer v_SIB19_SI_Index; select (p_NR_SysinfoCombination) { case (NR_28) { v_SIB19_SI_Index := 0; } case (NR_29) { v_SIB19_SI_Index := 1; } case (NR_32) { v_SIB19_SI_Index := 2; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } return v_SIB19_SI_Index; }

After Change:

function f_NR_GetSIB19_SI_Index (NR_SysinfoCombination_Type p_NR_SysinfoCombination) return integer { var integer v_SIB19_SI_Index; select (p_NR_SysinfoCombination) { case (NR_28) { v_SIB19_SI_Index := 1; } case (NR_29) { v_SIB19_SI_Index := 1; } case (NR_32) { v_SIB19_SI_Index := 2; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } return v_SIB19_SI_Index; }

## Change 3

| Function name | f_NR_CellInfo_GetSIB19 |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 in the system information list, correction the index of SIB19 for NR-28 combination. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_CellInfo_GetSIB19(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return SIB19_r17 { <<SKIPPED CODE>> select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_28) { v_SIB19 := v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700; } <<SKIPPED CODE>> }

After Change:

function f_NR_CellInfo_GetSIB19(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return SIB19_r17 { <<SKIPPED CODE>> select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_28) { v_SIB19 := v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700; } <<SKIPPED CODE>> }

## Change 4

| Function name | f_NR_InitSystemInformation |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | Add SIB2 message in SIB1 system list for the NR-28 combination. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_InitSystemInformation(NR_SysinfoCombination_Type p_NR_SysinfoCombination, NR_FrequencyBand_Type p_NR_FrequencyBand, SubcarrierSpacing p_SubcarrierSpacing, template (value) NR_NAS_Parameters_Type p_NAS_Parameters, template (value) NR_DownlinkBWP_List_Type p_DL_BWPs, template (omit) NR_UplinkBWP_List_Type p_UL_BWPs, integer p_DeltaNRFs, SSB_Index p_SSB_Index, NR_CellId_Type p_NR_CellId ) runs on NR_BASE_PTC return template (value) NR_CellSysInfo_Type { <<SKIPPED CODE>> case(NR_28){ //@sic R5-232436 sic@ //SIB1 with cellBarredNTN-r17 = notBarred v_SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.cellBarredNTN_r17 := notBarred; //SI1: SIB19 v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB19(f_NR_InitSIB19()))); v_SI_List := {v_BCCH_SI1}; } <<SKIPPED CODE>> }

After Change:

function f_NR_InitSystemInformation(NR_SysinfoCombination_Type p_NR_SysinfoCombination, NR_FrequencyBand_Type p_NR_FrequencyBand, SubcarrierSpacing p_SubcarrierSpacing, template (value) NR_NAS_Parameters_Type p_NAS_Parameters, template (value) NR_DownlinkBWP_List_Type p_DL_BWPs, template (omit) NR_UplinkBWP_List_Type p_UL_BWPs, integer p_DeltaNRFs, SSB_Index p_SSB_Index, NR_CellId_Type p_NR_CellId ) runs on NR_BASE_PTC return template (value) NR_CellSysInfo_Type { <<SKIPPED CODE>> case(NR_28){ //@sic R5-232436 sic@ //SIB1 with cellBarredNTN-r17 = notBarred v_SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.cellBarredNTN_r17 := notBarred; //SI1: SIB2 v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB2(v_SIB2))); //SI2: SIB19 v_BCCH_SI2 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB19(f_NR_InitSIB19()))); v_SI_List := {v_BCCH_SI1, v_BCCH_SI2}; } <<SKIPPED CODE>> }

## Change 5

| Function name | fl_NR_InitSI_SchedulingInfo_v1700 |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 in the SIB1 scheduling list, so correction to the index for SIB19. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NR_InitSI_SchedulingInfo_v1700(NR_SysinfoCombination_Type p_NR_SysinfoCombination) return template (omit) SI_SchedulingInfo_v1700 { <<SKIPPED CODE>> case(NR_28) { v_SchedulingInfoList2 := { cs_SchedulingInfo2_r17(1, tsc_SIB19_Periodicity, {cs_SIB_TypeInfo_Type1(sibType19)})}; v_SI_SchedulingInfo_v1700 := cs_SI_SchedulingInfo_v1700_Def(v_SchedulingInfoList2); } <<SKIPPED CODE>> }

After Change:

function fl_NR_InitSI_SchedulingInfo_v1700(NR_SysinfoCombination_Type p_NR_SysinfoCombination) return template (omit) SI_SchedulingInfo_v1700 { <<SKIPPED CODE>> case(NR_28) { v_SchedulingInfoList2 := { cs_SchedulingInfo2_r17(2, tsc_SIB19_Periodicity, {cs_SIB_TypeInfo_Type1(sibType19)})}; //WA# v_SI_SchedulingInfo_v1700 := cs_SI_SchedulingInfo_v1700_Def(v_SchedulingInfoList2); } <<SKIPPED CODE>> }

## Change 6

| Function name | fl_NR_InitSI_SchedulingInfo |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 message into SIB1 schedulinginfolist. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NR_InitSI_SchedulingInfo(NR_SysinfoCombination_Type p_NR_SysinfoCombination, FreqBandIndicatorNR p_FreqBandIndicatorNR) return template (omit) SI_SchedulingInfo { <<SKIPPED CODE>> case(NR_28) { // SIB19 scheduled in si-SchedulingInfo-v1700 - done inside fl_NR_InitSI_SchedulingInfo_v1700 } <<SKIPPED CODE>> }

After Change:

function fl_NR_InitSI_SchedulingInfo(NR_SysinfoCombination_Type p_NR_SysinfoCombination, FreqBandIndicatorNR p_FreqBandIndicatorNR) return template (omit) SI_SchedulingInfo { <<SKIPPED CODE>> case(NR_28) { // SIB19 scheduled in si-SchedulingInfo-v1700 - done inside fl_NR_InitSI_SchedulingInfo_v1700 v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB2_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType2)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } <<SKIPPED CODE>> }

## Change 7

| Function name | cs_NTN_SemistaticTA_GSO_Cell1 |
| --- | --- |
| Reason for change | According to the chapter 7.5.2.2 in 38.523-3, the NtnUeSpecificTA of GSO NTN Cell1 should be 59771544, but the current TTCN configure it with wrong value. |
| Summary of change | Follow the chapter 7.5.2.2 in 38.523-3, change the NtnUeSpecificTA of GSO NTN Cell1 from 7476932 to 59771544 |
| TTCN module | NR_NTN_Configuration.ttcn |
| MCC160 Comment |  |

Before Change:

//@sic R5-243597 sic@ template (value) NR_NTN_SemistaticTA_Type cs_NTN_SemistaticTA_GSO_Cell1 := cs_NTN_SemistaticTA_Def(65896378,7476932); /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

After Change:

//@sic R5-243597 sic@ template (value) NR_NTN_SemistaticTA_Type cs_NTN_SemistaticTA_GSO_Cell1 := cs_NTN_SemistaticTA_Def(65896378,59771544); //WI=1299522/* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

## Change 8

| Function name | cs_38508_UE_TimersAndConstants |
| --- | --- |
| Reason for change | For the current TTCN implementation, the value of t300 is ms1000, it is not suitable for the NR NTN GSO scenario, it does not take the NR NTN RTT(550ms) into account after the UE sends RRCSetupRequest and wait for RRCSetup message. |
| Summary of change | Enlarge the t300 from ms1000 to ms2000. A Prose CR will be raised for this. |
| TTCN module | NR_SysInfo_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) UE_TimersAndConstants cs_38508_UE_TimersAndConstants := { // Acc to 38.508-1 Table 4.6.3-143AE: UE-TimersAndConstants /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ t300 := ms1000, t301 := ms1000, t310 := ms1000, n310 := n1, t311 := ms30000, n311 := n1, t319 := ms1000 };

After Change:

template (value) UE_TimersAndConstants cs_38508_UE_TimersAndConstants := { // Acc to 38.508-1 Table 4.6.3-143AE: UE-TimersAndConstants /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ t300 := ms2000, t301 := ms1000, t310 := ms1000, n310 := n1, t311 := ms30000, n311 := n1, t319 := ms1000 };

## Change 9

| Function name | cs_38508_UL_RLC_SRB () |
| --- | --- |
| Reason for change | For the current TTCN implementation,configure the t_PollRetransmit to ms45 for SRB of UL RLC AM, it is not suitable for the NR NTN GSO scenario, it does not take the NR NTN RTT(550ms) into account. |
| Summary of change | Enlarge the t_PollRetransmit from ms45 to ms2000. Please see change 12, point 2. A Prose CR will be raised for this. |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) UL_AM_RLC cs_38508_UL_RLC_SRB := { //As per 38.331 cl 9.2.1 /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ sn_FieldLength := size12, t_PollRetransmit := ms45, pollPDU := infinity_, pollByte := infinity_, maxRetxThreshold := t8 };

After Change:

template (value) UL_AM_RLC cs_38508_UL_RLC_SRB := { //As per 38.331 cl 9.2.1 /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ sn_FieldLength := size12, t_PollRetransmit := ms2000, pollPDU := infinity_, pollByte := infinity_, maxRetxThreshold := t8 };

## Change 10

| Function name | cs_NR_PeriodicityAndOffsetSl640 () |
| --- | --- |
| Reason for change | For the current TTCN implementation,there is no suitable PeriodicityAndOffset settings for NR NTN testing. |
| Summary of change | For NR NTN GSO test, add a new template, cs_NR_PeriodicityAndOffsetSl640(). Please see change 11 below. A Prose CR will be raised for this. |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment |  |

New Add:

template (value) SchedulingRequestResourceConfig.periodicityAndOffset cs_NR_PeriodicityAndOffsetSl640 := { sl640 := 639 }; //WI=1398470

## Change 11

| Function name | f_NR_ReturnPeriodicityAndOffset () |
| --- | --- |
| Reason for change | For NR NTN GSO scenario,the value of PeriodicityAndOffset for kHz15 need to take the NR NTN RTT(550ms) into account. |
| Summary of change | For NR NTN GSO test, enlarge the PeriodicityAndOffset from sl10 to sl640. |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_ReturnPeriodicityAndOffset(SubcarrierSpacing p_SubcarrierSpacing) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset { var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset; //As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig select (p_SubcarrierSpacing) { case(kHz15) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl10; } case(kHz30) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20; } case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80; } case else { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } return v_PeriodicityAndOffset; }

After Change:

function f_NR_ReturnPeriodicityAndOffset(SubcarrierSpacing p_SubcarrierSpacing) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset { var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset; //As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig select (p_SubcarrierSpacing) { case(kHz15) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl640; } case(kHz30) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20; } case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80; } case else { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } return v_PeriodicityAndOffset; }

## Change 12

| Function name | f_TC_7_1_2_3_9a_NR5GC() |
| --- | --- |
| Reason for change | - The procedure of UE preconfiguration is already invokded by f_NR5GC_Preamble (). - For NR NTN GSO scenario, the value of t-PollRetransmit should take the NR NTN RTT(550ms) into account. |
| Summary of change | - Delete redundant UE preconfiguration procedure. - Configure the t-PollRetransmit = ms2000, For this, a Prose CR will be raised. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_2_3_9a_NR5GC() runs on NR5GC_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DRB_Identity v_DRBId; var UL_t_PollRetransmit_Type v_T_PollRetransmit; f_NR5GC_Init(NR_28); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // preconfigure the UE location f_PreconfigureUE_ForNTN(UT); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // @sic R5s190312 ch. 4.3 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Establishing UL AM RLC parameters v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, f_NR_38508_SetSNFieldLengthAM(), v_T_PollRetransmit, // t_PollRetransmit -, -, -, f_NR_38508_SetSNFieldLengthAM(), ms200,// t_Reassembly -,// t_StatusProhibit -, ms2200);// t_ReassemblyExt_r17 f_NR5GC_RRC_ConnectedState3N_RLC_AM (v_DRBId, v_RLC_BearerConfigList[0], f_NR_38508_SetSNFieldLengthAM(), -, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId)); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); // @sic R5s190906 ch. 1 sic@ if (pc_supportOfRedCap_r17 == true) { v_RLC_Rec.AM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.2; v_RLC_Rec.t_ReassemblyExt_r17 := 2.2; f_NR_TestBody_Set(true); f_TC_7_1_2_3_9a_NR_TestBody (v_RLC_Rec, v_DRBId); f_NR_TestBody_Set(false); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s190312 ch. 4.3 sic@ // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); @sic R5s230130 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_7_1_2_3_9a_NR5GC() runs on NR5GC_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DRB_Identity v_DRBId; var UL_t_PollRetransmit_Type v_T_PollRetransmit; f_NR5GC_Init(NR_28); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // preconfigure the UE location //f_PreconfigureUE_ForNTN(UT); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // @sic R5s190312 ch. 4.3 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Establishing UL AM RLC parameters v_T_PollRetransmit := ms2000; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, f_NR_38508_SetSNFieldLengthAM(), v_T_PollRetransmit, // t_PollRetransmit -, -, -, f_NR_38508_SetSNFieldLengthAM(), ms200,// t_Reassembly -,// t_StatusProhibit -, ms2200);// t_ReassemblyExt_r17 f_NR5GC_RRC_ConnectedState3N_RLC_AM (v_DRBId, v_RLC_BearerConfigList[0], f_NR_38508_SetSNFieldLengthAM(), -, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId)); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); // @sic R5s190906 ch. 1 sic@ if (pc_supportOfRedCap_r17 == true) { v_RLC_Rec.AM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.2; v_RLC_Rec.t_ReassemblyExt_r17 := 2.2; f_NR_TestBody_Set(true); f_TC_7_1_2_3_9a_NR_TestBody (v_RLC_Rec, v_DRBId); f_NR_TestBody_Set(false); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s190312 ch. 4.3 sic@ // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); @sic R5s230130 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 13

| Function name | f_TC_7_1_2_3_9a_NR_TestBody() |
| --- | --- |
| Reason for change | - The default value of ACK SN is 0, but according to the step2 and step 4 of Table 7.1.2.3.9a.3.2-1, the ACK SN of receiving message should be 1 - At step4, there are some duplicate TTCN code with step 2. - For different UEs, depend on its capability, some may need more time to send RLC STATUS PDU back after the t-ReassemblyExt-r17 expired. - t_WatchDog is not stopped after receiving the RLC STATUS PDU. |
| Summary of change | - Set the expected value of ACK SN to 1. - At step4, delete duplicate TTCN code. - Enlarge the timer t_WatchDog from 1s to 2s to compansate different UE capability. - Stop t_WatchDog after receiving the RLC STATUS PDU. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 2" siclog@ // Check: For 1 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(1.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-1.0); // 1 sec of step 2 has already elapsed ... // @siclog "Step 4" siclog@ // Check: For 1 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(1.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

After Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @siclog "Step 2" siclog@ // Check: For 1 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(1.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-1.0); // 1 sec of step 2 has already elapsed ... // @siclog "Step 4" siclog@ // Check: For 1 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // COMMENTED OUT v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; // COMMENTED OUT v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); // COMMENTED OUT v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n55” using NIA1 Integrity and NEA0 ciphering algorithms on R&S 5G Conformance Test platform.

# Execution Log Files

## Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_7_1_2_3_9a_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250101: Supporting information for addition of NR-NTN testcase 7.1.2.3.9a |
| --- | --- |