3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250102

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4890 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE SSAC ACB per PLMN test case 13.5.2c |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 13.5.2c to the LTE_A_R12 test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 13.5.2c required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.2c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

1.1 Verification Test Summary	3

2 Corrections required	3

2.1 Change 1	3

2	Branches executed	7

3	Execution Log Files	7

4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE	7

4	References	7

# 1 Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 13.5.2c which is part of the LTE_A_R12 test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

Mohit Kanchan

Mohit.kanchan@keysight.com

## 1.1 Verification Test Summary

Test Case: 	13.5.2c

Test Group:                        /LTE_A_R12/13/AccessControl_R12.ttcn

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Rohde & Schwarz CMW500, Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# 2 Corrections required

## 2.1 Change 1

| Function name | f_TC_13_5_2c_EUTRA () runs on EUTRA_PTC |
| --- | --- |
| Reason for change | - In existing TTCN implementation the timer tolerance is being set before the initialization of EUTRA cell which is causing test case execution error. - The SIB2 in step 3 is not implemented in TTCN as per Table 13.5.2c.3.3-3 in the prose. - 3.Below changes agreed for ACB per PLMN test case 13.5.1c in R5-250466 are applicable for ACB per PLMN MTSO MO video call test case 13.5.2c as well: - TTCN has handling for call release procedure after step 22 and those steps are missing in main behavior table 13.5.2c.3.2-1. - Implementing FPLMN USIM will impose additional SIM requirements and same functionality as adding 002-11 to FPLMN list can be achieved by modifying cellReservedForOperatorUse IE in SIB1 - The MTSI MO Video call establisthment not implmented in TTCN as per TS 36.508 4.5A.8.3-1. It contains unecessary UE Cap exchange procdure at step 20-23 which needs to be removed. |
| Summary of change | - Moved the timer tolerance function after f_EUTRA_Init - Modified the SIB2 information to align with prose. - Added steps 23-25 to release the video call. - Removed SIM configuration requirement and added cellReservedForOperatorUse IE in specific message contents. - Removed the UE Capability exchange procedure at step 20-23 |
| TTCN module | LTE_A_R12\13\ AccessControl_R12.ttcn |
| MCC160 comment |  |

Before Change:

function f_TC_13_5_2c_EUTRA() runs on EUTRA_PTC { // MTSI MO video call / SSAC / 0% access probability for MTSI MO video call / AC-Barring per PLMN var float v_TimerNoRRCSetupReq := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, nonProtocolTimer, 10.0); f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11 //PLMNid 2 (index 1) ); f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, omit, cs_AC_BarringConfig(p00, s128, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); f_UT_USIM_Insert(UT, "Insert special USIM"); //USIM with PLMN2 is set in it as 'forbidden PLMN' f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ // Make the UE attempt a MTSI MO Video call f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); // @siclog "Step 2" siclog@ // Check: Does the UE transmit a RRCConnectionRequest message within 10 s? if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerNoRRCSetupReq)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, " Step 2"); } // @siclog "Step 3" siclog@ //SS adjusts SIB2 of Cell1 and transmits a Paging message. The systemInfoValueTag in the SystemInformationBlockType1 is increased. f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false); f_EUTRA_ModifySysinfo(eutra_Cell1); // @siclog "Step 4" siclog@ // Wait for 15 s (Note 1) to allow the new system information to take effect f_Delay(15.0); // @siclog "Step 5" siclog@ // Make the UE attempt a MTSI MO Video call f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); // @siclog "Step 6 - 7" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with in 10s? // Wait for 132 s (Note 2) to make the ac-BarringTime 128s Ty expire. if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerNoRRCSetupReq)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, " Step 6"); } f_Delay(132.0); // @siclog "Step 8 - 21" siclog@ // Steps 1 to 15 of the generic test procedure for MTSI MO Video call establishment in EUTRA: Normal Service (TS 36.508 4.5A.8.3-1). // Make the UE attempt a MTSI MO Video call f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); f_EUTRA_ServiceRequestAndActivate_SRB2_DRB(eutra_Cell1, -, -, -, -, -, -, -, -, -, cr_EstablishmentCause_MoData); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_ActivateDedicatedEpsBearer_SpeechVideo(eutra_Cell1); // @siclog "Step 20 - 23" siclog@ f_EUTRA_508Check_ConnectedState (eutra_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change:

function f_TC_13_5_2c_EUTRA() runs on EUTRA_PTC { // MTSI MO video call / SSAC / 0% access probability for MTSI MO video call / AC-Barring per PLMN var float v_TimerNoRRCSetupReq ; //WA#WI1394899 f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); v_TimerNoRRCSetupReq := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, nonProtocolTimer, 10.0); //WA#WI1394899 f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11, //PLMNid 2 (index 1) -,true,1); // WA#R5-25xxxx_36.523-1_ACB_13.5.2c f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, omit, cs_AC_BarringConfig(p00, s128, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); //f_UT_USIM_Insert(UT, "Insert special USIM"); //USIM with PLMN2 is set in it as 'forbidden PLMN' WA#R5-25xxxx_36.523-1_ACB_13.5.2c f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ // Make the UE attempt a MTSI MO Video call f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); // @siclog "Step 2" siclog@ // Check: Does the UE transmit a RRCConnectionRequest message within 10 s? if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerNoRRCSetupReq)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, " Step 2"); } // @siclog "Step 3" siclog@ //SS adjusts SIB2 of Cell1 and transmits a Paging message. The systemInfoValueTag in the SystemInformationBlockType1 is increased. f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false,-,-,-,-,-,-,-,-,true); //WA#WI1394899 f_EUTRA_ModifySysinfo(eutra_Cell1); // @siclog "Step 4" siclog@ // Wait for 15 s (Note 1) to allow the new system information to take effect f_Delay(15.0); // @siclog "Step 5" siclog@ // Make the UE attempt a MTSI MO Video call f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); // @siclog "Step 6 - 7" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with in 10s? // Wait for 132 s (Note 2) to make the ac-BarringTime 128s Ty expire. if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerNoRRCSetupReq)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, " Step 6"); } f_Delay(132.0); // @siclog "Step 8 - 21" siclog@ // Steps 1 to 15 of the generic test procedure for MTSI MO Video call establishment in EUTRA: Normal Service (TS 36.508 4.5A.8.3-1). // Make the UE attempt a MTSI MO Video call f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); f_EUTRA_ServiceRequestAndActivate_SRB2_DRB(eutra_Cell1, -, -, -, -, -, -, -, -, -, cr_EstablishmentCause_MoData); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_ActivateDedicatedEpsBearer_SpeechVideo(eutra_Cell1); // @siclog "Step 20 - 23" siclog@ //f_EUTRA_508Check_ConnectedState (eutra_Cell1); WA#WI1394899 f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

# Branches executed

The test case was executed on on EUTRA band 1 using ciphering algorithm eea1 and integrity algorithm eia1 on Rohde &Sshwarz CMW 500 platform.

The test case was executed on EUTRA band 4 using ciphering algorithm eea1 and integrity algorithm eia1 on Keysight S8704A Protocol Conformance Toolset.

# Execution Log Files

## Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE passed this test case on Rohde & Schwarz CMW500 and on Keysight S8704A Protocol Conformance Toolset solution . The documentation below is enclosed as evidence of the successful test case run [1]:

R&S :

- Test case execution log file:

TC_13_5_2c_Qualcomm.log

Keysight :

- Test case execution log file:

TC_13_5_2c_LOG.html

- PICS/PIXIT parameter file:

TC_13_5_2c_PIXIT.xml

# References

| [1] | R5s250103: Supporting information for addition of LTE SSAC ACB per PLMN test case 13.5.2c |
| --- | --- |