3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250105

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3645 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 6.7.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 6.7.1.2 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.7.1.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes mentioned in R5s250100 are needed in addition to those mentioned |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 6.7.1.2	4

2.1	Change 1	4

2.2	Change 2	4

3	Branches executed	7

4	Execution Log Files	7

4.1	Unisoc UMS9632	7

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.7.1.2 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	6.7.1.2.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

# Corrections required to testcase 6.7.1.2

Changes mentioned in R5s250100 are needed in addition to the below mentioned changes.

## Change 1

| Function name | cs_NTN_SemistaticTA_GSO_Cell2 |
| --- | --- |
| Reason for change | Base on the configurerations of NTN Cell2 in Table 6.3.4.1-1 of 38.508-1 and according to the way on how to calculate the NtnUeSpecificTA in the chapter 7.5.2.2 of 38.523-3, the NtnUeSpecificTA of GSO NTN Cell2 should be 60143202, but the current TTCN configure it with wrong value. |
| Summary of change | Change the NtnUeSpecificTA of NR NTN GSO Cell2 from 7523436 to 60143202. |
| TTCN module | NR_NTN_Configuration.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_NTN_SemistaticTA_Type cs_NTN_SemistaticTA_GSO_Cell2 := cs_NTN_SemistaticTA_Def(66485757,7523436);

After Change:

template (value) NR_NTN_SemistaticTA_Type cs_NTN_SemistaticTA_GSO_Cell2 := cs_NTN_SemistaticTA_Def(66485757,60143202);

## Change 2

| Function name | fl_TC_6_7_1_2_Body() |
| --- | --- |
| Reason for change | - Start UL grant to allocate UL resource for the following generic test procedure on NR Cell11. - According to the Table 6.7.1.2.3.2-2 in 38.523-1, At step5-10. Generic test procedur are performed on NR Cell11. |
| Summary of change | - Start ULGrant on NR Cell11. - Change from the nr_Cell1 to nr_Cell11. |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_7_1_2_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS re-adjusts the SS/PBCH EPRE levels and according to row "T1" in table 6.1.2.12.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within 3 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell11, 180.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 11 within 3 minutes"); // @siclog "Step 3" siclog@ // The SS transmits a Paging message on NR Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); // @siclog "Step 4" siclog@ // Check: Does the UE transmit an RRCSetupRequest message on NR Cell 11? SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.2 Step 4"); // @siclog "Step 5-10" siclog@ // Steps 3 to 8 of the generic test procedure in TS 38.508-1 [4] subclause 4.9.4.2.2-1 are performed on NR Cell 11. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_NasInd); // @siclog "Step 11" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); }

After Change:

function fl_TC_6_7_1_2_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS re-adjusts the SS/PBCH EPRE levels and according to row "T1" in table 6.1.2.12.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within 3 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell11, 180.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 11 within 3 minutes"); f_NR_ULGrantConfiguration_Start(nr_Cell11); //WI=1333262 // @siclog "Step 3" siclog@ // The SS transmits a Paging message on NR Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); // @siclog "Step 4" siclog@ // Check: Does the UE transmit an RRCSetupRequest message on NR Cell 11? SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.2 Step 4"); // @siclog "Step 5-10" siclog@ // Steps 3 to 8 of the generic test procedure in TS 38.508-1 [4] subclause 4.9.4.2.2-1 are performed on NR Cell 11. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_NasInd); // @siclog "Step 11" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell11); //WI=1333262 }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n55” using NIA1 Integrity and NEA1 ciphering algorithms on R&S 5G Conformance Test platform.

# Execution Log Files

## Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_6_7_1_2_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250106: Supporting information for addition of NR-NTN testcase 6.7.1.2 |
| --- | --- |