3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250111

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3649 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC TBS test cases 7.1.1.4.x.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The calculation of PDCP SDU size is not in accordance with the prose which is leading to size of generated SDUs more than 1500 octets when number of PDCP SDU>20 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the calculation of PDCP SDU size in function fl_NR_GetPdcpSduList_1TB () |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.1.NR5GC, 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.4.NR5GC, 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_NR_GetPdcpSduList_1TB |
| --- | --- |
| Reason for change | The calculation of PDCP SDU size is not in accordance with the prose which is leading to size of generated SDUs more than 1500 octets when number of PDCP SDU>20 |
| Summary of change | Corrected the calculation of PDCP SDU size in function fl_NR_GetPdcpSduList_1TB () |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NR_GetPdcpSduList_TBoms(integer p_TBSize) return NR_PDCP_SDUList_Type { var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24200) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36272) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48344) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; } else if (p_TBSize <= 60416) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72488) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84560) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96632) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108704) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120776) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132848) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144920) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156992) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; } else if (p_TBSize <= 169064) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 181136) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 193208) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205280) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217352) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229424) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else if (p_TBSize <= 241496) { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } else if (p_TBSize <= 253568) { v_N := 21; v_PdcpSduSize := (p_TBSize - 1568)/160; } else if (p_TBSize <= 265640) { v_N := 22; v_PdcpSduSize := (p_TBSize - 1640)/160; } else if (p_TBSize <= 277712) { v_N := 23; v_PdcpSduSize := (p_TBSize - 1712)/160; } else if (p_TBSize <= 289784){ v_N := 24; v_PdcpSduSize := (p_TBSize - 1784)/160; } else if (p_TBSize <= 301856) { v_N := 25; v_PdcpSduSize := (p_TBSize - 1856)/160; } else if (p_TBSize <= 313928){ v_N := 26; v_PdcpSduSize := (p_TBSize - 1928)/160; } else if (p_TBSize <= 326000){ v_N := 27; v_PdcpSduSize := (p_TBSize - 2000)/160; } <<< skipped >>> };

After Change:

function fl_NR_GetPdcpSduList_TBoms(integer p_TBSize) return NR_PDCP_SDUList_Type { var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24200) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36272) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48344) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; } else if (p_TBSize <= 60416) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72488) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84560) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96632) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108704) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120776) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132848) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144920) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156992) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; } else if (p_TBSize <= 169064) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 181136) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 193208) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205280) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217352) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229424) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else if (p_TBSize <= 241496) { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } else if (p_TBSize <= 253568) { v_N := 21; v_PdcpSduSize := (p_TBSize - 1568)/168; } else if (p_TBSize <= 265640) { v_N := 22; v_PdcpSduSize := (p_TBSize - 1640)/176; } else if (p_TBSize <= 277712) { v_N := 23; v_PdcpSduSize := (p_TBSize - 1712)/184; } else if (p_TBSize <= 289784){ v_N := 24; v_PdcpSduSize := (p_TBSize - 1784)/192; } else if (p_TBSize <= 301856) { v_N := 25; v_PdcpSduSize := (p_TBSize - 1856)/200; } else if (p_TBSize <= 313928){ v_N := 26; v_PdcpSduSize := (p_TBSize - 1928)/208; } else if (p_TBSize <= 326000){ v_N := 27; v_PdcpSduSize := (p_TBSize - 2000)/216; } <<< skipped >>> };