3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250130

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4895 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT test case 22.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Due to R5-251427, Cell12 will not be applicable until Step101 & hence at step10, the expected GUTI in the attach request message should be updated to that of Cell1 as per the registration which happened in the preamble. For NTN scenario, the postamble needs to be updated from Cell12 to Cell1. #Draft proseCR would be raised for RAN5#107 meeting - As specified in R5-251427. Cell12 will not be applicable for NTN bands and postamble needs to happen on Cell1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the expected GUTI at step10 to that of cell1. - Introduce if-else condition to determine NTN scenario and perform postamble on either Cell1 or Cell12 accordingly. - Introduce if-else condition to determine NTN scenario and perform postamble on either Cell1 or Cell12 accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS 	iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_22_5_4_Body |
| --- | --- |
| Reason for change | Due to R5-251427, Cell12 will not be applicable until Step101 & hence at step10, the expected GUTI in the attach request message should be updated to that of Cell1 as per the registration which happened in the preamble. The variable v_NAS_Old_PlmnId will be redundant & needs to be removed. #Draft proseCR would be raised for RAN5#107 meeting |
| Summary of change | Updated the expected GUTI at step10 to that of cell1. Replace the variable v_NAS_Old_PlmnId with v_NAS_PlmnId and update the assignment of variable v_Guti_Params. |
| TTCN module | NBIOT\22_5\NBIOT_NAS_Attach.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_22_5_4_Body(GutiParameters_Type p_GutiOldParams) runs on NBIOT_PTC { // ========================================================================== // initialize constant test case data structures // ========================================================================== var NAS_MSG_Indication_Type v_NasInd; var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_UL_Message_Type v_ESMMessage := ?; var NAS_PlmnId v_NAS_Old_PlmnId := f_Asn2Nas_PlmnId(p_GutiOldParams.PLMN_Identity); var GutiParameters_Type v_Guti_Params := f_NBIOT_CellInfo_GetGuti (nbiot_Cell12); var NAS_PlmnId v_NAS_PlmnId := f_Asn2Nas_PlmnId(v_Guti_Params.PLMN_Identity); var template (value) NB_CellPowerList_Type v_CellPowerList; var float v_T3346Val; // SKIPPED CODE //@siclog "Step 10" siclog@ //EXCEPTION: Steps 10a1 and 10b1 describe behaviour that depends on UE capabilities; //the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. //IF px_DoAttachWithoutPDN THEN //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, an ESM DUMMY MESSAGE? //ELSE //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, a PDN CONNECTIVITY REQUEST? v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, // @sic R5-251427 sic@ cr_508_RRCConnectionSetupComplete_NB(-, -, -, ?), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_CIOT_ATTACH_REQUEST_WithGuti(cr_AdditionalUpdateType_CIOT, ?, f_Imsi2MobileIdentity(px_IMSI_Def), // @sic R5s170256 sic@ cr_TAI(v_NAS_Old_PlmnId, tsc_TAC1)ifpresent) , // @sic R5s170256 sic@ v_ESMMessage)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // SKIPPED CODE //@siclog "Step 102a4" siclog@ // @sic R5-251349 sic@ f_NBIOT_RRC_ConnectionSetup_Def(); if (px_DoAttachWithoutPDN) { //@siclog "Step 102a5a1" siclog@ // @sic R5-251349 sic@ v_ESMMessage := cr_ESM_DUMMY_MESSAGE; } else { //@siclog "Step 102a5b1" siclog@ // @sic R5-251349 sic@ v_ESMMessage := cr_CIOT_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, v_PDNType); } // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell12, // @sic R5-251349 sic@ cr_508_RRCConnectionSetupComplete_NB(-, -, -, ?), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_CIOT_ATTACH_REQUEST_WithGuti(cr_AdditionalUpdateType_CIOT, ?, f_Imsi2MobileIdentity(px_IMSI_Def), // @sic R5s170256 sic@ cr_TAI(v_NAS_Old_PlmnId, tsc_TAC1)ifpresent) , // @sic R5s170256 sic@ v_ESMMessage)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 102a5"); // @sic R5-251349 sic@ }

After Change

function fl_TC_22_5_4_Body(GutiParameters_Type p_GutiOldParams) runs on NBIOT_PTC { // ========================================================================== // initialize constant test case data structures // ========================================================================== var NAS_MSG_Indication_Type v_NasInd; var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_UL_Message_Type v_ESMMessage := ?; // REMOVED var GutiParameters_Type v_Guti_Params := p_GutiOldParams; var NAS_PlmnId v_NAS_PlmnId := f_Asn2Nas_PlmnId(v_Guti_Params.PLMN_Identity); var template (value) NB_CellPowerList_Type v_CellPowerList; var float v_T3346Val; // SKIPPED CODE //@siclog "Step 10" siclog@ //EXCEPTION: Steps 10a1 and 10b1 describe behaviour that depends on UE capabilities; //the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. //IF px_DoAttachWithoutPDN THEN //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, an ESM DUMMY MESSAGE? //ELSE //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST message (indicating GUTI, last visited registered TAI and KSI as per the registration which happened in the preamble), and, a PDN CONNECTIVITY REQUEST? v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, // @sic R5-251427 sic@ cr_508_RRCConnectionSetupComplete_NB(-, -, -, ?), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_CIOT_ATTACH_REQUEST_WithGuti(cr_AdditionalUpdateType_CIOT, ?, f_GutiParameters2MobileIdentity(omit, v_Guti_Params), // @sic R5s170256 sic@ cr_TAI(v_NAS_PlmnId, tsc_TAC1)ifpresent) , // @sic R5s170256 sic@ v_ESMMessage)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // SKIPPED CODE //@siclog "Step 102a4" siclog@ // @sic R5-251349 sic@ f_NBIOT_RRC_ConnectionSetup_Def(); if (px_DoAttachWithoutPDN) { //@siclog "Step 102a5a1" siclog@ // @sic R5-251349 sic@ v_ESMMessage := cr_ESM_DUMMY_MESSAGE; } else { //@siclog "Step 102a5b1" siclog@ // @sic R5-251349 sic@ v_ESMMessage := cr_CIOT_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, v_PDNType); } // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell12, // @sic R5-251349 sic@ cr_508_RRCConnectionSetupComplete_NB(-, -, -, ?), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_CIOT_ATTACH_REQUEST_WithGuti(cr_AdditionalUpdateType_CIOT, ?, f_Imsi2MobileIdentity(px_IMSI_Def), // @sic R5s170256 sic@ cr_TAI(v_NAS_PlmnId, tsc_TAC1)ifpresent) , // @sic R5s170256 sic@ v_ESMMessage)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 102a5"); // @sic R5-251349 sic@ }

## Change 2

| Function name | f_TC_22_5_4_NBIOT |
| --- | --- |
| Reason for change | As specified in R5-251427. Cell12 will not be applicable for NTN bands and postamble needs to happen on Cell1. |
| Summary of change | Introduce if-else condition to determine NTN scenario and perform postamble on either Cell1 or Cell12 accordingly. |
| TTCN module | NBIOT\22_5\NBIOT_NAS_Attach.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_22_5_4_NBIOT() runs on NBIOT_PTC { //NB-IoT / Attach to new PLMN GUTI reallocation / Network reject with Extended Wait Timer //Paging with IMSI / Attach Rejected Illegal ME/UE / Detach upon switch-off var GutiParameters_Type v_Guti_Old_Params; f_NBIOT_Init(c1,-,-,-,{nbiot_Cell12}); // @sic R5-237126 sic@ v_Guti_Old_Params := f_NBIOT_CellInfo_GetGuti (nbiot_Cell1); // create cell and send out system information f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_CellConfig_Def(nbiot_Cell12, CONTROL_PLANE); // Preamble: The UE is in State 1-NB "Switched OFF" // ========================================================================== f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); f_NBIOT_TestBody_Set(true); fl_TC_22_5_4_Body(v_Guti_Old_Params); // Postamble f_NBIOT_TestBody_Set(false); f_NBIOT_Postamble (nbiot_Cell12, CONTROL_PLANE, N1_IDLE); // @sic R5s170256 sic@ }

After Change

function f_TC_22_5_4_NBIOT() runs on NBIOT_PTC { //NB-IoT / Attach to new PLMN GUTI reallocation / Network reject with Extended Wait Timer //Paging with IMSI / Attach Rejected Illegal ME/UE / Detach upon switch-off var GutiParameters_Type v_Guti_Old_Params; f_NBIOT_Init(c1,-,-,-,{nbiot_Cell12}); // @sic R5-237126 sic@ v_Guti_Old_Params := f_NBIOT_CellInfo_GetGuti (nbiot_Cell1); // create cell and send out system information f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_CellConfig_Def(nbiot_Cell12, CONTROL_PLANE); // Preamble: The UE is in State 1-NB "Switched OFF" // ========================================================================== f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); f_NBIOT_TestBody_Set(true); fl_TC_22_5_4_Body(v_Guti_Old_Params); // Postamble f_NBIOT_TestBody_Set(false); if(pc_NB_ntn_GSO_ScenarioSupport) { f_NBIOT_Postamble (nbiot_Cell1, CONTROL_PLANE, N1_IDLE); // @sic R5s170256 sic@ } else { f_NBIOT_Postamble (nbiot_Cell12, CONTROL_PLANE, N1_IDLE); } }