3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250141

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3628 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TDD-UL-DL-ConfigCommon patterns defined for FR2 in Table 7.1.1.2.1.3.3-5 (to be active from step 19a1 onwards) conflict with the SchedulingRequestResourceConfig defined in Table 7.1.1.2.1.3.3-12 for SCS=120KHz. The 5th slot with such TDD-UL-DL-ConfigCommon is DL slot, therefore cannot be used for SR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update SchedulingRequestResourceConfig to use SR slot offset=4 (instead of 5) in case of SCS=120kHz. Note: An associated ProseCR (R5-251262) will be raised during the RAN5#106 meeting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase incorrectly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

3	References	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_NR_ReturnPeriodicityAndOffset_5 |
| --- | --- |
| Reason for change | The TDD-UL-DL-ConfigCommon patterns defined for FR2 in Table 7.1.1.2.1.3.3-5 (to be active from step 19a1 onwards) conflict with the SchedulingRequestResourceConfig defined in Table 7.1.1.2.1.3.3-12 for SCS=120KHz. The 5th slot with such TDD-UL-DL-ConfigCommon is DL slot, therefore cannot be used for SR. |
| Summary of change | Update SchedulingRequestResourceConfig to use SR slot offset=4 (instead of 5) in case of SCS=120kHz. Note: An associated ProseCR (R5-251262) will be raised during the RAN5#106 meeting |
| TTCN module | NR_RRC_Templates.ttcn |

Before Change:

function fl_NR_ReturnPeriodicityAndOffset_5(SubcarrierSpacing p_SubcarrierSpacing) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset {//@sic R5-197263 sic@ var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset; //As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig select (p_SubcarrierSpacing) { case(kHz15) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl10_5; } case(kHz30) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20_5; } case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80_5; } case else { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } return v_PeriodicityAndOffset; }

After Change:

function f_TC_7_1_1_2_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Steps 3-3A" siclog@ function fl_NR_ReturnPeriodicityAndOffset_5(SubcarrierSpacing p_SubcarrierSpacing) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset {//@sic R5-197263 sic@ var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset; //As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig select (p_SubcarrierSpacing) { case(kHz15) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl10_5; } case(kHz30) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20_5; } case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80_4; } case else { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } return v_PeriodicityAndOffset; }

# References

| [1] | R5-250392: Correction to NR5GC testcase 7.1.1.2.1 |
| --- | --- |