3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250145

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4888 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_6_2_3_21_GERAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of function f_TC_6_2_3_21_GERAN , at Step 5 the timer t_Wait is started, but not stopped when the coordination message is received. This results in the SS trasnmitting the data until t_Wait expires. This causes undefined behaviour at UE side, and causes TC failure. Hence this needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stopped t_Wait upon reception of the coordination message at Step 5. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_2_3_21_GERAN |
| --- | --- |
| Reason for change | In the current TTCN implementation of function f_TC_6_2_3_21_GERAN , at Step 5 the timer t_Wait is started, but not stopped when the coordination message is received. This results in the SS trasnmitting the data until t_Wait expires. This causes undefined behaviour at UE side, and causes TC failure. Hence this needs to be corrected. |
| Summary of change | Stopped t_Wait upon reception of the coordination message at Step 5. Please see below. |
| TTCN module | InterRat_CellReselection_GtoE_GERAN.ttcn |
| MCC160 Comments |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 5" siclog@ t_Wait.start; // start timer so not here all day alt { [] EUTRA.receive(cmr_IRAT_Trigger) {} // Trigger received, so can finish loop now [] t_Wait.timeout { f_GERAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE not camped on EUTRA"); } [else] { // Keep sending data until one of the 2 above shall happen G_SN.send (cas_G_SN_UnitData_REQ(1, v_NSAPI, cs_SNData)); // @sic R5140300 sic@ f_Delay(0.5); // To slow down the rate of data being sent to the SS @sic R5s120660 sic@ repeat; } }<<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 5" siclog@ t_Wait.start; // start timer so not here all day alt { [] EUTRA.receive(cmr_IRAT_Trigger) { t_Wait.stop; //WA#WI1371638 } // Trigger received, so can finish loop now [] t_Wait.timeout { f_GERAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE not camped on EUTRA"); } [else] { // Keep sending data until one of the 2 above shall happen G_SN.send (cas_G_SN_UnitData_REQ(1, v_NSAPI, cs_SNData)); // @sic R5140300 sic@ f_Delay(0.5); // To slow down the rate of data being sent to the SS @sic R5s120660 sic@ repeat; } } <<SKIPPED CODE>>