3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250160

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4896 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE RRC test case 8.1.2.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To verify LTE RRC testcase 8.1.2.7a using iWD D25wk11 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to LTE RRC testcase 8.1.2.7a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function f_TC_8_1_2_7a_EUTRA	3

2.2	Correction to function fl_TC_8_1_2_7a_TestBody	4

3	Branches executed	7

4	Execution Log Files	7

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.2.7a which is part of the LTE_A_R12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.2.7a

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6897 UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_2_7a_EUTRA |
| --- | --- |
| Reason for change | The last step (19) in the test procedure will happen on Cell 11 hence the postamble should be performed on Cell 11 instead of cell 1 |
| Summary of change | Replace cell 1 with cell 11 when calling postamble function |
| TTCN module | \LTE_A_R12\8_1\RRC_ConnEst_R12.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_2_7a_EUTRA() runs on EUTRA_PTC { /* RRC Connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed / AC-Barring per PLMN */ // SKIPPED CODE f_EUTRA_TestBody_Set(true); fl_TC_8_1_2_7a_TestBody(); f_EUTRA_TestBody_Set(false); //UE is switched off (powered off) and detached f_EUTRA_Postamble (eutra_Cell1, E1_IDLE); f_UT_USIM_Remove(UT); f_UT_USIM_Insert(UT, "Default USIM"); }

After Change

function f_TC_8_1_2_7a_EUTRA() runs on EUTRA_PTC { /* RRC Connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed / AC-Barring per PLMN */ // SKIPPED CODE f_EUTRA_TestBody_Set(true); fl_TC_8_1_2_7a_TestBody(); f_EUTRA_TestBody_Set(false); //UE is switched off (powered off) and detached f_EUTRA_Postamble (eutra_Cell11, E1_IDLE); f_UT_USIM_Remove(UT); f_UT_USIM_Insert(UT, "Default USIM"); }

## Change 2

| Function name | fl_TC_8_1_2_7a_TestBody |
| --- | --- |
| Reason for change | At step 2, UT system request to switch on the UE is missing |
| Summary of change | Introduce a function to perform UE switch on at Step 2 |
| TTCN module | \LTE_A_R12\8_1\RRC_ConnEst_R12.ttcn |
| MCC160 Comment |  |

Before change

function fl_TC_8_1_2_7a_TestBody() runs on EUTRA_PTC { var template (value) CellPowerList_Type v_CellPowerList_AtT0; var template (value) CellPowerList_Type v_CellPowerList_AtT1; var EUTRA_SecurityParams_Type v_AuthParams; var NAS_MSG_Indication_Type v_NasInd; var O4_Type v_NasCountUL; var NAS_KsiValue v_KsiValue; var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE(), f_LoopbackModeB_IP_Address_NW()) }; //Get FDD or TDD mode from Cell-configuration v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info(eutra_Cell1); v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (eutra_Cell1); //@siclog "Step 1" siclog@ //The SS changes Cell 1 and Cell 11 level according to the row "T0" in table 8.1.2.7a.3.2-1. v_CellPowerList_AtT0 := { cs_CellPower(eutra_Cell11, -91) }; f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 2" siclog@ //The UE is switched on and brought to state Loop Back Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18]. f_EUTRA_InitialRegistration(eutra_Cell1, PREAMBLE, STATE2A_TESTLOOP_ModeB, NORMAL); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '0A'O); // SKIPPED CODE }

After change

function fl_TC_8_1_2_7a_TestBody() runs on EUTRA_PTC { var template (value) CellPowerList_Type v_CellPowerList_AtT0; var template (value) CellPowerList_Type v_CellPowerList_AtT1; var EUTRA_SecurityParams_Type v_AuthParams; var NAS_MSG_Indication_Type v_NasInd; var O4_Type v_NasCountUL; var NAS_KsiValue v_KsiValue; var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE(), f_LoopbackModeB_IP_Address_NW()) }; //Get FDD or TDD mode from Cell-configuration v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info(eutra_Cell1); v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (eutra_Cell1); //@siclog "Step 1" siclog@ //The SS changes Cell 1 and Cell 11 level according to the row "T0" in table 8.1.2.7a.3.2-1. v_CellPowerList_AtT0 := { cs_CellPower(eutra_Cell11, -91) }; f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 2" siclog@ //The UE is switched on and brought to state Loop Back Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18]. f_UT_SwitchOnUE(UT); f_EUTRA_InitialRegistration(eutra_Cell1, PREAMBLE, STATE2A_TESTLOOP_ModeB, NORMAL); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '0A'O); // SKIPPED CODE }

# Branches executed

The test case was executed on LTE primary band 3 using LTE ciphering eea1 and integrity algorithm eia1.

# Execution Log Files

  - Mediatek MT6897 UE

Mediatek MT6897 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_2_7a_LOG.xml

- PICS/PIXIT parameter file:

TC_8_1_2_7a_PIXIT.txt

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250161: Supporting information for agreement of TC 8.1.2.7a |
| --- | --- |