3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250172

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4898 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN test case 22.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Steps 2-6 results in NPRACH preambles being retransmitted due to different RAR response. The retranmission of NPRACH preambles is based on nprach-Periodicity-r13 (640msec). For NTN test case execution, due to Koffset set to 550mec, the RAR response is received 550msec later and this results in msg3 timing overlapping with Prach transmissions. SS shall be able to monitor for msg3 to detect non conformant behaviour. In order to avoid this, it is proposed to configure nprach-Periodicity-r13 (1280msec) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated v_NPRACH_ParametersList_NB to ms1280 for NTN test execution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:	Parikshit Bhise
	Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_22_3_1_1_NBIOT |
| --- | --- |
| Reason for change | Steps 2-6 results in NPRACH preambles being retransmitted due to different RAR response. The retranmission of NPRACH preambles is based on nprach-Periodicity-r13 (640msec). For NTN test case execution, due to Koffset set to 550mec, the RAR response is received 550msec later and this results in msg3 timing overlapping with Prach transmissions. SS shall be able to monitor for msg3 to detect non conformant behaviour. In order to avoid this, it is proposed to configure nprach-Periodicity-r13 (1280msec) |
| Summary of change | Updated v_NPRACH_ParametersList_NB to ms1280 for NTN test execution. |
| TTCN module |  |
| MCC160 Comment |  |

Before Change

function f_TC_22_3_1_1_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var template (value) SystemInformationBlockType1_NB v_SIB1; var template (value) SystemInformationBlockType2_NB_r13 v_SIB2; var template (value) RSRP_ThresholdsNPRACH_InfoList_NB_r13 v_RSRP_ThresholdsNPRACH_InfoList_NB := {61, 41}; /* @sic R5-177119 sic@ */ var IntegerList_Type v_PowerLevelList := {-70, -90, -110}; /* @sic R5-177119 sic@ */ var template (value) NPRACH_ParametersList_NB_r13 v_NPRACH_ParametersList_NB := { cs_NPRACH_Parameters_Default(ms640, ms8, -, n3, n1), // CE level 0: nprach-StartTime := 8, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 1 cs_NPRACH_Parameters_Default(ms640, ms32, -, n3, n2), // CE level 1: nprach-StartTime := 32, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 2 cs_NPRACH_Parameters_Default(ms640, ms128, -, n3, n4) // CE level 2: nprach-StartTime := 128, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 4 }; var template (value) RACH_InfoList_NB_r13 v_RACH_InfoList := { cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default }; var template (value) NPUSCH_ACK_NACK_NumRepetitions_Msg4_Type v_NPUSCH_ACK_NACK_NumRepetitions_Msg4 := { r8, r8, r8 }; var NPRACH_Parameters_NB_r13 v_NPRACH_Parameters_NB; var RACH_TimingAdvance_Type v_RAR_TA := tsc_RACH_TA_Def; var template (value) DL_CCCH_Message_NB v_RRCConnectionSetup := f_NBIOT_508_RRCConnectionSetup(); var template (value) ContentionResolution_ContainedDlschSdu_Type v_ContentionResolution_RlcPdu := cs_ContentionResolution_DlschSdu(f_RRC_DL_CCCH_NB_Encvalue(v_RRCConnectionSetup)); var template (value) NB_RachProcedureList_Type v_RachProcedureList := { cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_DifferentRAPID), cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 2 -> step 3 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_MultiRAR_UnMatched), cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 4 -> step 5 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_MultiRAR_1Matched), cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 6 -> step 7 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_MultiRAR_1Matched), cs_NB_ContentionResolutionCtrl_TCRNTI_Fail(v_ContentionResolution_RlcPdu)), /* NPRACH Preamble at step 9 -> step 10, 12 @sic R5s180401: v_ContentionResolution_RlcPdu sic@ */ cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_None, cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 13 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_None, cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 15 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Def(v_RAR_TA), cs_NB_ContentionResolutionCtrl_Def(v_ContentionResolution_RlcPdu)) // NPRACH Preamble at step 16 -> step 17, 17B @sic R5-177119 sic@ }; var NBIOT_NPRACH_PreambleGroup_Type v_PreambleGroup; var integer v_CE_Level; var template (present) NAS_MSG_Indication_Type v_ExpectedNasMsg; var float v_WaitForUEtoMeasurePowerLevel := 15.0; f_NBIOT_Init(c1); f_NBIOT_CellInfo_InitMaxReferencePower(v_CellId, -70); // for T0 @sic R5s180024 sic@ // modify system information v_SIB1 := f_NBIOT_CellInfo_GetSIB1(v_CellId); v_SIB1.cellSelectionInfo_r13.q_RxLevMin_r13 := -70; /* Table 22.3.1.1.3.3-4 @sic R5-177119 sic@ */ f_NBIOT_CellInfo_SetSIB1(nbiot_Cell1, v_SIB1);

After Change

module NBIOT_MAC_Testcases { import from NBIOT_Parameters all; … … … function f_TC_22_3_1_1_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var template (value) SystemInformationBlockType1_NB v_SIB1; var template (value) SystemInformationBlockType2_NB_r13 v_SIB2; var template (value) RSRP_ThresholdsNPRACH_InfoList_NB_r13 v_RSRP_ThresholdsNPRACH_InfoList_NB := {61, 41}; /* @sic R5-177119 sic@ */ var IntegerList_Type v_PowerLevelList := {-70, -90, -110}; /* @sic R5-177119 sic@ */ var template (value) NPRACH_ParametersList_NB_r13 v_NPRACH_ParametersList_NB := { cs_NPRACH_Parameters_Default(ms640, ms8, -, n3, n1), // CE level 0: nprach-StartTime := 8, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 1 cs_NPRACH_Parameters_Default(ms640, ms32, -, n3, n2), // CE level 1: nprach-StartTime := 32, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 2 cs_NPRACH_Parameters_Default(ms640, ms128, -, n3, n4) // CE level 2: nprach-StartTime := 128, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 4 }; var template (value) RACH_InfoList_NB_r13 v_RACH_InfoList := { cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default }; var template (value) NPUSCH_ACK_NACK_NumRepetitions_Msg4_Type v_NPUSCH_ACK_NACK_NumRepetitions_Msg4 := { r8, r8, r8 }; var NPRACH_Parameters_NB_r13 v_NPRACH_Parameters_NB; var RACH_TimingAdvance_Type v_RAR_TA := tsc_RACH_TA_Def; var template (value) DL_CCCH_Message_NB v_RRCConnectionSetup := f_NBIOT_508_RRCConnectionSetup(); var template (value) ContentionResolution_ContainedDlschSdu_Type v_ContentionResolution_RlcPdu := cs_ContentionResolution_DlschSdu(f_RRC_DL_CCCH_NB_Encvalue(v_RRCConnectionSetup)); var template (value) NB_RachProcedureList_Type v_RachProcedureList := { cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_DifferentRAPID), cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 2 -> step 3 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_MultiRAR_UnMatched), cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 4 -> step 5 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_MultiRAR_1Matched), cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 6 -> step 7 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(cs_NB_RarList_MultiRAR_1Matched), cs_NB_ContentionResolutionCtrl_TCRNTI_Fail(v_ContentionResolution_RlcPdu)), /* NPRACH Preamble at step 9 -> step 10, 12 @sic R5s180401: v_ContentionResolution_RlcPdu sic@ */ cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_None, cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 13 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_None, cs_NB_ContentionResolutionCtrl_TCRNTI_None), // NPRACH Preamble at step 15 cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Def(v_RAR_TA), cs_NB_ContentionResolutionCtrl_Def(v_ContentionResolution_RlcPdu)) // NPRACH Preamble at step 16 -> step 17, 17B @sic R5-177119 sic@ }; var NBIOT_NPRACH_PreambleGroup_Type v_PreambleGroup; var integer v_CE_Level; var template (present) NAS_MSG_Indication_Type v_ExpectedNasMsg; var float v_WaitForUEtoMeasurePowerLevel := 15.0; f_NBIOT_Init(c1); f_NBIOT_CellInfo_InitMaxReferencePower(v_CellId, -70); // for T0 @sic R5s180024 sic@ if(pc_NB_ntn_only_Connectivity_EPC){ //updated to ms1280 taking k-offset for into account. v_NPRACH_ParametersList_NB := { cs_NPRACH_Parameters_Default(ms1280, ms8, -, n3, n1), // CE level 0: nprach-StartTime := 8, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 1 cs_NPRACH_Parameters_Default(ms1280, ms32, -, n3, n2), // CE level 1: nprach-StartTime := 32, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 2 cs_NPRACH_Parameters_Default(ms1280, ms128, -, n3, n4) // CE level 2: nprach-StartTime := 128, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 4 }; } // modify system information v_SIB1 := f_NBIOT_CellInfo_GetSIB1(v_CellId); v_SIB1.cellSelectionInfo_r13.q_RxLevMin_r13 := -70; /* Table 22.3.1.1.3.3-4 @sic R5-177119 sic@ */ f_NBIOT_CellInfo_SetSIB1(nbiot_Cell1, v_SIB1);