3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250193

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3680 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-17 NR NTN test case 6.7.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 6.7.1.3 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.7.1.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 6.7.1.3	4

2.1	Change 1	4

3	Branches executed	6

4	Execution Log Files	6

4.1	Exynos 2500	6

5	References	7

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.7.1.3 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.7.1.3.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos 2500

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_6_7_1_3_NR5GC() |
| --- | --- |
| Reason for change | At step 23, SS is waiting for SystemInfo modification request and here delay f_delay value comes as 521.753 seconds for cell1. So need to increase guard timer for this case, Also this case is having delay for 60 seconds on step2 and step25 |
| Summary of change | Increased guard timer to 1080 |
| TTCN module | NR5GC_Testsuite.ttcn |

Before Change:

testcase TC_6_7_1_3_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // NR NTN / Cell Selection / Serving cell becomes non-suitable (CellBarredNTN) var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_7_1_3_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_6_7_1_3_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // NR NTN / Cell Selection / Serving cell becomes non-suitable (CellBarredNTN) var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1080); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_7_1_3_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 2

| Function name | f_TC_6_7_1_3_NR5GC () |
| --- | --- |
| Reason for change | As per preamble condition UE must reach the Switch off state (state 0N-B) And to gain that NTN cell must not be barred. |
| Summary of change | Commented f_NR_CellInfo_SetSIB1_cellBarredNTN_r17 function to achieve preamble condition |
| TTCN module | NTN_Idle_NR5GC.ttcn |

Before Change:

function f_TC_6_7_1_3_NR5GC() runs on NR5GC_PTC { // Cell Selection / Serving cell becomes non-suitable (CellBarredNTN) // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_28); // The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select(UT); // The pre-configured UE location is defined in TS 38.508-1 Clause 4.5C f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1); // Table 6.7.1.3.3.3-1: SIB1 for NR cell 1 - cellBarredNTN-r17 Not present for TP1 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_6_7_1_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); } // end of f_TC_6_7_1_3_NR5GC

After Change:

function f_TC_6_7_1_3_NR5GC() runs on NR5GC_PTC { // Cell Selection / Serving cell becomes non-suitable (CellBarredNTN) // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_28); // The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select(UT); // The pre-configured UE location is defined in TS 38.508-1 Clause 4.5C //f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1); // Table 6.7.1.3.3.3-1: SIB1 for NR cell 1 - cellBarredNTN-r17 Not present for TP1 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_6_7_1_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); } // end of f_TC_6_7_1_3_NR5GC

## Change 3

| Function name | fl_TC_6_7_1_3_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - To achive TP1 condition at step2, SIB1 of cell1 must have, cellBarredNTN-r17 as Not present before step1 Power On the UE. - At step2 UE was not registered to Cell1, so it will not able to perform deregistration at step3. Instead deregistration procedure, switch off can be performed to achive step3. - At step7-19 UL grant was not started by the TTCN, so need to configure UL grants after step6 inorder to perform registration procedure. |
| Summary of change | - Added function f_NR_CellInfo_SetSIB1_cellBarredNTN_r17 and f_NR_SS_ConfigureSysinfo before power On - Commented f_NR_UE_DeRegisterOnSwitchOff and added f_UT_PowerOffUE. - Added f_NR_ULGrantConfiguration_Start before |
| TTCN module | NTN_Idle_NR5GC.ttcn |

Before Change:

function fl_TC_6_7_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; timer t_WaitRRCSetupRequest; // @siclog "Step 1" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0, -, "Step2: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); // @siclog "Step 3" siclog@ // Power Off the UE f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1,STATE_OFF_0B); // @siclog "Step 4" siclog@ // SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "not barred" f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1, notBarred); v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(nr_Cell1); f_NR_SS_ConfigureSysinfo(nr_Cell1, -, v_NR_BcchToPdschConfig); // @siclog "Step 5" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 6" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 60.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.3 Step 6"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.3 Step 6: No RRCSetupRequest received within 60 s"); } } // @siclog "Steps 7-19" siclog@ // Steps 3 to 15 of the registration procedure described in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete( tsc_NR_RRC_TI_Def), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF); // @siclog "Step 20" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 21" siclog@ // SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "barred" f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1, barred); // @siclog "Step 22" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 23" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); // @siclog "Step 24" siclog@ // The SS transmits a Paging message within a paging occasion f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 25" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0, -, "Step 25: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); } // end of fl_TC_6_7_1_3_NR5GC_TestBody

After Change:

function fl_TC_6_7_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; timer t_WaitRRCSetupRequest; // @siclog "Step 1" siclog@ // SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "not barred" f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1); v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(nr_Cell1); f_NR_SS_ConfigureSysinfo(nr_Cell1, -, v_NR_BcchToPdschConfig); // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0, -, "Step2: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); // @siclog "Step 3" siclog@ // Power Off the UE //f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1,STATE_OFF_0B); f_UT_PowerOffUE(UT); // @siclog "Step 4" siclog@ // SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "not barred" f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1, notBarred); v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(nr_Cell1); f_NR_SS_ConfigureSysinfo(nr_Cell1, -, v_NR_BcchToPdschConfig); // @siclog "Step 5" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 6" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 60.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.3 Step 6"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.3 Step 6: No RRCSetupRequest received within 60 s"); } } f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); // @siclog "Steps 7-19" siclog@ // Steps 3 to 15 of the registration procedure described in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete( tsc_NR_RRC_TI_Def), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF); // @siclog "Step 20" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); // @siclog "Step 21" siclog@ // SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "barred" f_NR_CellInfo_SetSIB1_cellBarredNTN_r17(nr_Cell1, barred); // @siclog "Step 22" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 23" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); // @siclog "Step 24" siclog@ // The SS transmits a Paging message within a paging occasion f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 25" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0, -, "Step 25: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); } // end of fl_TC_6_7_1_3_NR5GC_TestBody

## Change 4

| Function name | f_NR_SS_ConfigureSysinfo () |
| --- | --- |
| Reason for change | Current TTCN is skipping SI_SchedulingInfo_v1700 to be added and configure for System Informatoin block. Due to that SIB19 is not getting configured at step1 and step4 when SIB1 configuration getting modified for cellBarredNTN-r17 and SS tranmits only SIB1 and SIB2. To address this issue NR_SI_List_Type (v_SI) should be populated with schedulingInfoList2_r17 configuration |
| Summary of change | To populate v_SIs added multiple conditions, please check below for details. |
| TTCN module | NR5GC_SysinfoPaging.ttcn |

Before Change:

function f_NR_SS_ConfigureSysinfo(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_BcchToPdschConfig_Type p_NR_BcchToPdschConfig, boolean p_BroadcastAllSI := false ) runs on NR_BASE_PTC { //@sic R5s220880 : added p_BroadcastAllSI sic@ var NR_CellSysInfo_Type v_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo v_SI_SchedulingInfo; var NR_SI_List_Type v_SIs := {}; //@sic R5s221131 sic@ var integer i := 0; var integer j := 0; if (not(p_BroadcastAllSI) and v_CellSysInfo.NR_SysinfoCombination != MibOnly and v_CellSysInfo.NR_SysinfoCombination != NR_1 ) { //@sic R5s220880 sic@ v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); for (i := 0; i < lengthof(v_SI_SchedulingInfo.schedulingInfoList); i:=i+1) { if ( v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus == broadcasting and (ispresent(v_CellSysInfo.BcchInfo.SIs[i]))) { //@sic R5s221131 sic@ v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; j := j+1; } } v_CellSysInfo.BcchInfo.SIs := v_SIs; } f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_ModifySysinfo_Def_REQ(p_NR_CellId, p_TimingInfo, v_CellSysInfo.BcchInfo, /* system information to be scheduled */ p_NR_BcchToPdschConfig)); /* scheduling of SIs */ }

After Change:

function f_NR_SS_ConfigureSysinfo(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_BcchToPdschConfig_Type p_NR_BcchToPdschConfig, boolean p_BroadcastAllSI := false ) runs on NR_BASE_PTC { //@sic R5s220880 : added p_BroadcastAllSI sic@ var NR_CellSysInfo_Type v_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var template (omit) SI_SchedulingInfo_v1700 v_SI_SchedulingInfo_v1700 :=omit; var SI_SchedulingInfo v_SI_SchedulingInfo; var NR_SI_List_Type v_SIs := {}; //@sic R5s221131 sic@ var integer i := 0; var integer j := 0; var integer k := 0; if (not(p_BroadcastAllSI) and v_CellSysInfo.NR_SysinfoCombination != MibOnly and v_CellSysInfo.NR_SysinfoCombination != NR_1 ) { //@sic R5s220880 sic@ v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); v_SI_SchedulingInfo_v1700 := f_NR_GetSIB1_SiSchedulingInfo_v1700(p_NR_CellId); for (i := 0; i < lengthof(v_SI_SchedulingInfo.schedulingInfoList); i:=i+1) { if ( v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus == broadcasting and (ispresent(v_CellSysInfo.BcchInfo.SIs[i]))) { //@sic R5s221131 sic@ v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; j := j+1; } } if (isvalue(v_SI_SchedulingInfo_v1700)) for (k := 0 ; k < lengthof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17); k:=k+1) { if ( v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[k].si_BroadcastStatus_r17 == broadcasting and (ispresent(v_CellSysInfo.BcchInfo.SIs[i]))) { //@sic R5s221131 sic@ v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; j := j+1; i := i+1; } } } v_CellSysInfo.BcchInfo.SIs := v_SIs; } f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_ModifySysinfo_Def_REQ(p_NR_CellId, p_TimingInfo, v_CellSysInfo.BcchInfo, /* system information to be scheduled */ p_NR_BcchToPdschConfig)); /* scheduling of SIs */ }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms on Anritsu 5G Conformance Test platform in GSO.

# Execution Log Files

## Samsung Exynos 2500

The Samsung Exynos 2500 UE  passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_7_1_3_LOG.xml

- PICS/PIXIT parameter file:

TC_6_7_1_3_PIXIT.txt

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250194: Supporting information for addition of NR-NTN testcase 6.7.1.3 |
| --- | --- |