3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250201

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3685 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IsDNNForIMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The function f_IsDNNForIMS that check the DNN ignores the rules which were used when creating the DNN via the f_DomainName_EncodeLabels. Namely, f_DomainName_EncodeLabels applies 23.003 cl. 9.1, i.e. that each label is preceded by one octet of length and that "." chars are not encoded at all. Therefore the decoding function f_IsDNNForIMS cannot check the presence of “.” characters, but instead it should apply the same rules from 23.003 cl. 9.1. We propose the new implementation below, that also improves the logic of checking that both the first and last part of the DNN need to match (just the first one is not enough). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reworked f_IsDNNForIMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IRAT test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_IsDNNForIMS |
| --- | --- |
| Reason for change | The function f_IsDNNForIMS that check the DNN ignores the rules which were used when creating the DNN via the f_DomainName_EncodeLabels. Namely, f_DomainName_EncodeLabels applies 23.003 cl. 9.1, i.e. that each label is preceded by one octet of length and that "." chars are not encoded at all. Therefore the decoding function f_IsDNNForIMS cannot check the presence of “.” characters, but instead it should apply the same rules from 23.003 cl. 9.1. We propose the new implementation below, that also improves the logic of checking that both the first and last part need to match (just the first one is not enough). |
| Summary of change | Reworked f_IsDNNForIMS |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |

Before Change:

function f_IsDNNForIMS(octetstring p_DNNFromReq) return boolean { var charstring v_DNN := oct2char(p_DNNFromReq); var Char1List_Type v_SplitCharList := {"."}; var CharStringList_Type v_FieldList := f_StringSplit(v_DNN, v_SplitCharList); var integer v_FieldCnt := lengthof(v_FieldList); if (v_FieldCnt == 1) { // DNN = IMS if (match (v_FieldList[0], "IMS")) { return true; } else if (v_FieldCnt == 4) { if ((match (v_FieldList[0], "IMS")) and (match (v_FieldList[3], "gprs"))) { // DNN = IMS.mccMCC.mncMNC.gprs return true; } } } // if none of the above return false; }

After Change:

function f_IsDNNForIMS(octetstring p_DNNFromReq) return boolean { var octetstring v_LengthOctet, v_FieldOctet; var charstring v_FieldChar; var integer v_Begin := 0; var integer i := 0; var integer v_Length; var boolean v_IMS := false; while (v_Begin < lengthof(p_DNNFromReq)) { i := i + 1; v_LengthOctet := substr(p_DNNFromReq, v_Begin, 1); v_Length := oct2int(v_LengthOctet); v_FieldOctet := substr(p_DNNFromReq, v_Begin + 1, v_Length); v_FieldChar := oct2char(v_FieldOctet); if ((i == 1) and (match (v_FieldChar, "IMS"))) {v_IMS := true}; if ((v_IMS) and ((i == 4) and (match (v_FieldChar, "gprs")))) {return true}; //DNN = IMS.mccMCC.mncMNC.gprs v_Begin := v_Begin + v_Length + 1; } return false; }