3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250214

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3631 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_ExtdProtocolConfigOptions_CheckPSDataOff |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation when pc_PS_data_off = TRUE, the function f_Check_NG_PDUSessionEstablishmentReq --> f_ExtdProtocolConfigOptions_CheckPSDataOff is called. The PICS pc_PS_data_off only indicates Support of 3GPP PS data off on the UE side (38.508-2 Table A.4.3.7-1: UE General Capabilities). It does not indicate the value of the IE 3GPP PS data off UE status within the ePCO IE within the PDU Session Establishment REQ. According to Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element : MS -> NW: 0017H (3GPP PS data off UE status); When the container identifier indicates 3GPP PS data off UE status, the container identifier contents field contains information of the status of 3GPP PS data off in the UE for a PDN connection where "01H" indicates ’deactivated’ and "02H" indicates 'activated’. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. From above it is clear that this IE can have either '01'O, or '02'O and the setting of PICS pc_PS_data_off to TRUE should not be associated with whether the UE has activated or deactivated this feature (IE 3GPP PS data off UE status being set to '02'O or '01'O respectively). Basically, it is OK to have pc_PS_data_off = TRUE, and IE 3GPP PS data off UE status = '01'O, or pc_PS_data_off = TRUE, and IE 3GPP PS data off UE status = '02'O. The current TTCN in function f_ExtdProtocolConfigOptions_CheckPSDataOff only returns a TRUE when pc_PS_data_off = TRUE, and IE 3GPP PS data off UE status = '01'O. This would fail a conformant UE which supports the feature pc_PS_data_off (TRUE), but has activated PS data (IE 3GPP PS data off UE status = '02'O). This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the implementation of the function f_ExtdProtocolConfigOptions_CheckPSDataOff to accommodate the case when pc_PS_data_off (TRUE), but has activated PS data (IE 3GPP PS data off UE status = '02'O. Also updated the call to fucntion fl_TC_11_6_x_Body f_ExtdProtocolConfigOptions_CheckPSDataOff as a result of above change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC testcases, if run with pc_PS_data_off = TRUE and UE configured such that IE 3GPP PS data off UE status = '02'O |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | function f_ExtdProtocolConfigOptions_CheckPSDataOff |
| --- | --- |
| Reason for change | In the current TTCN implementation when pc_PS_data_off = TRUE, the function f_Check_NG_PDUSessionEstablishmentReq --> f_ExtdProtocolConfigOptions_CheckPSDataOff is called. The PICS pc_PS_data_off only indicates Support of 3GPP PS data off on the UE side (38.508-2 Table A.4.3.7-1: UE General Capabilities). It does not indicate the value of the IE 3GPP PS data off UE status within the ePCO IE within the PDU Session Establishment REQ. According to Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element : MS -> NW: 0017H (3GPP PS data off UE status); When the container identifier indicates 3GPP PS data off UE status, the container identifier contents field contains information of the status of 3GPP PS data off in the UE for a PDN connection where "01H" indicates ’deactivated’ and "02H" indicates 'activated’. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. From above it is clear that this IE can have either '01'O, or '02'O and the setting of PICS pc_PS_data_off to TRUE should not be associated with whether the UE has activated or deactivated this feature (IE 3GPP PS data off UE status being set to '02'O or '01'O respectively). Basically, it is OK to have pc_PS_data_off = TRUE, and IE 3GPP PS data off UE status = '01'O, or pc_PS_data_off = TRUE, and IE 3GPP PS data off UE status = '02'O. The current TTCN in function f_ExtdProtocolConfigOptions_CheckPSDataOff only returns a TRUE when pc_PS_data_off = TRUE, and IE 3GPP PS data off UE status = '01'O. This would fail a conformant UE which supports the feature pc_PS_data_off (TRUE), but has activated PS data (IE 3GPP PS data off UE status = '02'O). This needs to be corrected. |
| Summary of change | Changed the implementation of the function f_ExtdProtocolConfigOptions_CheckPSDataOff to accommodate the case when pc_PS_data_off (TRUE), but has activated PS data (IE 3GPP PS data off UE status = '02'O. |
| TTCN module | NG_NASTemplateFunctions.ttcn |

Before Change:

function f_ExtdProtocolConfigOptions_CheckPSDataOff(template (omit) ExtdProtocolConfigOptions p_Pco, O1_Type p_Status:='01'O) return boolean { var template (omit) octetstring v_Content := f_ExtdProtocolConfigOptions_Get(p_Pco, '0017'O); var boolean v_Result := false; if (isvalue(v_Content)) { if (lengthof(v_Content) > 0) { v_Result := match (p_Status, v_Content); //@sic R5s220424 R5s220828 sic@ } } return v_Result; }

After Change:

function f_ExtdProtocolConfigOptions_CheckPSDataOff(template (omit) ExtdProtocolConfigOptions p_Pco, boolean p_CheckPSDataActivated := false) return boolean //WA#1390938 { var template (omit) octetstring v_Content := f_ExtdProtocolConfigOptions_Get(p_Pco, '0017'O); var boolean v_Result := false; if (isvalue(v_Content)) { if (lengthof(v_Content) > 0) { if(p_CheckPSDataActivated == true) { v_Result := match ('02'O, v_Content); //@sic R5s220424 R5s220828 sic@ } else { v_Result := (match ('02'O, v_Content) or match ('01'O, v_Content)) ; } } } return v_Result; }

## Change 2

| Template name | function fl_TC_11_6_x_Body |
| --- | --- |
| Reason for change | Please see change 1 above |
| Summary of change | Updated the call to fucntion fl_TC_11_6_x_Body f_ExtdProtocolConfigOptions_CheckPSDataOff as a result of above change |
| TTCN module | PS_Data_OFF_NR%GC.ttcn |

Before Change:

<<SKIPPED CODE>> //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco,'02'O)) // @sic R5s220828 sic@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco,true)) // @sic R5s220828 sic@ //WA#1390938 '02'O --> true { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } <<SKIPPED CODE>>