3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250245

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3706 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cr_PSDataOffInContactParams_Inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current implementation of TTCN incorrectly uses "webrtc-datachannel" for the parameter value cr_PSDataOffInContactParams_Inactive. A new handling condition check has been introduced as per prose CR R5-253112. This involves checking the pc_PS_data_off condition based on both the ID and the parameter value in the function fl_IMS_Register_ContactHeader_CheckParams(). It's important that the TCs pc_PS_data_off parameter value will change from inactive to active at a given time, so it cannot be evaluated in a single condition. Additionally, it is crucial to ensure that the UE includes the +g.3gpp.ps-data-off. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation of cr_PSDataOffInContactParams_Inactive. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.1.NR5GC, 11.6.2.NR5GC, 11.6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | cr_PSDataOffInContactParams_Inactive |
| --- | --- |
| Reason for change | The current implementation of TTCN incorrectly uses "webrtc-datachannel" for the parameter value cr_PSDataOffInContactParams_Inactive. A new handling condition check has been introduced as per prose CR R5-253112. This involves checking the pc_PS_data_off condition based on both the ID and the parameter value in the function fl_IMS_Register_ContactHeader_CheckParams(). It's important that the TCs pc_PS_data_off parameter value will change from inactive to active at a given time, so it cannot be evaluated in a single condition. Additionally, it is crucial to ensure that the UE includes the +g.3gpp.ps-data-off. |
| Summary of change | Update the TTCN template to handle the scenarion. |
| TTCN module | IMS_SIP_Templates.ttcn |

Before Change:

template (present) SemicolonParam_List cr_PSDataOffInContactParams_Inactive := /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NR5GC, NR5GC_IRAT, POS) */ cr_SemicolonParam_List_OneSpecificParamWithValue("+g.3gpp.ps-data-off", "webrtc-datachannel");

After Change:

template (present) SemicolonParam_List cr_PSDataOffInContactParams_Inactive := /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NR5GC, NR5GC_IRAT, POS) */ cr_SemicolonParam_List_OneSpecificParamWithValue("+g.3gpp.ps-data-off", ?);