3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250250

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3708 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR/5GC test case 10.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, Sequans Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Race condition has been detected between step 1 and step 2. UE sends PDU Session Establishment Request between 1 and 2 causing unexpected NAS message reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Declared a default variable v_MyDefaultVar and activated it to capture any PDU Session Establishment REQ between step 1 and 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 10.1.4.1	4

2.1	Change 1	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 10.1.4.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Olivier Genoud

Olivier.Genoud@etsi.org

# Corrections required

## Change 1

| Function name | f_TC_10_1_4_1_NR5GC () |
| --- | --- |
| Reason for change | Race condition has been detected between step 1 and step 2. UE sends PDU Session Establishment Req between 1 and 2 causing unexpected NAS message reception |
| Summary of change | Declared a default variable v_MyDefaultVar and activated it to capture any PDU Session Establishment REQ between setp 1 and 2. Deactivated this default handler just before the start of timer at Step 2 |
| TTCN module | GSM_NR5GC.ttcn |

Before Change:

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) DNN v_RequestedDNN; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_Count := 0; var float v_T3580Val := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 16.0); var charstring v_NSSAI := ""; // @sic R5-213490 sic@ var charstring v_DNN := ""; // @sic R5-213490 sic@ var template (omit)S_NSSAI_Type v_RequestedNSSAI; // @sic R5s210148 sic@ var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5s230865 sic@ timer t_Wait; f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_NoOfSessions +1, -, tsc_RandomAPN);// @sic R5-217818 sic@ v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5-217818 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfoList[v_NoOfSessions -1].SessionId; // @sic R5-217818 sic@ v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); // @sic R5-213490 sic@ if (isvalue(v_RequestedNSSAI)){ // compiler warning may be ignored due to check // @sic R5s201028 sic@ v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI.lvPart); // @sic R5s221179 Baseline Moving sic@ } if (isvalue(v_RequestedDNN)){ // @sic R5s210437 sic@ v_DNN := oct2char(valueof(substr((v_RequestedDNN.dnnValue),1,lengthof(v_RequestedDNN.dnnValue)-1))); } // @sic R5-206357 sic@ //@siclog "Steps 1" siclog@ // The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s230497 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause "Reactivation requested" //@siclog "Steps 2 - 15 void" siclog@ //@siclog "Steps 16" siclog@ //@sic R5-202664 sic@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2). t_Wait.start (1.0); alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { // @sic R5-213490 sic@ f_UT_RequestPDUSession(UT, v_NoOfSessions +2, v_DNN, -, -, v_NSSAI); // @sic R5s230865 sic@ //AT command to trigger a PDU session @sic R5s201230 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } //@siclog "Steps 17-25" siclog@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { //@siclog "Steps 17-19-21-23" siclog@ //The SS waits 16 seconds (T3580). f_Delay(v_T3580Val); //@siclog "Steps 18-20-22-24" siclog@ //Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message sent in step 16? SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } //@siclog "Step 25" siclog@ //The SS waits 16 seconds (5th expiry of T3580). f_Delay(v_T3580Val); //@siclog "Step 26" siclog@ t_Wait.start(20.0); alt{ [] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu,-, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN))// @sic R5s201028 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26"); } } } //End of test body of TC_10_1_4_1

After Change:

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) DNN v_RequestedDNN; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_Count := 0; var float v_T3580Val := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 16.0); var charstring v_NSSAI := ""; // @sic R5-213490 sic@ var charstring v_DNN := ""; // @sic R5-213490 sic@ var template (omit)S_NSSAI_Type v_RequestedNSSAI; // @sic R5s210148 sic@ var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5s230865 sic@ var default v_MyDefaultVarForPDUSessEstReq := null; timer t_Wait; f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_NoOfSessions +1, -, tsc_RandomAPN);// @sic R5-217818 sic@ f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); v_MyDefaultVarForPDUSessEstReq := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(v_NGC_NASCellA)); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5-217818 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfoList[v_NoOfSessions -1].SessionId; // @sic R5-217818 sic@ v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); // @sic R5-213490 sic@ if (isvalue(v_RequestedNSSAI)){ // compiler warning may be ignored due to check // @sic R5s201028 sic@ v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI.lvPart); // @sic R5s221179 Baseline Moving sic@ } if (isvalue(v_RequestedDNN)){ // @sic R5s210437 sic@ v_DNN := oct2char(valueof(substr((v_RequestedDNN.dnnValue),1,lengthof(v_RequestedDNN.dnnValue)-1))); } // @sic R5-206357 sic@ //@siclog "Steps 1" siclog@ // The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s230497 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause "Reactivation requested" deactivate(v_MyDefaultVarForPDUSessEstReq); //@siclog "Steps 2 - 15 void" siclog@ //@siclog "Steps 16" siclog@ //@sic R5-202664 sic@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2). if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > 0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); } else { t_Wait.start (1.0); alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { / / @sic R5-213490 sic@ f_UT_RequestPDUSession(UT, v_NoOfSessions +2, v_DNN, -, -, v_NSSAI); // @sic R5s230865 sic@ //AT command to trigger a PDU session @sic R5s201230 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } } if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } //@siclog "Steps 17-25" siclog@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { //@siclog "Steps 17-19-21-23" siclog@ //The SS waits 16 seconds (T3580). f_Delay(v_T3580Val); //@siclog "Steps 18-20-22-24" siclog@ //Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message sent in step 16? SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } //@siclog "Step 25" siclog@ //The SS waits 16 seconds (5th expiry of T3580). f_Delay(v_T3580Val); //@siclog "Step 26" siclog@ t_Wait.start(20.0); alt{ [] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu,-, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN))// @sic R5s201028 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26"); } } } //End of test body of TC_10_1_4_1