3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250251

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3709 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC eCall over IMS testcase 11.5.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case signaling will happen mostly on EUTRA PTC calling EUTRA functions, postamble function must be aligned. Calling f_EUTRA38_MultiPDN_Postamble will provoke a runtime error. This can be solved calling f_EUTRA_Postamble that implement same LTE procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called f_EUTRA_Postamble implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to f_TC_11_5_12_NR5GC_EUTRA	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to f_TC_11_5_12_NR5GC_EUTRA

| Function name | f_TC_11_5_12_NR5GC_EUTRA |
| --- | --- |
| Reason for change | Test case signaling will happen mostly on EUTRA PTC calling EUTRA functions, postamble function must be aligned. Calling f_EUTRA38_MultiPDN_Postamble will provoke a runtime error. |
| Summary of change | Called f_EUTRA_Postamble implementation. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change:

function f_TC_11_5_12_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS */ //@sic R5s250075 sic@ f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA, USE_BIG_GRANTS); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. //@sic R5s240599 sic@ // Create cell and send out system information f_EUTRA_CellConfig_Def(eutra_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "configuration 24 specified in table 6.4.1-24 TS 38.508-1"); //@sic R5s240599 sic@ f_EUTRA_TestBody_Set(true); fl_TC_11_5_12_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // Tell the NR to release the resources f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); //Postamble on Eutra Cell f_EUTRA38_MultiPDN_Postamble (eutra_Cell1, E1_IDLE); //@sic R5s250125 sic@ } //End of f_TC_11_5_12_NR5GC_EUTRA()

After Change:

function f_TC_11_5_12_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS */ //@sic R5s250075 sic@ f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA, USE_BIG_GRANTS); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. //@sic R5s240599 sic@ // Create cell and send out system information f_EUTRA_CellConfig_Def(eutra_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "configuration 24 specified in table 6.4.1-24 TS 38.508-1"); //@sic R5s240599 sic@ f_EUTRA_TestBody_Set(true); fl_TC_11_5_12_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // Tell the NR to release the resources f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); //Postamble on Eutra Cell f_EUTRA_Postamble (eutra_Cell1, E1_IDLE); } //End of f_TC_11_5_12_NR5GC_EUTRA()