3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250264

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3712 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC testcase 7.1.1.6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation fails the testcase when run with a device in Multi-PDU mode. The issue here is that the logical channel ID that is used for looopback data is hardcoded by usage of tsc_LCID_SCG_Bearer. If the UE only established the Internet PDU session, then this works fine. However, for a MultiPDU scenario, this results in the loopback data being sent on the DRB associated with Internet PDU session, and the TTCN expects loopback data on the DRB associated with IMS PDU session which never happens. This results in testcase failure. TTCN CR R5s250158 was raised to address the same issue for testcase 7.1.1.6.2.NR5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Declared and initialized a variable v_LCID: var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); Used v_LCID instead of tsc_LCID_SCG_Bearer in functions f_TC_7_1_1_6_3_NR_TestBody1 and f_TC_7_1_1_6_3_NR_TestBody2. Please see below. This solution is already implemented for 7.1.1.6.2.NR5GC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	16

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_3_NR_TestBody1 |
| --- | --- |
| Reason for change | The current TTCN implementation fails the testcase when run with a device in Multi-PDU mode. The issue here is that the logical channel ID that is used for looopback data is hardcoded by usage of tsc_LCID_SCG_Bearer. If the UE only established the Internet PDU session, then this works fine. However, for a MultiPDU scenario, this results in the loopback data being sent on the DRB associated with Internet PDU session, and the TTCN expects loopback data on the DRB associated with IMS PDU session which never happens. This results in testcase failure. TTCN CR R5s250158 was raised to address the same issue for testcase 7.1.1.6.2.NR5GC. |
| Summary of change | Declared and initialized a variable v_LCID: var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); Used v_LCID instead of tsc_LCID_SCG_Bearer in function f_TC_7_1_1_6_3_NR_TestBody1. Please see below. This solution is already implemented for 7.1.1.6.2.NR5GC |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_3_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { const integer const_SPS_Period :=40; // period in milli seconds const integer const_UL_TBS_CRNTI := 144; // const integer const_UL_TBS_CSRNTI := 160; //160 bits var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu6; var NR_MAC_SDU_Type v_EncodedRlcPdu7; var NR_MAC_SDU_Type v_EncodedRlcPdu8; var NR_MAC_SDU_Type v_EncodedRlcPdu9; //@sic R5-225377 sic@ var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var SubFrameTiming_Type v_TimingUL_11A;//@sic R5-225377 sic@ var SubFrameTiming_Type v_TimingUL_12; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CRNTI ; var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CSRNTI; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL3; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL5; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL6; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL7; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL8; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL9; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL10; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL11; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL11A;//@sic R5-225377 sic@ var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL12; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL; timer t_Watchdog := 10.0; v_NR_ResourceAllocation_CRNTI := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS_CRNTI, v_NR_UplinkBWP,-, formats0_0_And_1_0); v_NR_ResourceAllocation_CSRNTI:= f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS_CSRNTI, v_NR_UplinkBWP,-, formats0_0_And_1_0); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); // Configure for indication of UL Data and UL grant on PDCCH // As per TS 38.508-1 Table 4.6.3-122 the default K2 is 4 slots v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 480); v_TimingUL_1:= v_TimingDL; v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); //@sic R5s230067 sic@ select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 5; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } // @siclog "Steps 3" siclog@ // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =18 byte =144 bits // 6 MAC Sub PDU's 6*144 = 864. 888 is next allowed downlink TBS in all tables, 24 bit =3 byte padding in DL. //@sic R5-227518 sic@ v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_12B, 1))); v_EncodedRlcPdu2 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_12B1, 1))); v_EncodedRlcPdu3 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_12B2, 1))); v_EncodedRlcPdu4 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_12B3, 1))); v_EncodedRlcPdu5 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_12B4, 1))); v_EncodedRlcPdu6 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_12B5, 1))); v_EncodedRlcPdu7 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_12B6, 1))); v_EncodedRlcPdu8 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_12B7, 1))); v_EncodedRlcPdu9 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (8, crs_NR_PDCP_SDU_12B8, 1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu4), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu5), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu6)}; v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu7), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu8),//@sic R5-225377 sic@ cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu9)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header +1 byte padding =19 byte =152 bits v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; // prepare UL PDU's v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ConfigGrantCnf_SubHeader),* }; //@sic R5s230507 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,60); //60 ms after f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); // Prepare step 6 v_TimingUL_1 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,60); //Step 6 timing v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; //Prepare step 7 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); //Step 7 Timing //@siclog "Step 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); //Prepare step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2);//Step 9 Timing //Prepare step 10 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(*, *, *) }; v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2);//Step 10 Timing //Prepare step 11 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_5 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2);//Step 11 Timing //@siclog "Step 12" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI, true); // NDI =1 //Prepare step 13 //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL6 := v_MAC_PDUListUL5; v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_5,const_SPS_Period/2);//Step 13 Timing //Prepare step 14 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL7 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period/2);//Step 14 Timing //@siclog "Step 15" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), v_NR_ResourceAllocation_CRNTI); //prepare step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL8 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period);//Step 16 Timing //prepare step 17 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_MAC_PDUListUL9 := { cr_NR_UL_MAC_PDU(omit, *, *) }; v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_8,const_SPS_Period);//Step 17 Timing //@siclog "Step 18" siclog@ //@sic R5s230507 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL1)); //prepare step 19 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL10 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,const_SPS_Period);//Step 19 Timing //@siclog "Step 20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_UL); //prepare step 21 //@sic R5-225377 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu8)}; v_MAC_PDUListUL11 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period);//Step 21 Timing //prepare step 21A //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_MAC_PDUListUL11A := { cr_NR_UL_MAC_PDU(*, *, *) }; v_TimingUL_11A := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_11,const_SPS_Period);//Step 21 Timing //@siclog "Step 22" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,2*const_SPS_Period); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); //prepare step 23 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu9)}; v_MAC_PDUListUL12 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; //@sic R5s230067 sic@ v_TimingUL_12 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_11,2*const_SPS_Period);//Step 23 Timing t_Watchdog.start; // @siclog "Step 6" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_1), v_MAC_PDUListUL1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL2)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_3), v_MAC_PDUListUL3)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 and 11" siclog@ alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_4), v_MAC_PDUListUL4)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_5), v_MAC_PDUListUL5)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } }; // @siclog "Step 13" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_6), v_MAC_PDUListUL6)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 14" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_7), v_MAC_PDUListUL7)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); // @siclog "Step 16" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_8), v_MAC_PDUListUL8)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @siclog "Step 17, 19" siclog@ alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_10), v_MAC_PDUListUL10)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_9), v_MAC_PDUListUL9)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } } // @siclog "Step 21" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_11), v_MAC_PDUListUL11)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 21A and 23" siclog@ alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_11A), v_MAC_PDUListUL11A)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21A"); repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_12), v_MAC_PDUListUL12)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); } }; t_Watchdog.stop; };

After Change:

function f_TC_7_1_1_6_3_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { const integer const_SPS_Period :=40; // period in milli seconds const integer const_UL_TBS_CRNTI := 144; // const integer const_UL_TBS_CSRNTI := 160; //160 bits var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu6; var NR_MAC_SDU_Type v_EncodedRlcPdu7; var NR_MAC_SDU_Type v_EncodedRlcPdu8; var NR_MAC_SDU_Type v_EncodedRlcPdu9; //@sic R5-225377 sic@ var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var SubFrameTiming_Type v_TimingUL_11A;//@sic R5-225377 sic@ var SubFrameTiming_Type v_TimingUL_12; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CRNTI ; var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CSRNTI; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL3; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL5; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL6; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL7; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL8; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL9; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL10; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL11; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL11A;//@sic R5-225377 sic@ var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL12; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); //WA#WI1432348 WA#7_1_1_6_3 timer t_Watchdog := 10.0; v_NR_ResourceAllocation_CRNTI := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS_CRNTI, v_NR_UplinkBWP,-, formats0_0_And_1_0); v_NR_ResourceAllocation_CSRNTI:= f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS_CSRNTI, v_NR_UplinkBWP,-, formats0_0_And_1_0); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); // Configure for indication of UL Data and UL grant on PDCCH // As per TS 38.508-1 Table 4.6.3-122 the default K2 is 4 slots v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 480); v_TimingUL_1:= v_TimingDL; v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); //@sic R5s230067 sic@ select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 5; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } // @siclog "Steps 3" siclog@ // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =18 byte =144 bits // 6 MAC Sub PDU's 6*144 = 864. 888 is next allowed downlink TBS in all tables, 24 bit =3 byte padding in DL. //@sic R5-227518 sic@ v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_12B, 1))); v_EncodedRlcPdu2 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_12B1, 1))); v_EncodedRlcPdu3 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_12B2, 1))); v_EncodedRlcPdu4 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_12B3, 1))); v_EncodedRlcPdu5 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_12B4, 1))); v_EncodedRlcPdu6 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_12B5, 1))); v_EncodedRlcPdu7 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_12B6, 1))); v_EncodedRlcPdu8 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_12B7, 1))); v_EncodedRlcPdu9 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (8, crs_NR_PDCP_SDU_12B8, 1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu2), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu3), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu4), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu5), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu6)}; v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu7), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu8),//@sic R5-225377 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu9)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header +1 byte padding =19 byte =152 bits v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; // prepare UL PDU's v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ConfigGrantCnf_SubHeader),* }; //@sic R5s230507 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,60); //60 ms after f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); // Prepare step 6 v_TimingUL_1 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,60); //Step 6 timing v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; //Prepare step 7 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); //Step 7 Timing //@siclog "Step 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); //Prepare step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2);//Step 9 Timing //Prepare step 10 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(*, *, *) }; v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2);//Step 10 Timing //Prepare step 11 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_5 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2);//Step 11 Timing //@siclog "Step 12" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI, true); // NDI =1 //Prepare step 13 //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL6 := v_MAC_PDUListUL5; v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_5,const_SPS_Period/2);//Step 13 Timing //Prepare step 14 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL7 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period/2);//Step 14 Timing //@siclog "Step 15" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), v_NR_ResourceAllocation_CRNTI); //prepare step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL8 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period);//Step 16 Timing //prepare step 17 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_MAC_PDUListUL9 := { cr_NR_UL_MAC_PDU(omit, *, *) }; v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_8,const_SPS_Period);//Step 17 Timing //@siclog "Step 18" siclog@ //@sic R5s230507 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL1)); //prepare step 19 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL10 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,const_SPS_Period);//Step 19 Timing //@siclog "Step 20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_UL); //prepare step 21 //@sic R5-225377 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu8)}; v_MAC_PDUListUL11 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period);//Step 21 Timing //prepare step 21A //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_MAC_PDUListUL11A := { cr_NR_UL_MAC_PDU(*, *, *) }; v_TimingUL_11A := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_11,const_SPS_Period);//Step 21 Timing //@siclog "Step 22" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,2*const_SPS_Period); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); //prepare step 23 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, int2bit(16, 8)), v_EncodedRlcPdu9)}; v_MAC_PDUListUL12 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; //@sic R5s230067 sic@ v_TimingUL_12 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_11,2*const_SPS_Period);//Step 23 Timing t_Watchdog.start; // @siclog "Step 6" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_1), v_MAC_PDUListUL1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL2)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_3), v_MAC_PDUListUL3)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 and 11" siclog@ alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_4), v_MAC_PDUListUL4)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_5), v_MAC_PDUListUL5)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } }; // @siclog "Step 13" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_6), v_MAC_PDUListUL6)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 14" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_7), v_MAC_PDUListUL7)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); // @siclog "Step 16" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_8), v_MAC_PDUListUL8)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @siclog "Step 17, 19" siclog@ alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_10), v_MAC_PDUListUL10)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_9), v_MAC_PDUListUL9)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } } // @siclog "Step 21" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_11), v_MAC_PDUListUL11)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 21A and 23" siclog@ alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_11A), v_MAC_PDUListUL11A)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21A"); repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_12), v_MAC_PDUListUL12)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); } }; t_Watchdog.stop; };

## Change 2

| Function name | f_TC_7_1_1_6_3_NR_TestBody2 |
| --- | --- |
| Reason for change | Same as reason mentioned in change 1 |
| Summary of change | Same as reason mentioned in change 1 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_3_NR_TestBody2(DRB_Identity p_NR_DRB_Id //SubFrameTiming_Type p_TimingUL )runs on NR_BASE_PTC { //const integer const_SPS_Period :=40; // period in milli seconds var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //var SubFrameTiming_Type v_TimingUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(*, *, *) }; timer t_Watchdog := 0.1; //@sic R5s230067 R5s230507 sic@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); //v_TimingUL := f_SubFrameTiming_AddMilliSeconds(p_TimingUL,const_SPS_Period *p_X); v_EncodedRlcPdu := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (9, crs_NR_PDCP_SDU_12B7, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := {cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); //@sic R5s230507 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 27"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); } } // Get back PDU from UE f_NR_ULGrantConfiguration_Start(nr_Cell1); DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) };

After Change:

function f_TC_7_1_1_6_3_NR_TestBody2(DRB_Identity p_NR_DRB_Id //SubFrameTiming_Type p_TimingUL )runs on NR_BASE_PTC { //const integer const_SPS_Period :=40; // period in milli seconds var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //var SubFrameTiming_Type v_TimingUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(*, *, *) }; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); //WA#WI1432348 WA#7_1_1_6_3 timer t_Watchdog := 0.1; //@sic R5s230067 R5s230507 sic@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); //v_TimingUL := f_SubFrameTiming_AddMilliSeconds(p_TimingUL,const_SPS_Period *p_X); v_EncodedRlcPdu := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (9, crs_NR_PDCP_SDU_12B7, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := {cs_NR_MAC_SDU_SubPDU_LI8(v_LCID /*WA#WI1432348 WA#7_1_1_6_3 tsc_LCID_SCG_Bearer*/, v_EncodedRlcPdu)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); //@sic R5s230507 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 27"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); } } // Get back PDU from UE f_NR_ULGrantConfiguration_Start(nr_Cell1); DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) };