3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250272

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4915 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NB-IOT NTN testcase 22.4.30 on NGSO band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To re-verify NB-IOT NTN testcase 22.4.30 on NGSO band using iWD D25wk24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NB-IOT NTN testcase 22.4.30 on NGSO band required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	7

4	Execution Log Files	8

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 22.4.30 which is part of the NBIOT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	22.4.30

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Nordic nRF9151 UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_22_4_30_NBIOT |
| --- | --- |
| Reason for change | Postmable of the testcase is removed as part of implementation of prose CR R5-253230. Hence it is proposed to re-add the step. |
| Summary of change | It is proposed to include postamble step after step 24B |
| TTCN module | NBIOT_RRC_NTN.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry @sic R5-235429, R5s230708 Ch2.1, R5s240295 Ch 2.2, R5-253230 removing TP3 (T318 Expiry) related steps 24C-32 sic@ // SKIPPED CODE f_NBIOT_TestBody_Set(false); }

After Change

function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry @sic R5-235429, R5s230708 Ch2.1, R5s240295 Ch 2.2, R5-253230 removing TP3 (T318 Expiry) related steps 24C-32 sic@ // SKIPPED CODE f_NBIOT_TestBody_Set(false); f_NBIOT_Postamble ( nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED ); }

## Change 2

| Function name | f_NBIOT_RRC_ConnectionRelease_Common |
| --- | --- |
| Reason for change | Function f_NBIOT_NTN_Get_T_NTNoffset returns t_NTNdelay of a cell that is a sum of Koffset and Kmac configured for the cell. In case of NGSO, Koffset and Kmac are set to very low value. Since the sum will be less than 100ms TTCN fails to get next SearchSpace for the rrcConnectionRelease message and in turn resulting in the testcase failure. |
| Summary of change | Proposed to assign the variable v_MilliSecondsDelay with the t_NTNdelay only in case of NTN GSO scenario. |
| TTCN module | NBIOT_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_NBIOT_RRC_ConnectionRelease_Common(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, template (value) DL_DCCH_Message_NB p_RRCConnectionRelease, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NB_SRB_Identity_Type p_SrbId := omit) runs on NBIOT_PTC { /* @sic R5s170250: p_IOT_State instead of p_SrbId sic@ */ var NB_SRB_Identity_Type v_SrbId; var SubFrameTiming_Type v_TimingAtT; // time T: sending of RRCConnectionRelease acc. to 36.523-3 clause 7A.7 var SubFrameTiming_Type v_TimingNow := f_NBIOT_GetCurrentTiming(p_CellId); var integer v_MilliSecondsDelay := 300; var integer v_Duration; timer t_Timer; if(f_NBIOT_MobileInfo_GetNTNSatelliteType() != ntn_None) {//@sic R5s230706, R5-237436 cases sic@ v_MilliSecondsDelay := f_NBIOT_NTN_Get_T_NTNoffset(p_CellId); } if (isvalue(p_SrbId)) { // @sic R5s180217 sic@ v_SrbId := valueof(p_SrbId); } else { v_SrbId := f_NBIOT_GetSrb1Id(p_IOT_State); } if (f_TimingInfo_IsNow(p_TimingInfo)) { v_TimingAtT := f_NBIOT_GetNextSearchSpace(p_CellId, v_MilliSecondsDelay); } else { // dedicated timing info is given v_TimingAtT := valueof(p_TimingInfo.SubFrame); } v_Duration := f_SubFrameTiming_Duration(v_TimingNow, v_TimingAtT) + tsc_NBIOT_DelayAfterRRCConnectionRelease; /* @sic R5s170263: tsc_NBIOT_DelayAfterRRCConnectionRelease sic@ */ t_Timer.start(int2float(v_Duration) / 1000.0); SRB.send(cas_NB_SRB_RrcPdu_REQ(p_CellId, v_SrbId, cs_TimingInfo_EUTRA_NB(v_TimingAtT), p_RRCConnectionRelease)); f_NBIOT_RRC_ConnectionRelease_Local(p_CellId, p_IOT_State, v_TimingAtT); alt { [] t_Timer.timeout {} [] SRB.check(receive(car_NB_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest_NB(cr_EstablishmentCause_NB_Any, ?)))) { /* @sic R5s170366: ? as InitialUE_Identity sic@ */ // there is an RRCConnectionRequest sent by the UE which can be received by TTCN-3 in a subsequent function t_Timer.stop; } } }

After Change

function f_NBIOT_RRC_ConnectionRelease_Common(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, template (value) DL_DCCH_Message_NB p_RRCConnectionRelease, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NB_SRB_Identity_Type p_SrbId := omit) runs on NBIOT_PTC { /* @sic R5s170250: p_IOT_State instead of p_SrbId sic@ */ var NB_SRB_Identity_Type v_SrbId; var SubFrameTiming_Type v_TimingAtT; // time T: sending of RRCConnectionRelease acc. to 36.523-3 clause 7A.7 var SubFrameTiming_Type v_TimingNow := f_NBIOT_GetCurrentTiming(p_CellId); var integer v_MilliSecondsDelay := 300; var integer v_Duration; timer t_Timer; if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO) {//@sic R5s230706, R5-237436 sic@ v_MilliSecondsDelay := f_NBIOT_NTN_Get_T_NTNoffset(p_CellId); } if (isvalue(p_SrbId)) { // @sic R5s180217 sic@ v_SrbId := valueof(p_SrbId); } else { v_SrbId := f_NBIOT_GetSrb1Id(p_IOT_State); } if (f_TimingInfo_IsNow(p_TimingInfo)) { v_TimingAtT := f_NBIOT_GetNextSearchSpace(p_CellId, v_MilliSecondsDelay); } else { // dedicated timing info is given v_TimingAtT := valueof(p_TimingInfo.SubFrame); } v_Duration := f_SubFrameTiming_Duration(v_TimingNow, v_TimingAtT) + tsc_NBIOT_DelayAfterRRCConnectionRelease; /* @sic R5s170263: tsc_NBIOT_DelayAfterRRCConnectionRelease sic@ */ t_Timer.start(int2float(v_Duration) / 1000.0); SRB.send(cas_NB_SRB_RrcPdu_REQ(p_CellId, v_SrbId, cs_TimingInfo_EUTRA_NB(v_TimingAtT), p_RRCConnectionRelease)); f_NBIOT_RRC_ConnectionRelease_Local(p_CellId, p_IOT_State, v_TimingAtT); alt { [] t_Timer.timeout {} [] SRB.check(receive(car_NB_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest_NB(cr_EstablishmentCause_NB_Any, ?)))) { /* @sic R5s170366: ? as InitialUE_Identity sic@ */ // there is an RRCConnectionRequest sent by the UE which can be received by TTCN-3 in a subsequent function t_Timer.stop; } } }

# Branches executed

The test case was executed on LTE primary FDD band 255 with NGSO mode using LTE ciphering eea1 and integrity algorithm eia1.

# Execution Log Files

  - Nordic nRF9151 UE

Nordic nRF9151 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_22_4_30_LOG.xml

- PICS/PIXIT parameter file:

TC_22_4_30_PIXIT.txt

# References

| [1] | R5s250269: Supporting information for addition of NB-IOT NTN testcase 22.4.30 on NGSO band |
| --- | --- |