3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250274

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3716 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR NTN testcase 7.1.2.3.9a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 38.523-1 Table 7.1.2.3.9a.3.2-1, at Step 2 a timer of 2s is started to check TP1. At Step 3 of the same Table, another timer of 2200ms is started from Step 1. Out of the 2200 ms (2.2s), 2s are already elapsed at Step 2, but the TTCN implementation at Step 3 is not taking this into account. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step3, correct the value, which need to subtract, from 1.0 to 2.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.9a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Overview	3

2	Corrections required to test case 7.1.2.3.9a	4

2.1	Change 1	4

# Overview

This document lists all the essencial changes needed to correct problems in the ATS iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 7.1.2.3.9a

## Change 1

| Function name | f_TC_7_1_2_3_9a_NR_TestBody () |
| --- | --- |
| Reason for change | According to 38.523-1 Table 7.1.2.3.9a.3.2-1, at Step 2 a timer of 2s is started to check TP1. At Step 3 of the same Table, another timer of 2200ms is started from Step 1. Out of the 2200 ms (2.2s), 2s are already elapsed at Step 2, but the TTCN implementation at Step 3 is not taking this into account. This needs to be corrected. |
| Summary of change | At step3, correct the value, which need to subtract, from 1.0 to 2.0. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @sic R5s250100 ch. 13.1 sic@ // @siclog "Step 2" siclog@ // Check: For 2 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); // @sic R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-1.0); // 2 sec of step 2 has already elapsed ...// WI = 1309760 // @siclog "Step 4" siclog@ // Check: For 2 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // @sic R5-250100 ch. 13.2 sic@ t_WatchDog.start(2.0); // @sic R5-250100 ch. 13.3, R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; // @sic R5-250100 ch. 13.4 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

After Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @sic R5s250100 ch. 13.1 sic@ // @siclog "Step 2" siclog@ // Check: For 2 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); // @sic R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-2.0); // 2 sec of step 2 has already elapsed ...// WI = 1309760 // @siclog "Step 4" siclog@ // Check: For 2 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // @sic R5-250100 ch. 13.2 sic@ t_WatchDog.start(2.0); // @sic R5-250100 ch. 13.3, R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; // @sic R5-250100 ch. 13.4 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }