3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250292

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3688 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UTRAN SSNITZ testcase 15.7.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the current TTCN implementation, AT command AT+CHLD=2 is sent to the UE after establishing Call A-D. This takes call A-D to HOLD. Prior to that, after the earlier two calls are in MultiParty, AT command AT+CHLD=2 is sent, which takes the MultiParty call to HOLD as well. After this, to clear off call A-D the AT command AT+CHLD=13 is sent (Step 1 of Prose) According to 22.030, cl 6.5.5.1 : Entering 1X followed by SEND - Releases a specific active call X. However, as explained above, none of the call is in active state. Thus the usage of AT command AT+CHLD=13 is incorrect, and causes error at UE side. This needs to be addressed. It is proposed to use AT+CHLD=0 instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Another function “f_UT_CallHoldRelease” is used to clear off a held call-AD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.7.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_15_7_20_UTRAN() runs on UTRAN_PTC |
| --- | --- |
| Reason for change | As per the current TTCN implementation, AT command AT+CHLD=2 is sent to the UE after establishing Call A-D. This takes call A-D to HOLD. Prior to that, after the earlier two calls are in MultiParty, AT command AT+CHLD=2 is sent, which takes the MultiParty call to HOLD as well. After this, to clear off call A-D the AT command AT+CHLD=13 is sent (Step 1 of Prose) According to 22.030, cl 6.5.5.1 : Entering 1X followed by SEND - Releases a specific active call X. However, as explained above, none of the call is in active state. Thus the usage of AT command AT+CHLD=13 is incorrect, and causes error at UE side. This needs to be addressed. It is proposed to use AT+CHLD=0 instead. |
| Summary of change | Another function “f_UT_CallHoldRelease” is used to clear off a held call-AD |
| TTCN module | MultiParty.ttcn |

Before Change:

function f_TC_15_7_20_UTRAN() runs on UTRAN_PTC { /* Multi-party, Managing a single call and a MultiParty call, Disconnect the single call, single call held */ var MultiTI_Type v_MultiTI_R; var TI v_TI_S_B; var TI v_TI_S_C; var TI v_TI_S_D; // Initialise all UTRAN cells according to 34.108 f_UTRAN34_Init(); f_UTRAN34_SS_CreateCellDCH(utran34_Cell1); f_UTRAN34_SendDefSysInfo(utran34_Cell1); f_UTRAN34_Preamble(utran34_Cell1); <<SKIPPED CODE>> // @siclog "Step 1" siclog@ // Using MMI or AT commands, clear call A-D f_UT_CallResume(UT, 3); // @siclog "Step 2-3-4" siclog@ //DISCONNECT, RELEASE and RELEASE COMPLETE with Transaction identifier of call A-D f_UTRAN_CC_DisconnectMO(v_MultiTI_R.A_D); //@sic R5s150601 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_15_7_20_UTRAN() runs on UTRAN_PTC { /* Multi-party, Managing a single call and a MultiParty call, Disconnect the single call, single call held */ var MultiTI_Type v_MultiTI_R; var TI v_TI_S_B; var TI v_TI_S_C; var TI v_TI_S_D; // Initialise all UTRAN cells according to 34.108 f_UTRAN34_Init(); f_UTRAN34_SS_CreateCellDCH(utran34_Cell1); f_UTRAN34_SendDefSysInfo(utran34_Cell1); f_UTRAN34_Preamble(utran34_Cell1); <<SKIPPED CODE>> // @siclog "Step 1" siclog@ // Using MMI or AT commands, clear call A-D f_UT_CallHoldRelease(UT); // @siclog "Step 2-3-4" siclog@ //DISCONNECT, RELEASE and RELEASE COMPLETE with Transaction identifier of call A-D f_UTRAN_CC_DisconnectMO(v_MultiTI_R.A_D); //@sic R5s150601 sic@ <<SKIPPED CODE>>