3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250302

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3652 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC testcases 7.1.1.3.1, 7.1.1.3.4, 7.1.1.3.5 and 7.1.1.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MAC is put into Normal Mode from Transparent Mode immediately after sending RLC STATUS PDU at the end of the TestBody. However, this creates instability in the affected TCs as there is no certainity that the RLC STATUS PDU has indeed been transmitted to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a small delay of 50ms to ensure that the RLC STATUS PDU has been transmitted to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.1.NR5GC, 7.1.1.3.4.NR5GC, 7.1.1.3.5.NR5GC, 7.1.1.3.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_1_NR5GC |
| --- | --- |
| Reason for change | MAC is put into Normal Mode from Transparent Mode immediately after sending RLC STATUS PDU at the end of the TestBody. However, this creates instability in the affected TCs as there is no certainity that the RLC STATUS PDU has indeed been transmitted to the UE |
| Summary of change | Introduced a small delay of 50ms to ensure that the RLC STATUS PDU has been transmitted to the UE. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_1_NR5GC() runs on NR5GC_PTC { // Correct Handling of UL MAC PDU / Assignment / HARQ process <<SKIPPED CODE>> f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_1_NR5GC() runs on NR5GC_PTC { // Correct Handling of UL MAC PDU / Assignment / HARQ process <<SKIPPED CODE>> f_NR_TestBody_Set(false); f_Delay(0.05); //WA# //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 2

| Function name | f_TC_7_1_1_3_4_NR5GC |
| --- | --- |
| Reason for change | MAC is put into Normal Mode from Transparent Mode immediately after sending RLC STATUS PDU at the end of the TestBody. However, this creates instability in the affected TCs as there is no certainity that the RLC STATUS PDU has indeed been transmitted to the UE |
| Summary of change | Introduced a small delay of 50ms to ensure that the RLC STATUS PDU has been transmitted to the UE. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_4_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Regular BSR <<SKIPPED CODE>> f_NR_TestBody_Set(false); //Postamble //@sic R5s190374 sic@ // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_4_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Regular BSR <<SKIPPED CODE>> f_NR_TestBody_Set(false); f_Delay(0.05); //WA# //Postamble //@sic R5s190374 sic@ // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 3

| Function name | f_TC_7_1_1_3_5_NR5GC |
| --- | --- |
| Reason for change | MAC is put into Normal Mode from Transparent Mode immediately after sending RLC STATUS PDU at the end of the TestBody. However, this creates instability in the affected TCs as there is no certainity that the RLC STATUS PDU has indeed been transmitted to the UE |
| Summary of change | Introduced a small delay of 50ms to ensure that the RLC STATUS PDU has been transmitted to the UE. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_5_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR <<SKIPPED CODE>> f_NR_TestBody_Set(false); //Postamble // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_5_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR <<SKIPPED CODE>> f_NR_TestBody_Set(false); f_Delay(0.05); //WA# //Postamble // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 4

| Function name | f_TC_7_1_1_3_6_NR5GC |
| --- | --- |
| Reason for change | MAC is put into Normal Mode from Transparent Mode immediately after sending RLC STATUS PDU at the end of the TestBody. However, this creates instability in the affected TCs as there is no certainity that the RLC STATUS PDU has indeed been transmitted to the UE |
| Summary of change | Introduced a small delay of 50ms to ensure that the RLC STATUS PDU has been transmitted to the UE. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_6_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer status / Periodic BSR timer expires <<SKIPPED CODE>> f_NR_TestBody_Set(false); //Postamble // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_6_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer status / Periodic BSR timer expires <<SKIPPED CODE>> f_NR_TestBody_Set(false); f_Delay(0.05); //WA# //Postamble // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };