3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250308

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3679 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 NR NTN test case 7.1.3.5.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 7.1.3.5.1a to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.3.5.1a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 7.1.3.5.1a	4

2.1	Change 1	4

3	Branches executed	6

4	Execution Log Files	6

4.1	Exynos 2500	6

5	References	7

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.3.5.1a which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1.3.5.1a.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos 2500

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_5_1a_NR_TestBody() |
| --- | --- |
| Reason for change | - Prose 38.523-1, is expecting Data PDU = 62 bytes at step 6, but in TTCN 64 bytes were allocated. - Step7, is not expecting any UL PDU from UE. But Sequence number of DL PDU is increamented here at step6, And on following step8 DL PDU is transmitted with increamented Sequence number and for UL data PDU TTCN should be expecting same increamented sequence number at step10. But currently TTCN expecting same sequence number at step10, what SS transmitted on step6 |
| Summary of change | - Updated v_SduList for correcting data size at step6. - Increamented UL PDCP sequence number by 1 at step7. |
| TTCN module | PDCP_NTN_NR5GC.ttcn |

Before Change:

function f_TC_7_1_3_5_1a_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var NR_PDCP_SDUList_Type v_SduList; var float v_T_discardTimer := 500.0; var float v_T_discardTimerExt := 2000.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //Initilize SDU to be sent v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 64, 94}); //The SS does not allocate UL grants unless when explicitly stated so in the procedure f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_T_discardTimer := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer,v_T_discardTimer); //DiscardTimer 500 ms v_TimingInfo2 := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfo1, float2int(v_T_discardTimer)); //@siclog "Step 1" siclog@ The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 46 bytes. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[0], v_TimingInfo1); //@siclog "Step 2" siclog@ Wait Discard timer to expire. //@siclog "Step 3" siclog@ The SS resumes normal UL grant allocation. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 4" Does UE transmit a PDCP Data PDU of size 46 bytes? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_SduList[0], cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ Configure SS not to allocate UL grant to the UE f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 62 bytes v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[1], v_TimingInfo1); //@siclog "Step 7" siclog@ Wait for discardTimerExt2-r17 to expire v_T_discardTimerExt := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer, v_T_discardTimerExt); //DiscardTimer 2000 ms v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, float2int(v_T_discardTimerExt)); //@siclog "Step 8" siclog@ The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 94 bytes. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[2], v_TimingInfo2); //@siclog "Step 9" siclog@ The SS resumes normal UL grant allocation. v_TimingInfo2 := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfo1, float2int(v_T_discardTimerExt)); f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 10" siclog@ The SS resumes normal UL grant allocation. f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_SduList[2], cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); }

After Change:

function f_TC_7_1_3_5_1a_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var NR_PDCP_SDUList_Type v_SduList; var float v_T_discardTimer := 500.0; var float v_T_discardTimerExt := 2000.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //Initilize SDU to be sent v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 62, 94}); //The SS does not allocate UL grants unless when explicitly stated so in the procedure f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_T_discardTimer := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer,v_T_discardTimer); //DiscardTimer 500 ms v_TimingInfo2 := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfo1, float2int(v_T_discardTimer)); //@siclog "Step 1" siclog@ The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 46 bytes. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[0], v_TimingInfo1); //@siclog "Step 2" siclog@ Wait Discard timer to expire. //@siclog "Step 3" siclog@ The SS resumes normal UL grant allocation. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 4" Does UE transmit a PDCP Data PDU of size 46 bytes? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_SduList[0], cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ Configure SS not to allocate UL grant to the UE f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 62 bytes v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[1], v_TimingInfo1); //@siclog "Step 7" siclog@ Wait for discardTimerExt2-r17 to expire v_T_discardTimerExt := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer, v_T_discardTimerExt); //DiscardTimer 2000 ms v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, float2int(v_T_discardTimerExt)); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State, 1); // Increamented sequence number count, as it will not be updated due to Rx PDU is not expected at this step //@siclog "Step 8" siclog@ The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 94 bytes. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[2], v_TimingInfo2); //@siclog "Step 9" siclog@ The SS resumes normal UL grant allocation. v_TimingInfo2 := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfo1, float2int(v_T_discardTimerExt)); f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 10" siclog@ The SS resumes normal UL grant allocation. f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_SduList[2], cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms in GSO on Anritsu 5G Conformance Test platform.

# Execution Log Files

## Samsung Exynos 2500

The Samsung Exynos 2500 UE  passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_3_5_1a_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_3_5_1a_PIXIT.xml

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250192: Supporting information for agreement of TC 7.1.3.5.1a |
| --- | --- |