3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250322

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3700 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to templates cads_NR_TDD_UL_DL_ConfigCommon_REQ and cads_NR_Common_UplinkBWP_ConfigCommon_REQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of 25wk24 , “ERedCap” is introduced as part of NR_CellConfigPhysicalLayerCommon_Type. This leads to uninitialised run time issue when using the templates cads_NR_TDD_UL_DL_ConfigCommon_REQ and cads_NR_Common_UplinkBWP_ConfigCommon_REQ. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To avoid the run time issue , it is proposed to initialise ERedCap in the affected templates . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail due to run time exception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1a.NR5GC , 7.1.1.2.1.ENDC , 7.1.1.2.1.NR5GC , 7.1.1.3.1.ENDC , 7.1.1.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | cads_NR_TDD_UL_DL_ConfigCommon_REQ |
| --- | --- |
| Reason for change | “ERedCap” is introduced as part of NR_CellConfigPhysicalLayerCommon_Type. This leads to uninitialised run time issue when using the template cads_NR_TDD_UL_DL_ConfigCommon_REQ |
| Summary of change | To avoid the run time issue , it is proposed to initialise ERedCap in the affected template |
| TTCN module | NR_TC_Common\7_1_1\MAC_TC_Common_NR.ttcn |

Before Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_TDD_UL_DL_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) TDD_UL_DL_ConfigCommon p_TDD_UL_DL_ConfigCommon) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { PhysicalCellId := omit, //@sic R5s211670 sic@ DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon }, Dedicated := omit, // @sic R5s220678 sic@ SharedSpectrum := omit //@sic R5s240542 sic@ } } } }, Downlink := omit, Uplink := omit } } } } };

After Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_TDD_UL_DL_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) TDD_UL_DL_ConfigCommon p_TDD_UL_DL_ConfigCommon) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { PhysicalCellId := omit, //@sic R5s211670 sic@ DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon }, Dedicated := omit, // @sic R5s220678 sic@ SharedSpectrum := omit //@sic R5s240542 sic@ } } }, //WA#WI=1426287 ERedCap := pc_supportOfERedCap_r18 //WA#WI=1426287 }, Downlink := omit, Uplink := omit } } } } };

## Change 2

| Template name | cads_NR_Common_UplinkBWP_ConfigCommon_REQ |
| --- | --- |
| Reason for change | “ERedCap” is introduced as part of NR_CellConfigPhysicalLayerCommon_Type. This leads to uninitialised run time issue when using the template cads_NR_Common_UplinkBWP_ConfigCommon_REQ. |
| Summary of change | To avoid the run time issue , it is proposed to initialise ERedCap in the affected template |
| TTCN module | NR_TC_Common\7_1_1\MAC_TC_Common_NR.ttcn |

Before Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_Common_UplinkBWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_UplinkBWP_List_Type p_NR_UplinkBWP_List, template (value) TDD_UL_DL_ConfigCommon p_TDD_UL_DL_ConfigCommon) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (NR5GC) */ //@sic R5s211670 sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { PhysicalCellId := omit, DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon }, Dedicated := omit, //@sic R5s240571 sic@ SharedSpectrum := omit } } } }, Downlink := omit, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := omit, BwpArray := p_NR_UplinkBWP_List, InitialBWP_RedCap := omit, //@sic R5s230289 sic@ BWP_SwitchIndicator := omit //@sic R5w240304 sic@ }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } }, SupplementaryUplink := omit, TimingAdvance := omit, PUSCH_ServingCellConfig := omit, PUSCH_ServingCellConfigSUL := omit } } } } } };

After Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_Common_UplinkBWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_UplinkBWP_List_Type p_NR_UplinkBWP_List, template (value) TDD_UL_DL_ConfigCommon p_TDD_UL_DL_ConfigCommon) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (NR5GC) */ //@sic R5s211670 sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { PhysicalCellId := omit, DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon }, Dedicated := omit, //@sic R5s240571 sic@ SharedSpectrum := omit } } }, //WA#WI=1426287 ERedCap := pc_supportOfERedCap_r18 //WA#WI=1426287 }, Downlink := omit, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := omit, BwpArray := p_NR_UplinkBWP_List, InitialBWP_RedCap := omit, //@sic R5s230289 sic@ BWP_SwitchIndicator := omit //@sic R5w240304 sic@ }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } }, SupplementaryUplink := omit, TimingAdvance := omit, PUSCH_ServingCellConfig := omit, PUSCH_ServingCellConfigSUL := omit } } } } } };