3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250329

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0208 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the agreement in R5-253162, some of the Rel-18 IEs were added to function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList() as a mandatory IE. But Rel-16 UE is failing the affected testcases as Rel-16 UE is unable to send the required IEs of Rel-18. Rel-18 IEs are applicable to Rel-18 UE onwards and can’t be made mandatory for Rel-16 UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed AND condition to OR defined in function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList() for the corresponding newly added R18 pics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2_5s.NR5GC, 9.3.1.2.19s.NR5GC, 9.3.1.2.20s.NR5GC, 9.3.1.2.21s.NR5GC, 9.3.4.2.19s.NR5GC, 9.3.4.2.20s.NR5GC, 9.3.4.2.21s.NR5GC, 9.3.4.4.19s.NR5GC, 9.3.4.4.20s.NR5GC, 9.3.4.4.21s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList() |
| --- | --- |
| Reason for change | As per the agreement in R5-253162, some of the Rel-18 IEs were added to function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList() as a mandatory IE. But Rel-16 UE is failing the affected testcases as Rel-16 UE is unable to send the required IEs of Rel-18. Rel-18 IEs are applicable to Rel-18 UE onwards and can’t be made mandatory for Rel-16 UEs. |
| Summary of change | Changed AND condition to OR defined in function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList() for the corresponding newly added R18 pics |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList (NR_DL_PRS_ProcessingCapability_r16.prs_ProcessingCapabilityBandList_r16 p_NrDlPrsProcessingCapabilityBandList) runs on NR_BASE_PTC { //@sic POS Rel-16 R5-213638 sic@ var integer v_Index; var boolean v_Found_SupportedBandwidthPRS_FR1 := false; var boolean v_Found_SupportedBandwidthPRS_FR2 := false; var boolean v_Found_Dl_PRS_BufferType := false; var boolean v_Found_DurationOfPRS_ProcessingSymbols := false; var boolean v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := false; var boolean v_Found_SupportedDL_PRS_ProcessingSamples_RRC_CONNECTED := false; //@sic R5-235341 sic@ var boolean v_Found_Prs_ProcessingWindowType1A := false; //@sic R5-235341 sic@ var boolean v_Found_Prs_ProcessingWindowType1B := false; //@sic R5-235341 sic@ var boolean v_Found_Prs_ProcessingWindowType2 := false; //@sic R5-235341 sic@ var boolean v_Found_MaximumPRS_BandwidthAcrossAllHopsFR1_r18 := false; //@sic R5-253162 sic@ var boolean v_Found_MaximumPRS_BandwidthAcrossAllHopsFR2_r18 := false; //@sic R5-253162 sic@ for (v_Index := 0; v_Index < lengthof(p_NrDlPrsProcessingCapabilityBandList); v_Index:=v_Index+1) { fl_CheckNRBandCapability(p_NrDlPrsProcessingCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1 == pc_supportedBandwidthPRS_FR1)) { v_Found_SupportedBandwidthPRS_FR1 := true; } if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2 == pc_supportedBandwidthPRS_FR2)) { v_Found_SupportedBandwidthPRS_FR2 := true; } if (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_BufferType_r16 == pc_dl_PRS_BufferType) { v_Found_Dl_PRS_BufferType := true; } if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbols_r16 == pc_durationOfPRS_ProcessingSymbols) { // R5w220203 v_Found_DurationOfPRS_ProcessingSymbols := true; } if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbolsInEveryTms_r16 == pc_PRS_ProcessingSymbolsInEveryTms) { v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedDL_PRS_ProcessingSamples_RRC_CONNECTED_r17) == pc_supportedDL_PRS_ProcessingSamples_RRC_CONNECTED)) { //@sic R5-235341, R5s230755 sic@ v_Found_SupportedDL_PRS_ProcessingSamples_RRC_CONNECTED := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingWindowType1A_r17)) == pc_prs_ProcessingWindowType1A) { //@sic R5-235341, R5s230755 sic@ v_Found_Prs_ProcessingWindowType1A := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingWindowType1B_r17)) == pc_prs_ProcessingWindowType1B) { //@sic R5-235341, R5s230755 sic@ v_Found_Prs_ProcessingWindowType1B := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingWindowType2_r17)) == pc_prs_ProcessingWindowType2) { //@sic R5-235341, R5s230755 sic@ v_Found_Prs_ProcessingWindowType2 := true; } if (ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingCapabilityOutsideMGinPPW_r17)) { f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityOutsideMGinPPW(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingCapabilityOutsideMGinPPW_r17); } if (ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18)) { //@sic R5-253162 sic@ if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR1_r18)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR1_r18 == pc_maximumPRS_BandwidthAcrossAllHopsFR1_value)) { v_Found_MaximumPRS_BandwidthAcrossAllHopsFR1_r18 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR2_r18)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR2_r18 == pc_maximumPRS_BandwidthAcrossAllHopsFR2_value)) { v_Found_MaximumPRS_BandwidthAcrossAllHopsFR2_r18 := true; } } } if (not ((v_Found_SupportedBandwidthPRS_FR1 or v_Found_SupportedBandwidthPRS_FR2) and v_Found_Dl_PRS_BufferType and v_Found_DurationOfPRS_ProcessingSymbols and v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms and (v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120) and v_Found_SupportedDL_PRS_ProcessingSamples_RRC_CONNECTED and v_Found_Prs_ProcessingWindowType1A and v_Found_Prs_ProcessingWindowType1B and v_Found_Prs_ProcessingWindowType2 and v_Found_MaximumPRS_BandwidthAcrossAllHopsFR1_r18 and v_Found_MaximumPRS_BandwidthAcrossAllHopsFR2_r18)) { //@sic R5s221041, R5-235341, R5-253162 sic@ f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for Prs_ProcessingCapabilityBandList"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList (NR_DL_PRS_ProcessingCapability_r16.prs_ProcessingCapabilityBandList_r16 p_NrDlPrsProcessingCapabilityBandList) runs on NR_BASE_PTC { //@sic POS Rel-16 R5-213638 sic@ var integer v_Index; var boolean v_Found_SupportedBandwidthPRS_FR1 := false; var boolean v_Found_SupportedBandwidthPRS_FR2 := false; var boolean v_Found_Dl_PRS_BufferType := false; var boolean v_Found_DurationOfPRS_ProcessingSymbols := false; var boolean v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := false; var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := false; var boolean v_Found_SupportedDL_PRS_ProcessingSamples_RRC_CONNECTED := false; //@sic R5-235341 sic@ var boolean v_Found_Prs_ProcessingWindowType1A := false; //@sic R5-235341 sic@ var boolean v_Found_Prs_ProcessingWindowType1B := false; //@sic R5-235341 sic@ var boolean v_Found_Prs_ProcessingWindowType2 := false; //@sic R5-235341 sic@ var boolean v_Found_MaximumPRS_BandwidthAcrossAllHopsFR1_r18 := false; //@sic R5-253162 sic@ var boolean v_Found_MaximumPRS_BandwidthAcrossAllHopsFR2_r18 := false; //@sic R5-253162 sic@ for (v_Index := 0; v_Index < lengthof(p_NrDlPrsProcessingCapabilityBandList); v_Index:=v_Index+1) { fl_CheckNRBandCapability(p_NrDlPrsProcessingCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1 == pc_supportedBandwidthPRS_FR1)) { v_Found_SupportedBandwidthPRS_FR1 := true; } if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2 == pc_supportedBandwidthPRS_FR2)) { v_Found_SupportedBandwidthPRS_FR2 := true; } if (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_BufferType_r16 == pc_dl_PRS_BufferType) { v_Found_Dl_PRS_BufferType := true; } if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbols_r16 == pc_durationOfPRS_ProcessingSymbols) { // R5w220203 v_Found_DurationOfPRS_ProcessingSymbols := true; } if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbolsInEveryTms_r16 == pc_PRS_ProcessingSymbolsInEveryTms) { v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) { v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedDL_PRS_ProcessingSamples_RRC_CONNECTED_r17) == pc_supportedDL_PRS_ProcessingSamples_RRC_CONNECTED)) { //@sic R5-235341, R5s230755 sic@ v_Found_SupportedDL_PRS_ProcessingSamples_RRC_CONNECTED := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingWindowType1A_r17)) == pc_prs_ProcessingWindowType1A) { //@sic R5-235341, R5s230755 sic@ v_Found_Prs_ProcessingWindowType1A := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingWindowType1B_r17)) == pc_prs_ProcessingWindowType1B) { //@sic R5-235341, R5s230755 sic@ v_Found_Prs_ProcessingWindowType1B := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingWindowType2_r17)) == pc_prs_ProcessingWindowType2) { //@sic R5-235341, R5s230755 sic@ v_Found_Prs_ProcessingWindowType2 := true; } if (ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingCapabilityOutsideMGinPPW_r17)) { f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityOutsideMGinPPW(p_NrDlPrsProcessingCapabilityBandList[v_Index].prs_ProcessingCapabilityOutsideMGinPPW_r17); } if (ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18)) { //@sic R5-253162 sic@ log("TESTA:"); if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR1_r18)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR1_r18 == pc_maximumPRS_BandwidthAcrossAllHopsFR1_value)) { v_Found_MaximumPRS_BandwidthAcrossAllHopsFR1_r18 := true; } if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR2_r18)) and (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_MeasurementWithRxFH_RRC_Connected_r18.maximumPRS_BandwidthAcrossAllHopsFR2_r18 == pc_maximumPRS_BandwidthAcrossAllHopsFR2_value)) { v_Found_MaximumPRS_BandwidthAcrossAllHopsFR2_r18 := true; } } } if (not ((v_Found_SupportedBandwidthPRS_FR1 or v_Found_SupportedBandwidthPRS_FR2) and v_Found_Dl_PRS_BufferType and v_Found_DurationOfPRS_ProcessingSymbols and v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms and (v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120) and v_Found_SupportedDL_PRS_ProcessingSamples_RRC_CONNECTED and v_Found_Prs_ProcessingWindowType1A and v_Found_Prs_ProcessingWindowType1B and v_Found_Prs_ProcessingWindowType2 or v_Found_MaximumPRS_BandwidthAcrossAllHopsFR1_r18 or v_Found_MaximumPRS_BandwidthAcrossAllHopsFR2_r18)) { //@sic R5s221041, R5-235341, R5-253162 sic@ f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for Prs_ProcessingCapabilityBandList"); } } <<SKIPPED CODE>>