3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250335

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3732 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current TTCN implementation for starting the timer at step 3B is not aligned with test procedure as TTCN is starting the timer before step 3A. Due to this in case of some UE this timer gets expired before MO Release Voice call (Annex A.7 of TS 34.229-5) procedure is completed leading to failing of the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed TTCN Implementation to start step 3B timer after step 3A completion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	9

2.3	Change 3	13

2.4	Change 4	14

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_11_4_1_TestBody |
| --- | --- |
| Reason for change | Current TTCN implementation for starting the timer at step 3B is not aligned with test procedure as TTCN is starting the timer before step 3A. Due to this in case of some UE this timer gets expired before MO Release Voice call (Annex A.7 of TS 34.229-5) procedure is completed leading to failing of the test case. |
| Summary of change | Changed TTCN Implementation to start step 3B timer after step 3A completion |
| TTCN module | NR5GC\11_4\Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_4_1_TestBody() runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN_Cell1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionIdToRelease; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer i; var float v_T3520 := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 15.0); //@sic R5s221039 sic@ var ProcedureTransactionIdentifier v_PTI; //@sic R5s240233 sic@ //@siclog "Step 1" siclog@ //Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. f_UT_RequestIMSEmergencyCall (UT, "144"); //@siclog "Step 2" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration? f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 2A" siclog@ //The SS initiates the 5G AKA based primary authentication and key agreement procedure by sending an AUTHENTICATION REQUEST message; v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN_Cell1); v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, //@sic R5s221039 sic@ -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 2B" siclog@ //The UE sends an AUTHENTICATION FAILURE message with 5GMM cause #71 "ngKSI already in use" SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s221039 sic@ if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_AUTHENTICATION_FAILURE(cr_GMM_GSM_Cause(omit, '01000111'B)))) { // ngKSI already in use f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2B"); } //@siclog "Step 2BA" siclog@ //Wait for T3520 expires. f_Delay(v_T3520); //@sic R5s221039 sic@ //@siclog "Step 2C" siclog@ //PDU session release procedure to release non-IMS Emergency relevant PDUs. //Make the UE to transmit a PDU SESSION RELEASE REQ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //@sic R5s230143 sic@ //Reset the stored messages, to capture the PDU Session Release REQs f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Capture PDU Session Release REQs { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp); } v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Release_Request.procedureTransactionIdentifier; //@sic R5s240233 sic@ for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Now release fully the captured PDU Session Release REQs { if (v_PDUSessionInfo[i].PDUType != Emergency_PDN) { v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i+1); if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2C-1"); } f_NR5GC_PDUSessionRelease(nr_Cell1, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100100'B), -, -, -, -,v_PTI); //@sic R5s240233 sic@ } } //@siclog "Step 2CA" siclog@ //@sic R5-237482 sic@ //The SS is configured to Stop RA response f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s240136 sic@ //@siclog "Step 3-3BA" siclog@ //@sic R5-253194 sic@ f_ReleaseEmergencyCallWithOptDeregisterOrPDURelease(nr_Cell1); //@siclog "Step 3Ca1-3E" siclog@ //Void @sic R5-253194 sic@ f_Delay(0.1); //@siclog "Step 3EA" siclog@ //@sic R5-237482 sic@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); f_NR5GC_ResetIMS(PDN_1); //@sic R5s240026 sic@ //@siclog "Step 3EB" siclog@ //@sic R5-237482 sic@ //SS configures the RA Response. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@siclog "Step 3F-9A2" siclog@ //Void //@sic R5-237482 sic@ //@siclog "Step 9A3 - 9A21a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, -, Initial_NoSecurity); //@sic R5s230748 sic@ //@siclog "Step 9A22-23" siclog@ //Void //@sic R5-237482 sic@ }

After Change

function fl_TC_11_4_1_TestBody() runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN_Cell1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionIdToRelease; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer i; var float v_T3520 := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 15.0); //@sic R5s221039 sic@ var ProcedureTransactionIdentifier v_PTI; //@sic R5s240233 sic@ //@siclog "Step 1" siclog@ //Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. f_UT_RequestIMSEmergencyCall (UT, "144"); //@siclog "Step 2" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration? f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 2A" siclog@ //The SS initiates the 5G AKA based primary authentication and key agreement procedure by sending an AUTHENTICATION REQUEST message; v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN_Cell1); v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, //@sic R5s221039 sic@ -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 2B" siclog@ //The UE sends an AUTHENTICATION FAILURE message with 5GMM cause #71 "ngKSI already in use" SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s221039 sic@ if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_AUTHENTICATION_FAILURE(cr_GMM_GSM_Cause(omit, '01000111'B)))) { // ngKSI already in use f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2B"); } //@siclog "Step 2BA" siclog@ //Wait for T3520 expires. f_Delay(v_T3520); //@sic R5s221039 sic@ //@siclog "Step 2C" siclog@ //PDU session release procedure to release non-IMS Emergency relevant PDUs. //Make the UE to transmit a PDU SESSION RELEASE REQ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //@sic R5s230143 sic@ //Reset the stored messages, to capture the PDU Session Release REQs f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Capture PDU Session Release REQs { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp); } v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Release_Request.procedureTransactionIdentifier; //@sic R5s240233 sic@ for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Now release fully the captured PDU Session Release REQs { if (v_PDUSessionInfo[i].PDUType != Emergency_PDN) { v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i+1); if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2C-1"); } f_NR5GC_PDUSessionRelease(nr_Cell1, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100100'B), -, -, -, -,v_PTI); //@sic R5s240233 sic@ } } //@siclog "Step 2CA" siclog@ //@sic R5-237482 sic@ //The SS is configured to Stop RA response f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s240136 sic@ //@siclog "Step 3-3BA" siclog@ //@sic R5-253194 sic@ f_ReleaseEmergencyCallWithOptDeregisterOrPDURelease_11_4_1(nr_Cell1); //@siclog "Step 3Ca1-3E" siclog@ //Void @sic R5-253194 sic@ f_Delay(0.1); //@siclog "Step 3EA" siclog@ //@sic R5-237482 sic@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); f_NR5GC_ResetIMS(PDN_1); //@sic R5s240026 sic@ //@siclog "Step 3EB" siclog@ //@sic R5-237482 sic@ //SS configures the RA Response. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@siclog "Step 3F-9A2" siclog@ //Void //@sic R5-237482 sic@ //@siclog "Step 9A3 - 9A21a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, -, Initial_NoSecurity); //@sic R5s230748 sic@ //@siclog "Step 9A22-23" siclog@ //Void //@sic R5-237482 sic@ }

## Change 2

| Function name | function f_ReleaseEmergencyCallWithOptDeregisterOrPDURelease_11_4_1 |
| --- | --- |
| Reason for change | Current TTCN implementation for starting the timer at step 3B is not aligned with test procedure as TTCN is starting the timer before step 3A. Due to this in case of some UE this timer gets expired before MO Release Voice call (Annex A.7 of TS 34.229-5) procedure is completed leading to failing of the test case. |
| Summary of change | Changed TTCN Implementation to start step 3B timer after step 3A completion |
| TTCN module | NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_ReleaseEmergencyCallWithOptDeregisterOrPDURelease (NR_CellId_Type p_NR_CellId, float p_WaitTimer := 5.0) runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var O1_Type v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var boolean v_WaitForReleaseComplete := false; timer t_Wait := p_WaitTimer; // @sic R5-223439, R5-235301 sic@ <<skipped code>>

After Change

function f_ReleaseEmergencyCallWithOptDeregisterOrPDURelease (NR_CellId_Type p_NR_CellId, float p_WaitTimer := 5.0) runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var O1_Type v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var boolean v_WaitForReleaseComplete := false; timer t_Wait := p_WaitTimer; // @sic R5-223439, R5-235301 sic@ <<skipped code>> function f_ReleaseEmergencyCallWithOptDeregisterOrPDURelease_11_4_1 (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var O1_Type v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var boolean v_WaitForReleaseComplete := false; //timer t_Wait := p_WaitTimer; // @sic R5-223439, R5-235301 sic@ //@siclog "Step 5" siclog@ //Make the UE release the emergency call. // @sic R5-223439 sic@ //@siclog "Step 6A-6C" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5-225362 sic@ //@siclog "Step 7" siclog@ //t_Wait.start; // @sic R5s220560 sic@ //EXCEPTION: In parallel to step 8 below the steps specified in the parallel behaviours may take place alt { [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")) // @sic R5-225362 sic@ sic@ { repeat} // @sic R5s221113 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_RxdMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu; // The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". if (ischosen(v_RxdMsg.Msg.deregistration_RequestMO)) { if (not f_Check_NG_DeregistrationReqMOMsg (v_RxdMsg, crs_DeregisterType('0'B, '0'B,'01'B))) { // Jun24 @sic R5s240531 Baseline Moving sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request failed"); } //The SS replies with a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); } // @sic R5-223439 sic@ // The UE transmits a PDU SESSION RELEASE REQUEST message else if (ischosen(v_RxdMsg.PiggybackedPduList[0].Msg.pdu_Session_Release_Request)) { if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfoList, v_RxdMsg, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } // set flag to expect complete v_WaitForReleaseComplete := true; // Complete the release procedure v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); } // The UE transmits a PDU SESSION RELEASE COMPLETE message? else if (v_WaitForReleaseComplete and ischosen(v_RxdMsg.PiggybackedPduList[0].Msg.pdu_Session_Release_Complete)) { if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_RxdMsg, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } v_WaitForReleaseComplete := false; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } // Keep going round until timer expires @sic R5-223439 sic@ repeat; } //@siclog "Step 8" siclog@ //[] t_Wait.timeout //@sic R5-216198 sic@ [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("Step 3BA timeOut")) { // if Release Complete hasn't arrived before timeout, then need to wait for it so it doesn't cause an error when it does arrive if (v_WaitForReleaseComplete) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } } } } …

## Change 3

| Function name | f_TC_11_4_1_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Current TTCN implementation for starting the timer at step 3B is not aligned with test procedure as TTCN is starting the timer before step 3A. Due to this in case of some UE this timer gets expired before MO Release Voice call (Annex A.7 of TS 34.229-5) procedure is completed leading to failing of the test case. |
| Summary of change | Changed TTCN Implementation to start step 3B timer after step 3A completion |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_1_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session // The test case currently sets up 1 IMS Emg Call in Normal service f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndRelease(); }

After Change

function f_TC_11_4_1_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session // The test case currently sets up 1 IMS Emg Call in Normal service f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndRelease_11_4_1(); }

## Change 4

| Function name | function f_IMS_EmergencyCallAndRelease_11_4_1 |
| --- | --- |
| Reason for change | Current TTCN implementation for starting the timer at step 3B is not aligned with test procedure as TTCN is starting the timer before step 3A. Due to this in case of some UE this timer gets expired before MO Release Voice call (Annex A.7 of TS 34.229-5) procedure is completed leading to failing of the test case. |
| Summary of change | Changed TTCN Implementation to start step 3B timer after step 3A completion |
| TTCN module | IMS_NR5GC_Functions |
| MCC160 Comment |  |

Before Change

function f_IMS_EmergencyCallAndRelease (IMS_IpcanPortType_Type p_IpcanType := IPCAN_E) runs on IMS_PTC { // Emergency registration: 34.229-5 Annex A.3 f_IMS_EmergencyRegistration(-, -, p_IpcanType); // @sic R5-212384 sic@ f_IMS_EmergencyCallAndReleaseWithoutRegistration(A_2_1_A7); // @sic R5s210518 sic@ // Release IMS as this will be (re)started in the next loop f_IMS_GlobalRelease(); // @sic R5-212383 sic@ } <<skipped code>>

After Change

function f_IMS_EmergencyCallAndRelease (IMS_IpcanPortType_Type p_IpcanType := IPCAN_E) runs on IMS_PTC { // Emergency registration: 34.229-5 Annex A.3 f_IMS_EmergencyRegistration(-, -, p_IpcanType); // @sic R5-212384 sic@ f_IMS_EmergencyCallAndReleaseWithoutRegistration(A_2_1_A7); // @sic R5s210518 sic@ // Release IMS as this will be (re)started in the next loop f_IMS_GlobalRelease(); // @sic R5-212383 sic@ } function f_IMS_EmergencyCallAndRelease_11_4_1 (IMS_IpcanPortType_Type p_IpcanType := IPCAN_E) runs on IMS_PTC { var INVITE_Request v_InviteRequest; timer t_Wait := 5.0; // Emergency registration: 34.229-5 Annex A.3 f_IMS_EmergencyRegistration(-, -, p_IpcanType); // @sic R5-212384 sic@ // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup_NR5GC_A_6(A_2_1_A7); // @sic R5s210875, R5-217857 sic@ // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Emergency Call Established")); // @sic R5-225362 sic@ f_IMS_IPCAN_WaitForTrigger(IPCAN, "Trigger MO Call Release"); // @sic R5-225362 sic@ //Release Emergency Call MO f_IMS_EmergencyCallReleaseMO(v_InviteRequest); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); // @sic R5-225362 sic@ t_Wait.start; alt { [] t_Wait.timeout { //timeout and if UE did not release send trigger to ss to release f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Step 3BA timeOut")); } } // Release IMS as this will be (re)started in the next loop f_IMS_GlobalRelease(); // @sic R5-212383 sic@ } <<skipped code>>