3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250340

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0984 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to P-Access-Network-Info header for Redcap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest, TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CP-240098 introduced “3GPP-NR-REDCAP” parameter for "access-type" and "access-class" in the P-Access-Network-Info header field to identify traffic to/from UEs accessing over NR RedCap. Currently this is not handled in default messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “3GPP-NR-REDCAP” in the P-Access-Network-Info header field parameters for Redcap UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant RedCap UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All IMS and 5G test cases that require SIP signalling |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-1 CR 1632 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to f1_PAccessNetworkInfo_AccessType	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  mohit.kanchan@keysight.com

# Corrections required

## Correction to f1_PAccessNetworkInfo_AccessType

| Template name | f1_PAccessNetworkInfo_AccessType |
| --- | --- |
| Reason for change | CP-240098 introduced “3GPP-NR-REDCAP” parameter for "access-type" and "access-class" in the P-Access-Network-Info header field to identify traffic to/from UEs accessing over NR RedCap. Currently this is not handled in default messages. |
| Summary of change | Added “3GPP-NR-REDCAP” in the P-Access-Network-Info header field parameters for Redcap UEs. |
| TTCN module | IMS_SIP_Templates.ttcn |

Before Change:

function fl_PAccessNetworkInfo_AccessType(template (omit) IPCAN_RAN_Type p_RAN_Type) return template (present) charstring { var template (present) charstring v_RanString := ?; var IPCAN_RAN_Type v_RAN_Type; if (ispresent(p_RAN_Type)) { v_RAN_Type := valueof(p_RAN_Type); select (v_RAN_Type) { case (UTRAN_FDD) { v_RanString := "3GPP-UTRAN-FDD"; } case (UTRAN_TDD) { v_RanString := "3GPP-UTRAN-TDD"; } case (GERAN) { v_RanString := "3GPP-GERAN"; } case (EUTRA_FDD) { v_RanString := "3GPP-E-UTRAN-FDD"; } // @sic R5s120530 change 1.4 sic@ case (EUTRA_TDD) { v_RanString := "3GPP-E-UTRAN-TDD"; } case (WLAN) { v_RanString := pattern "IEEE-802.11*"; } // @sic R5-165947: WLAN registration sic@ case (NR) { v_RanString := ("3GPP-NR", "3GPP-NR-FDD", "3GPP-NR-TDD"); } case else { FatalError(__FILE__, __LINE__, "Unknown RANTech"); } } } return v_RanString; }

After Change:

function fl_PAccessNetworkInfo_AccessType(template (omit) IPCAN_RAN_Type p_RAN_Type) return template (present) charstring { var template (present) charstring v_RanString := ?; var IPCAN_RAN_Type v_RAN_Type; if (ispresent(p_RAN_Type)) { v_RAN_Type := valueof(p_RAN_Type); select (v_RAN_Type) { case (UTRAN_FDD) { v_RanString := "3GPP-UTRAN-FDD"; } case (UTRAN_TDD) { v_RanString := "3GPP-UTRAN-TDD"; } case (GERAN) { v_RanString := "3GPP-GERAN"; } case (EUTRA_FDD) { v_RanString := "3GPP-E-UTRAN-FDD"; } // @sic R5s120530 change 1.4 sic@ case (EUTRA_TDD) { v_RanString := "3GPP-E-UTRAN-TDD"; } case (WLAN) { v_RanString := pattern "IEEE-802.11*"; } // @sic R5-165947: WLAN registration sic@ case (NR) { if (pc_supportOfRedCap_r17 or pc_supportOfERedCap_r18) { v_RanString := ("3GPP-NR", "3GPP-NR-FDD", "3GPP-NR-TDD", "3GPP-NR-REDCAP"); } else { v_RanString := ("3GPP-NR", "3GPP-NR-FDD", "3GPP-NR-TDD"); } } case else { FatalError(__FILE__, __LINE__, "Unknown RANTech"); } } } return v_RanString; }