3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250343

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0210 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per 3GPP TS 37.355 , the parameters maxNumberAP-SRS-PosResourcesPerBWP-r16 and maxNumberSP-SRS-PosResourcesPerBWP-r16 are optional. The current implementation of f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList() provides a FAIL if any of these parameters is not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the capabiliy check handling in f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList to fail only in case of pics mismatch, and allow the optional parameters to not be present |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4, 9.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList

| Function name | f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList() |
| --- | --- |
| Reason for change | As per 3GPP TS 37.355 , the parameters maxNumberAP-SRS-PosResourcesPerBWP-r16 and maxNumberSP-SRS-PosResourcesPerBWP-r16 are optional. The current implementation of f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList() provides a FAIL, if any of these parameters is not present.. |
| Summary of change | Changed the capabiliy check handling in f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList to fail only in case of pics mismatch, and allow the optional parameters to not be present |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList (NR_UL_SRS_Capability_r16.srs_PosResourceConfigCA_BandList_r16 p_NrUlSrsPosResourceConfigCABandList) runs on NR_BASE_PTC { //@sic POS Rel-16 R5-213638 sic@ var integer v_Index; var boolean v_Found_MaxNumberSRS_PosResourceSetsPerBWP := false; var boolean v_Found_MaxNumberSRS_PosResourcesPerBWP := false; var boolean v_Found_MaxNumberPeriodicSRS_PosResourcesPerBWP := false; var boolean v_Found_MaxNumberAP_SRS_PosResourcesPerBWP := false; var boolean v_Found_MaxNumberSP_SRS_PosResourcesPerBWP := false; for (v_Index := 0; v_Index < lengthof(p_NrUlSrsPosResourceConfigCABandList); v_Index:=v_Index+1) { fl_CheckNRBandCapability(p_NrUlSrsPosResourceConfigCABandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS if (pc_maxNumberSRS_PosResourceSetsPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSRS_PosResourceSetsPerBWP_r16) { v_Found_MaxNumberSRS_PosResourceSetsPerBWP := true; } if (pc_maxNumberSRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSRS_PosResourcesPerBWP_r16) { v_Found_MaxNumberSRS_PosResourcesPerBWP := true; } if (pc_maxNumberPeriodicSRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberPeriodicSRS_PosResourcesPerBWP_r16) { v_Found_MaxNumberPeriodicSRS_PosResourcesPerBWP := true; } if (ispresent(p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberAP_SRS_PosResourcesPerBWP_r16) and (pc_maxNumberAP_SRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberAP_SRS_PosResourcesPerBWP_r16)) { v_Found_MaxNumberAP_SRS_PosResourcesPerBWP := true; } if (ispresent(p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSP_SRS_PosResourcesPerBWP_r16) and (pc_maxNumberSP_SRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSP_SRS_PosResourcesPerBWP_r16)) { v_Found_MaxNumberSP_SRS_PosResourcesPerBWP := true; } } if (not (v_Found_MaxNumberSRS_PosResourceSetsPerBWP and v_Found_MaxNumberSRS_PosResourcesPerBWP and v_Found_MaxNumberPeriodicSRS_PosResourcesPerBWP and v_Found_MaxNumberAP_SRS_PosResourcesPerBWP and v_Found_MaxNumberSP_SRS_PosResourcesPerBWP)) { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for srs-PosResourceConfigCA-BandList-r16"); } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList (NR_UL_SRS_Capability_r16.srs_PosResourceConfigCA_BandList_r16 p_NrUlSrsPosResourceConfigCABandList) runs on NR_BASE_PTC { //@sic POS Rel-16 R5-213638 sic@ var integer v_Index; var boolean v_Found_MaxNumberSRS_PosResourceSetsPerBWP := false; var boolean v_Found_MaxNumberSRS_PosResourcesPerBWP := false; var boolean v_Found_MaxNumberPeriodicSRS_PosResourcesPerBWP := false; var boolean v_Found_MaxNumberAP_SRS_PosResourcesPerBWP := false; var boolean v_Found_MaxNumberSP_SRS_PosResourcesPerBWP := false; for (v_Index := 0; v_Index < lengthof(p_NrUlSrsPosResourceConfigCABandList); v_Index:=v_Index+1) { fl_CheckNRBandCapability(p_NrUlSrsPosResourceConfigCABandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS if (pc_maxNumberSRS_PosResourceSetsPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSRS_PosResourceSetsPerBWP_r16) { v_Found_MaxNumberSRS_PosResourceSetsPerBWP := true; } if (pc_maxNumberSRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSRS_PosResourcesPerBWP_r16) { v_Found_MaxNumberSRS_PosResourcesPerBWP := true; } if (pc_maxNumberPeriodicSRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberPeriodicSRS_PosResourcesPerBWP_r16) { v_Found_MaxNumberPeriodicSRS_PosResourcesPerBWP := true; } if (not ispresent(p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberAP_SRS_PosResourcesPerBWP_r16) or (pc_maxNumberAP_SRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberAP_SRS_PosResourcesPerBWP_r16)) { v_Found_MaxNumberAP_SRS_PosResourcesPerBWP := true; } if (not ispresent(p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSP_SRS_PosResourcesPerBWP_r16) or (pc_maxNumberSP_SRS_PosResourcesPerBWP == p_NrUlSrsPosResourceConfigCABandList[v_Index].maxNumberSP_SRS_PosResourcesPerBWP_r16)) { v_Found_MaxNumberSP_SRS_PosResourcesPerBWP := true; } } if (not (v_Found_MaxNumberSRS_PosResourceSetsPerBWP and v_Found_MaxNumberSRS_PosResourcesPerBWP and v_Found_MaxNumberPeriodicSRS_PosResourcesPerBWP and v_Found_MaxNumberAP_SRS_PosResourcesPerBWP and v_Found_MaxNumberSP_SRS_PosResourcesPerBWP)) { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for srs-PosResourceConfigCA-BandList-r16"); } }<<SKIPPED CODE>>