3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250498

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4950 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.1.6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | when configuration default grant allocation, the TA period is set to the default value 600. In TC 7.1.6.3 LongDRXCycle is 640. This will cause the UE to receive the TAC in the DRX mode. Therefore, remove default grant allocation configuration, continue to use the previous configuration, continue to stop sending TAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | remove default grant allocation configuration, continue to use the previous configuration, continue to stop sending TAC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-10_D25wk38 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_6_3_EUTRA |
| --- | --- |
| Reason for change | when configuration default grant allocation, the TA period is set to the default value 600. In TC 7.1.6.3 LongDRXCycle is 640. This will cause the UE to receive the TAC in the DRX mode. Therefore, remove default grant allocation configuration, continue to use the previous configuration, continue to stop sending TAC. |
| Summary of change | remove default grant allocation configuration, continue to use the previous configuration, continue to stop sending TAC. |
| TTCN module | \LTE\7_1\MAC_716.ttcn |
| MCC160 Comments |  |

Before Change:

| /* * @desc REFERENCE TS 36.523-1 clause 7.1.6.3 * @status APPROVED (LTE) */ function f_TC_7_1_6_3_EUTRA() runs on EUTRA_PTC { /* L2/MAC * 7.1.6.3 : DRX operation / Short cycle configured / Parameters configured by RRC */ const integer tsc_LongDRXCycle := 640; const integer tsc_ShortDRXCycle := 64; const integer tsc_DrxStartOffset := 4; const integer tsc_OnDurationTimer := 20; const integer tsc_ShortDRXCycleTimer := 4; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var SubFrameTiming_Type v_Timing2 := v_Timing; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; var octetstring v_EncodedRlcPdu; var integer v_DRXFrame; var integer v_FrameNumber; var integer v_FrameNumberCalculation; var integer v_NormalSF; timer t_Watchdog := 5.0; /////////////////////////////////////////////////////////////////////////////////// // PREAMBLE ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_Init(c1); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA); f_EUTRA_LoopBackActivation_State4_Def(eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1)); /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_SubFrame(4), cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small cs_UplinkTimeAlignment_Synch_Auto_Def, cs_UL_GrantConfig_ContPeriodic(5)); // periodic grant every 5ms (80% of 6ms the Inactivity Timer) //@siclog "Step 1 - 2" siclog@ //SS Transmits RRCConnectionReconfiguration to configure specific DRX parameters //The UE transmits RRCConnectionReconfigurationComplete f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig(eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_ShortDRX(tsc_DrxStartOffset, tsc_ShortDRXCycleTimer)); f_Delay(0.660); //@sic R5s160641 sic@ // suppress number of DL retransmissions: f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1); f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // to configure SS to report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Enable); //@sic R5s150062 sic@ v_Timing1 := f_GetNextSendOccasion_7_1_6_X_ShortDRX(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_ShortDRXCycle, tsc_DrxStartOffset, 0); // stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped: f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_DciInfo_CcchDcchDtchUL_Def); v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, tsc_ShortDRXCycle); v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF); //@siclog "Step 3" siclog@ // Choose the next subframe when the onDurationTimer is started. // The other things in Step 1 automatically taken care by SS v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_17B)))); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1,v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_Watchdog.stop; //@siclog "Step 5" siclog@ //@sic R5s150062 sic@ v_Timing1 := f_GetNextSendOccasion_7_1_6_X_ShortDRX(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_ShortDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1); // The other things in Step 3 automatically taken care by SS v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(1, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN1, crs_PDCP_SDU_17B)))); //@sic R5s110552 sic@ DRB.send(cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1,v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing2 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 6" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing2.SFN.Number, v_Timing2.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; //@siclog "Step 7" siclog@ //UE waits for drxShortCycleTimer expire. //do nothing //@siclog "Step 8" siclog@ v_DRXFrame := tsc_LongDRXCycle / 10; v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, 4 * tsc_ShortDRXCycle); v_FrameNumberCalculation := (((v_Timing1.SFN.Number / v_DRXFrame) + 1 ) * v_DRXFrame); //@sic R5s150062, R5w160204r1 sic@ v_FrameNumber := v_FrameNumberCalculation mod 1024; //@sic R5s150062 sic@ v_Timing1 := valueof(cs_SubFrameTiming(v_FrameNumber, tsc_DrxStartOffset, (v_Timing1.HSFN.Number + (v_FrameNumberCalculation/1024)) mod 1024)); v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); //@sic R5s180170 sic@ v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF); // The other things in Step 8 automatically taken care by SS // In the first PDCCH sub frame when the OnDurationTimer of Long DRX cycle is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(2, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B)))); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 9" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 8? SYSIND.receive ( car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 9"); t_Watchdog.stop; //@siclog "Step 10" siclog@ v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1); // The other things in Step 10 automatically taken care by SS v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(3, tsc_NoPoll, f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN3, crs_PDCP_SDU_17B)))); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 11" siclog@ // Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); t_Watchdog.stop; //test body ends f_EUTRA_TestBody_Set(false); /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// // to configure SS to stop report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Disable); // configure SS for default grant allocation f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); //leaving UE test state 4 v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, 0); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1, tsc_RRC_TI_Def, other, cs_TimingInfo_EUTRA_NB(v_Timing1)); // Switch off f_EUTRA_Postamble(eutra_Cell1, E1_IDLE); } |  |
| --- | --- |

After Change:

| /* * @desc REFERENCE TS 36.523-1 clause 7.1.6.3 * @status APPROVED (LTE) */ function f_TC_7_1_6_3_EUTRA() runs on EUTRA_PTC { /* L2/MAC * 7.1.6.3 : DRX operation / Short cycle configured / Parameters configured by RRC */ const integer tsc_LongDRXCycle := 640; const integer tsc_ShortDRXCycle := 64; const integer tsc_DrxStartOffset := 4; const integer tsc_OnDurationTimer := 20; const integer tsc_ShortDRXCycleTimer := 4; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var SubFrameTiming_Type v_Timing2 := v_Timing; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; var octetstring v_EncodedRlcPdu; var integer v_DRXFrame; var integer v_FrameNumber; var integer v_FrameNumberCalculation; var integer v_NormalSF; var Dl_Bandwidth_Type v_DL_Bandwidth; //@sic R5-253974 sic@ timer t_Watchdog := 5.0; /////////////////////////////////////////////////////////////////////////////////// // PREAMBLE ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_Init(c1); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA); f_EUTRA_LoopBackActivation_State4_Def(eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1)); /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_SubFrame(4), cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small cs_UplinkTimeAlignment_Synch_Auto_Def, cs_UL_GrantConfig_ContPeriodic(5)); // periodic grant every 5ms (80% of 6ms the Inactivity Timer) //@siclog "Step 1 - 2" siclog@ //SS Transmits RRCConnectionReconfiguration to configure specific DRX parameters //The UE transmits RRCConnectionReconfigurationComplete f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig(eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_ShortDRX(tsc_DrxStartOffset, tsc_ShortDRXCycleTimer)); f_Delay(0.660); //@sic R5s160641 sic@ // suppress number of DL retransmissions: f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1); f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // to configure SS to report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Enable); //@sic R5s150062 sic@ v_Timing1 := f_GetNextSendOccasion_7_1_6_X_ShortDRX(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_ShortDRXCycle, tsc_DrxStartOffset, 0); // stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped: f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_DciInfo_CcchDcchDtchUL_Def); v_DL_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1); v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, tsc_ShortDRXCycle, v_DL_Bandwidth); v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF, v_DL_Bandwidth); //@siclog "Step 3" siclog@ // Choose the next subframe when the onDurationTimer is started. // The other things in Step 1 automatically taken care by SS v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_17B)))); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1,v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_Watchdog.stop; //@siclog "Step 5" siclog@ //@sic R5s150062 sic@ v_Timing1 := f_GetNextSendOccasion_7_1_6_X_ShortDRX(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_ShortDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1); // The other things in Step 3 automatically taken care by SS v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(1, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN1, crs_PDCP_SDU_17B)))); //@sic R5s110552 sic@ DRB.send(cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1,v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing2 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 6" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing2.SFN.Number, v_Timing2.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; //@siclog "Step 7" siclog@ //UE waits for drxShortCycleTimer expire. //do nothing //@siclog "Step 8" siclog@ v_DRXFrame := tsc_LongDRXCycle / 10; v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, 4 * tsc_ShortDRXCycle, v_DL_Bandwidth); v_FrameNumberCalculation := (((v_Timing1.SFN.Number / v_DRXFrame) + 1 ) * v_DRXFrame); //@sic R5s150062, R5w160204r1 sic@ v_FrameNumber := v_FrameNumberCalculation mod 1024; //@sic R5s150062 sic@ v_Timing1 := valueof(cs_SubFrameTiming(v_FrameNumber, tsc_DrxStartOffset, (v_Timing1.HSFN.Number + (v_FrameNumberCalculation/1024)) mod 1024)); v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); //@sic R5s180170 sic@ v_Timing1 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF, v_DL_Bandwidth); // The other things in Step 8 automatically taken care by SS // In the first PDCCH sub frame when the OnDurationTimer of Long DRX cycle is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(2, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B)))); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 9" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 8? SYSIND.receive ( car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 9"); t_Watchdog.stop; //@siclog "Step 10" siclog@ v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1); // The other things in Step 10 automatically taken care by SS v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(3, tsc_NoPoll, f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN3, crs_PDCP_SDU_17B)))); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing1), cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)))); t_Watchdog.start; // Get the time when UE sent ACK v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode); //@siclog "Step 11" siclog@ // Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), ack)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); t_Watchdog.stop; //test body ends f_EUTRA_TestBody_Set(false); /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// // to configure SS to stop report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Disable); // configure SS for default grant allocation //COMMENTED OUT f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); //leaving UE test state 4 v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, 0); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1, tsc_RRC_TI_Def, other, cs_TimingInfo_EUTRA_NB(v_Timing1)); // Switch off f_EUTRA_Postamble(eutra_Cell1, E1_IDLE); } |  |
| --- | --- |