3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250519

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3788 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Emergency call shall allow maximum flexibility for the UE to find all available networks and RAN5 shall not enforce capability restrictions without core spec justification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Skip check of pc_supportOfRedCap_r17 during emergency call test cases. List of such test cases to be defined in Change 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case unfairly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.7.NR5GC, 11.4.2.NR5GC, 11.4.3.NR5GC, 11.4.5.NR5GC, 11.4.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_NR_CheckRedCapUEIndicator() |
| --- | --- |
| Reason for change | Emergency call shall allow maximum flexibility for the UE to find all available networks and RAN5 shall not enforce capability restrictions without core spec justification |
| Summary of change | Skip check of pc_supportOfRedCap_r17 during registration under emergency conditions. p_RegType to be passed from calling functions to implement this. |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function fl_NR_CheckRedCapUEIndicator(UE_NR_Capability p_NRCapability) { // @sic R5s240258 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var template (omit) integer v_UE_Release := omit; v_Received_NRCapabilityMsg := p_NRCapability; if (pc_supportOfRedCap_r17) { // Checks done in case support of RedCap PICS is set if (isvalue(v_Received_NRCapabilityMsg.accessStratumRelease)) { v_UE_Release := f_NR_ConvertAccessStratumReleaseToInteger(v_Received_NRCapabilityMsg.accessStratumRelease); // Check the UE release } if (valueof(v_UE_Release) < 17 ) //valueof is OK as it is checked in v_Received_NRCapabilityMsg.accessStratumRelease { f_SetVerdictInconc(__FILE__, __LINE__, "UE did not report rel-17 or higher with pc_supportOfRedCap_r17 enabled"); } else { if (not (ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCapParameters_r17.supportOfRedCap_r17))) { f_SetVerdictInconc(__FILE__, __LINE__, "pc_supportOfRedCap_r17 is set but UE did not signal capability for supportOfRedCap_r17"); } } } }

After Change:

function fl_NR_CheckRedCapUEIndicator(UE_NR_Capability p_NRCapability, TypeOfRegistration_Type p_RegType := Initial_Secure) { // @sic R5s240258 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var template (omit) integer v_UE_Release := omit; v_Received_NRCapabilityMsg := p_NRCapability; if (pc_supportOfRedCap_r17) { // Checks done in case support of RedCap PICS is set if (isvalue(v_Received_NRCapabilityMsg.accessStratumRelease)) { v_UE_Release := f_NR_ConvertAccessStratumReleaseToInteger(v_Received_NRCapabilityMsg.accessStratumRelease); // Check the UE release } if (valueof(v_UE_Release) < 17 ) //valueof is OK as it is checked in v_Received_NRCapabilityMsg.accessStratumRelease { f_SetVerdictInconc(__FILE__, __LINE__, "UE did not report rel-17 or higher with pc_supportOfRedCap_r17 enabled"); } else if (p_RegType != Emergency) // relax RedCap check for emergency call { if (not (ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCapParameters_r17.supportOfRedCap_r17))) { f_SetVerdictInconc(__FILE__, __LINE__, "pc_supportOfRedCap_r17 is set but UE did not signal capability for supportOfRedCap_r17"); } } } }

## Change 2

| Function name | function fl_NR_Capability() |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function f_NR_Capability(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var integer v_NRBandInd := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); var template (omit) FreqBandIndicatorNR v_MFBI_Band := f_NR_CellInfo_GetMFBI_Band(p_NR_CellId); var UE_NR_Capability v_NrCapability; var integer i; var boolean v_Found1stBand := false; var boolean v_Found2ndBand := false; var template (value) FreqBandInformation v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(p_NR_CellId)); // @sic R5s200933 sic@ var template (value) FreqBandInformation v_SecondaryBand; var template (value) FreqBandList v_FreqBandList := { v_FirstBand }; var octetstring v_EncodeCapRequestNR; var boolean v_CA_InterBand := false; var boolean v_NR_DC_Band := false; var template (omit) octetstring v_Enquiry_v1560 := omit; var octetstring v_EncodeCapRequestEUTRANR; var template (value) DL_DCCH_Message v_DL_DCCH_Message; var integer v_EUTRA_Band; var boolean v_NEDC_Band := false; var UE_MRDC_Capability v_NrCapability_MRDC; if (isvalue (v_MFBI_Band)) { // @sic R5s200826 R5s201305 R5s250353 sic@ v_NRBandInd := valueof(v_MFBI_Band); v_FreqBandList[0].bandInformationNR.bandNR := v_NRBandInd; } if (f_GetTestcaseAttrib_NR_CA(testcasename())) { //@sic R5-204397 sic@ if (f_NR_GetCA_CombinationType(px_NR_CA_BandCombination) == CA_InterBand) { //For CA interBand includes 2 entries as per 38.508-1 Table 4.6.4-21: FreqBandList v_CA_InterBand := true; // @sic R5-204397 sic@ v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetCA_SecondaryBand(px_NR_CA_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; } } if (f_GetTestcaseAttrib_NR_DC(testcasename())) { // @sic R5-215679 sic@ v_NR_DC_Band := true; v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetNR_DC_SecondaryBand (px_NR_DC_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; // Table 4.6.1-31: UECapabilityEnquiry with condition NR-DC - Include UE-CapabilityRequestFilterCommon with consition NR-DC v_Enquiry_v1560 := bit2oct(encvalue( cs_UECapabilityEnquiry_v1560_Def(cs_UE_CapabilityRequestFilterCommon))); //@sic R5s253066 sic@ } if (f_GetTestcaseAttrib_NEDC(testcasename())) { // @sic R5-221422 sic@ v_NEDC_Band := true; v_EUTRA_Band := f_NEDC_GetEUTRA_Band(px_NEDC_BandCombination); v_FreqBandList := { v_FirstBand }; v_EncodeCapRequestEUTRANR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR( {cs_NR_BandInformationEUTRA(v_EUTRA_Band), v_FirstBand}))); } v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR(v_FreqBandList) )); // @sic R5-213580 R5-221422 sic@ if (v_NEDC_Band) { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiryMultipleEntries (tsc_NR_RRC_TI_Def, {cs_UE_CapabilityRAT_Request( nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestEUTRANR)}, v_Enquiry_v1560); } else { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR, v_Enquiry_v1560); } SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, -, v_DL_DCCH_Message)); // @sic R5s200933 R5-210414 R5-221422 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))) -> value v_ReceivedAspForUeCapabilityInfo; // Decode and store the UE-CapabilityRAT-Container if (v_NEDC_Band) { //@sic R5-221422 sic@ if (v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].rat_Type == nr) { // First entry is nr, second enry is eutra_nr v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } else { // First entry is eutra_nr, second enry is nr v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } } else { v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); } f_NR_MobileInfo_NRCap_Set(v_NrCapability); // Check the RedCap fl_NR_CheckRedCapUEIndicator(v_NrCapability); // @sic R5s240258 sic@ // Check the UE release f_NR_CheckReleaseIndicator(v_NrCapability.accessStratumRelease); select (v_NrCapability.accessStratumRelease) { case (rel15, rel16, rel17, rel18) { // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 R5s240531 BASELINE MOVING 2024 rel-18 sic@ } case else{ FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered"); } } for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_NRBandInd)) { v_Found1stBand := true; break; } } if (v_CA_InterBand) { // @sic R5-204397 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NR_DC_Band) { // @sic R5-215679 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NEDC_Band) { //@sic R5-221422 sic@ // Check MRDC capabilities for (i := 0; i < lengthof(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter); i := i+1) { if (match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationEUTRA.bandEUTRA, v_EUTRA_Band ) and match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationNR.bandNR, v_NRBandInd)) { v_Found2ndBand := true; break; } } } if (not v_Found1stBand or ((v_NR_DC_Band or v_CA_InterBand or v_NEDC_Band) and not v_Found2ndBand)) { // @sic R5s190109, R5-204397, R5-215679 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NR Freq Band Ind must match value sent"); } }

After Change:

function f_NR_Capability(NR_CellId_Type p_NR_CellId, TypeOfRegistration_Type p_RegType := Initial_Secure) runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var integer v_NRBandInd := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); var template (omit) FreqBandIndicatorNR v_MFBI_Band := f_NR_CellInfo_GetMFBI_Band(p_NR_CellId); var UE_NR_Capability v_NrCapability; var integer i; var boolean v_Found1stBand := false; var boolean v_Found2ndBand := false; var template (value) FreqBandInformation v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(p_NR_CellId)); // @sic R5s200933 sic@ var template (value) FreqBandInformation v_SecondaryBand; var template (value) FreqBandList v_FreqBandList := { v_FirstBand }; var octetstring v_EncodeCapRequestNR; var boolean v_CA_InterBand := false; var boolean v_NR_DC_Band := false; var template (omit) octetstring v_Enquiry_v1560 := omit; var octetstring v_EncodeCapRequestEUTRANR; var template (value) DL_DCCH_Message v_DL_DCCH_Message; var integer v_EUTRA_Band; var boolean v_NEDC_Band := false; var UE_MRDC_Capability v_NrCapability_MRDC; if (isvalue (v_MFBI_Band)) { // @sic R5s200826 R5s201305 R5s250353 sic@ v_NRBandInd := valueof(v_MFBI_Band); v_FreqBandList[0].bandInformationNR.bandNR := v_NRBandInd; } if (f_GetTestcaseAttrib_NR_CA(testcasename())) { //@sic R5-204397 sic@ if (f_NR_GetCA_CombinationType(px_NR_CA_BandCombination) == CA_InterBand) { //For CA interBand includes 2 entries as per 38.508-1 Table 4.6.4-21: FreqBandList v_CA_InterBand := true; // @sic R5-204397 sic@ v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetCA_SecondaryBand(px_NR_CA_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; } } if (f_GetTestcaseAttrib_NR_DC(testcasename())) { // @sic R5-215679 sic@ v_NR_DC_Band := true; v_SecondaryBand := cs_NR_BandInformationNR(f_NR_GetNR_DC_SecondaryBand (px_NR_DC_BandCombination)); v_FreqBandList := { v_FirstBand, v_SecondaryBand }; // Table 4.6.1-31: UECapabilityEnquiry with condition NR-DC - Include UE-CapabilityRequestFilterCommon with consition NR-DC v_Enquiry_v1560 := bit2oct(encvalue( cs_UECapabilityEnquiry_v1560_Def(cs_UE_CapabilityRequestFilterCommon))); //@sic R5s253066 sic@ } if (f_GetTestcaseAttrib_NEDC(testcasename())) { // @sic R5-221422 sic@ v_NEDC_Band := true; v_EUTRA_Band := f_NEDC_GetEUTRA_Band(px_NEDC_BandCombination); v_FreqBandList := { v_FirstBand }; v_EncodeCapRequestEUTRANR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR( {cs_NR_BandInformationEUTRA(v_EUTRA_Band), v_FirstBand}))); } v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR(v_FreqBandList) )); // @sic R5-213580 R5-221422 sic@ if (v_NEDC_Band) { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiryMultipleEntries (tsc_NR_RRC_TI_Def, {cs_UE_CapabilityRAT_Request( nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestEUTRANR)}, v_Enquiry_v1560); } else { v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR, v_Enquiry_v1560); } SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, -, v_DL_DCCH_Message)); // @sic R5s200933 R5-210414 R5-221422 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))) -> value v_ReceivedAspForUeCapabilityInfo; // Decode and store the UE-CapabilityRAT-Container if (v_NEDC_Band) { //@sic R5-221422 sic@ if (v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].rat_Type == nr) { // First entry is nr, second enry is eutra_nr v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } else { // First entry is eutra_nr, second enry is nr v_NrCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); } } else { v_NrCapability := f_NR_DecodeNrCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); } f_NR_MobileInfo_NRCap_Set(v_NrCapability); // Check the RedCap fl_NR_CheckRedCapUEIndicator(v_NrCapability, p_RegType); // @sic R5s240258 sic@ // Check the UE release f_NR_CheckReleaseIndicator(v_NrCapability.accessStratumRelease); select (v_NrCapability.accessStratumRelease) { case (rel15, rel16, rel17, rel18) { // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 R5s240531 BASELINE MOVING 2024 rel-18 sic@ } case else{ FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered"); } } for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_NRBandInd)) { v_Found1stBand := true; break; } } if (v_CA_InterBand) { // @sic R5-204397 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NR_DC_Band) { // @sic R5-215679 sic@ for (i := 0; i < lengthof(v_NrCapability.rf_Parameters.supportedBandListNR); i := i+1) { // @sic R5-190783 sic@ if (match(v_NrCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_SecondaryBand.bandInformationNR.bandNR)) { v_Found2ndBand := true; break; } } } if (v_NEDC_Band) { //@sic R5-221422 sic@ // Check MRDC capabilities for (i := 0; i < lengthof(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter); i := i+1) { if (match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationEUTRA.bandEUTRA, v_EUTRA_Band ) and match(v_NrCapability_MRDC.rf_ParametersMRDC.appliedFreqBandListFilter[i].bandInformationNR.bandNR, v_NRBandInd)) { v_Found2ndBand := true; break; } } } if (not v_Found1stBand or ((v_NR_DC_Band or v_CA_InterBand or v_NEDC_Band) and not v_Found2ndBand)) { // @sic R5s190109, R5-204397, R5-215679 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NR Freq Band Ind must match value sent"); } }

## Change 3

| Function name | function f_NR5GC_RRC_Idle_Steps5_13() |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | NR5GC_NASSteps.ttcn |

Before Change:

function f_NR5GC_RRC_Idle_Steps5_13 (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, // @sic R5s211357 sic@ NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var NasCount_Type v_NasCount; // Steps 5 - 6 // Do Authentication v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_RegType, p_IwkN26, p_AssignmentMode); // @sic R5-202564, R5s230092 sic@ // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(p_GMM_MobilityInfo_Type); // @sic R5-195328, R5s190388 sic@ // Step 9a1 - 9a2 select (p_TestMode) { case (TESTModeA_ON, TEST_LOOPModeA_ON) { f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeA); } case (TESTModeB_ON, TEST_LOOPModeB_ON) { f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeB); } case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeB); } } case (TESTModeC_ON, TEST_LOOPModeC_ON) { //@sic R5-244716 sic@ f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeC); } case (TESTModeE_ON, TEST_LOOPModeE_ON) { // @sic R5-225291 sic@ f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeE); } case else { // do nothing } } //Steps 10 - 11 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, v_NasCount); // Use NasCount returned from NAS S.M. Complete // Steps 12 - 13 // UE Capability f_NR_Capability(p_NR_CellId); }

After Change:

function f_NR5GC_RRC_Idle_Steps5_13 (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, // @sic R5s211357 sic@ NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var NasCount_Type v_NasCount; // Steps 5 - 6 // Do Authentication v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_RegType, p_IwkN26, p_AssignmentMode); // @sic R5-202564, R5s230092 sic@ // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(p_GMM_MobilityInfo_Type); // @sic R5-195328, R5s190388 sic@ // Step 9a1 - 9a2 select (p_TestMode) { case (TESTModeA_ON, TEST_LOOPModeA_ON) { f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeA); } case (TESTModeB_ON, TEST_LOOPModeB_ON) { f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeB); } case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeB); } } case (TESTModeC_ON, TEST_LOOPModeC_ON) { //@sic R5-244716 sic@ f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeC); } case (TESTModeE_ON, TEST_LOOPModeE_ON) { // @sic R5-225291 sic@ f_NR5GC_Activate_TestMode (p_NR_CellId, tsc_UE_TestLoopMode_TypeE); } case else { // do nothing } } //Steps 10 - 11 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, v_NasCount); // Use NasCount returned from NAS S.M. Complete // Steps 12 - 13 // UE Capability f_NR_Capability(p_NR_CellId, p_RegType); }

## Change 4

| Function name | function f_NR5GC_EmergencyCallLimitedService() |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId, boolean p_IncludeTAIList := true) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var NR_SecurityParams_Type v_RRC_SecurityParams; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var template (omit) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); if (not p_IncludeTAIList) { // @sic R5s211510 sic@ v_TAIList := omit; } // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); } // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // Set all algorithms to 0 v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_RRC_SecurityParams := f_NR_Security_Get(); v_NAS_SecurityParams.NAS_Integrity.Algorithm := '0000'B; // IA0 v_NAS_SecurityParams.NAS_Ciphering.Algorithm := '0000'B; // EA0 v_NAS_SecurityParams.KSIamf := '000'B; f_NR5GC_Security_Set(v_NAS_SecurityParams); v_RRC_SecurityParams.AS_Integrity.Algorithm := nia0; v_RRC_SecurityParams.AS_Ciphering.Algorithm := nea0; f_NR_Security_Set(v_RRC_SecurityParams); //Steps 4-5 v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_NAS_SecurityParams, Initial_NoSecurity, v_GMM_MobilityInfo); // @sic R5s210518 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 5"); //Anritsu TTCN CR R5s250417 MCC if (isvalue (v_NAS_SecurityReturnValue.NasMsg)) { // As first time Registration, Registration Req should be replayed // overwrite 2nd message received with the replayed msg as this will be the complete message, so it can be checked again v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg := valueof(v_NAS_SecurityReturnValue.NasMsg); v_ReceivedMsg.Pdu.PiggybackedPduList[0].PiggybackedPduList := omit; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Not Replayed"); } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //Steps 6-7 f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 7"); //Steps 8-9 f_NR_Capability(p_NR_CellId); //Step 10 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList)))); // Step 11 f_NR5GC_Registration_Complete(p_NR_CellId); // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1, -, false); // @sic R5s210518, R5-223987 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // @sic R5-225362 sic@ // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); }

After Change:

function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId, boolean p_IncludeTAIList := true) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var NR_SecurityParams_Type v_RRC_SecurityParams; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var template (omit) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); if (not p_IncludeTAIList) { // @sic R5s211510 sic@ v_TAIList := omit; } // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); } // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // Set all algorithms to 0 v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_RRC_SecurityParams := f_NR_Security_Get(); v_NAS_SecurityParams.NAS_Integrity.Algorithm := '0000'B; // IA0 v_NAS_SecurityParams.NAS_Ciphering.Algorithm := '0000'B; // EA0 v_NAS_SecurityParams.KSIamf := '000'B; f_NR5GC_Security_Set(v_NAS_SecurityParams); v_RRC_SecurityParams.AS_Integrity.Algorithm := nia0; v_RRC_SecurityParams.AS_Ciphering.Algorithm := nea0; f_NR_Security_Set(v_RRC_SecurityParams); //Steps 4-5 v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_NAS_SecurityParams, Initial_NoSecurity, v_GMM_MobilityInfo); // @sic R5s210518 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 5"); //Anritsu TTCN CR R5s250417 MCC if (isvalue (v_NAS_SecurityReturnValue.NasMsg)) { // As first time Registration, Registration Req should be replayed // overwrite 2nd message received with the replayed msg as this will be the complete message, so it can be checked again v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg := valueof(v_NAS_SecurityReturnValue.NasMsg); v_ReceivedMsg.Pdu.PiggybackedPduList[0].PiggybackedPduList := omit; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Not Replayed"); } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //Steps 6-7 f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 7"); //Steps 8-9 f_NR_Capability(p_NR_CellId, Emergency); //Step 10 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList)))); // Step 11 f_NR5GC_Registration_Complete(p_NR_CellId); // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1, -, false); // @sic R5s210518, R5-223987 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // @sic R5-225362 sic@ // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); }