3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250052

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4881 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SSAC ACB per PLMN test case 13.5.1c |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 13.5.1c to the LTE_A_R12 test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 13.5.1c required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.1c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	7

2.5	Change 5	8

2.6	Change 6	10

3	Branches executed	11

4	Execution Log Files	11

4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 13.5.1c which is part of the LTE_A_R12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Mohit Kanchan

mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	13.5.1c

ATS Version:	iwd-TTCN3-B2024-06_D24wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset solution

UE used:	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm

Technologies’ 6th generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC |
| --- | --- |
| Reason for change | In existing TTCN implementation the timer tolerance is being set before the initialization of EUTRA cell which is causing test case execution error. |
| Summary of change | Moved the timer tolerance function after f_EUTRA_Init |
| TTCN module | LTE_A_R12\13\ AccessControl_R12.ttcn |
| MCC160 comment | Accepted |

Before Change:

function f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC { // MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; var float v_TimerNoRRCSetupReq := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, nonProtocolTimer, 10.0); f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); f_EUTRA_CellConfig_Def(eutra_Cell1); <<! SKIPPED CODE !>> }

After Change:

function f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC { // MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; var float v_TimerNoRRCSetupReq ; f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); v_TimerNoRRCSetupReq := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, nonProtocolTimer, 10.0); f_EUTRA_CellConfig_Def(eutra_Cell1); <<! SKIPPED CODE !>> }

## Change 2

| Function name | f_EUTRA_CellInfo_SetPLMN_2Entries |
| --- | --- |
| Reason for change | The current TTCN implementation of f_EUTRA_CellInfo_SetPLMN_Common function does not handling setting the IE “cellReservedForOperatorUse” to “reserved” which is required as in the test procedure (please refer to the associated prose CR. |
| Summary of change | Modified the input PLMN parameters and added appropriate flag to handle “reserved” value for the IE “cellReservedForOperatorUse” |
| TTCN module | Common\EUTRA\EUTRA_CellInfo.ttcn |
| MCC160 comment | Accepted by R5-250466 but see MCC160 Implementation |

Before Change:

function f_EUTRA_CellInfo_SetPLMN_2Entries(EUTRA_CellId_Type p_CellId, template (value) PLMN_Identity p_PLMN1, template (value) PLMN_Identity p_PLMN2, integer p_PlmnIndex := 0) runs on EUTRA_PTC { // @sic R5s110468: adding parameter p_PlmnIndex sic@ f_EUTRA_CellInfo_SetPLMN_Common(p_CellId, {p_PLMN1, p_PLMN2}, p_PlmnIndex); // @sic R5s110468 sic@ }

After Change:

function f_EUTRA_CellInfo_SetPLMN_2Entries(EUTRA_CellId_Type p_CellId, template (value) PLMN_Identity p_PLMN1, template (value) PLMN_Identity p_PLMN2, integer p_PlmnIndex := 0) runs on EUTRA_PTC { // @sic R5s110468: adding parameter p_PlmnIndex sic@ f_EUTRA_CellInfo_SetPLMN_Common(p_CellId, {p_PLMN1, p_PLMN2}, p_PlmnIndex,-,true,1); // @sic R5s110468 sic@ }

MCC160 Implementation

function f_EUTRA_CellInfo_SetPLMN_2Entries(EUTRA_CellId_Type p_CellId,

template (value) PLMN_Identity p_PLMN1,

template (value) PLMN_Identity p_PLMN2,

integer p_PlmnIndex := 0,

boolean p_CellReservedCheck := false,  //@sic R5s250052sic@

integer reserved_plmn_idx := 0) runs on EUTRA_PTC

{ // @sic R5s110468: adding parameter p_PlmnIndex sic@

f_EUTRA_CellInfo_SetPLMN_Common(p_CellId, {p_PLMN1, p_PLMN2}, p_PlmnIndex,-,p_CellReservedCheck,reserved_plmn_idx);  // @sic R5s110468 R5s250052 sic@

-----------------------------------------------------------

function f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC

{ // MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN

var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def;

var NAS_MSG_Indication_Type v_NasInd;

var float v_TimerNoRRCSetupReq;

f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS);

v_TimerNoRRCSetupReq := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, nonProtocolTimer, 10.0); // @sic R5s250052 sic@

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1,

cs_HPLMN_Def, //PLMNid 1 (index 0)

cs_HPLMN_002_11, //PLMNid 2 (index 1)

-, true, 1);  // R5s250052 sic@

f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, cs_AC_BarringConfig(p00, s128, '11111'B));

f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1);

//f_UT_USIM_Insert(UT, "Insert special USIM"); //USIM with PLMN2 is set in it as 'forbidden PLMN'REMOVED

f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE);

f_EUTRA_TestBody_Set(true);

}

## Change 3

| Function name | f_EUTRA_CellInfo_SetPLMN_Common |
| --- | --- |
| Reason for change | Please refer Change 2.2 |
| Summary of change | Please refer Change 2.2 |
| TTCN module | Common\EUTRA\EUTRA_CellInfo.ttcn |
| MCC160 comment | Accepted by R5-250466 |

Before Change:

function f_EUTRA_CellInfo_SetPLMN_Common(EUTRA_CellId_Type p_CellId, template (value) PLMN_IdentityList_Type p_PlmnList, integer p_PlmnIndex := 0, boolean p_UtranConfigured := false) //@sic R5s160174 sic@ runs on EUTRA_PTC { // @sic R5s110468: adding parameter p_PlmnIndex sic@ var EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); var template (value) EUTRA_CellInfo_Type v_NewCellInfo := v_CellInfo; var integer v_NoOfPlmns := lengthof(p_PlmnList); var MCC v_MCC_JapanBand6; var integer i; var Combination_Type v_SysinfoCombination := f_EUTRA_CellInfo_GetCombination(p_CellId); var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-167569 sic@ v_NewCellInfo.NAS_Parameters.Guti_Parameters.PLMN_Identity := p_PlmnList[p_PlmnIndex]; // @sic R5s110468 sic@ for (i:=0; i<v_NoOfPlmns; i:=i+1) { v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity := p_PlmnList[i]; v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].cellReservedForOperatorUse := notReserved; //@sic R5s100850 sic@ if (v_SysinfoType==sysinfoAndBR) { //UE supports category M1 or enhanced coverage TC v_NewCellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity := p_PlmnList[i]; v_NewCellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.cellAccessRelatedInfo.plmn_IdentityList[i].cellReservedForOperatorUse := notReserved; //@sic R5s100850 sic@ } } if (v_CellInfo.Frequency.FrequencyBandIndicator == 6 or (px_FDD_OperationBand==6 and ((v_SysinfoCombination == c4) or (v_SysinfoCombination == c9) or (v_SysinfoCombination == c10) or (v_SysinfoCombination == c10a) or (v_SysinfoCombination == c11 or p_UtranConfigured)))) { // For Band 6: set MCC to the Japan MCC value v_MCC_JapanBand6 := f_EUTRA_ConvertMCC_Nas2Asn(px_eJapanMCC_Band6); v_NewCellInfo.NAS_Parameters.Guti_Parameters.PLMN_Identity.mcc := v_MCC_JapanBand6; for (i:=0; i<v_NoOfPlmns; i:=i+1) { v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity.mcc := v_MCC_JapanBand6; if (v_SysinfoType==sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_NewCellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity.mcc := v_MCC_JapanBand6; } } } f_EUTRA_CellInfo_Set(p_CellId, v_NewCellInfo); }

After Change:

function f_EUTRA_CellInfo_SetPLMN_Common(EUTRA_CellId_Type p_CellId, template (value) PLMN_IdentityList_Type p_PlmnList, integer p_PlmnIndex := 0, boolean p_UtranConfigured := false, boolean p_CellReservedCheck := false, integer reserved_plmn_idx := 0) //@sic R5s160174 sic@ runs on EUTRA_PTC { // @sic R5s110468: adding parameter p_PlmnIndex sic@ var EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); var template (value) EUTRA_CellInfo_Type v_NewCellInfo := v_CellInfo; var integer v_NoOfPlmns := lengthof(p_PlmnList); var MCC v_MCC_JapanBand6; var integer i; var Combination_Type v_SysinfoCombination := f_EUTRA_CellInfo_GetCombination(p_CellId); var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-167569 sic@ v_NewCellInfo.NAS_Parameters.Guti_Parameters.PLMN_Identity := p_PlmnList[p_PlmnIndex]; // @sic R5s110468 sic@ for (i:=0; i<v_NoOfPlmns; i:=i+1) { v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity := p_PlmnList[i]; if (p_CellReservedCheck and i == reserved_plmn_idx) { v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].cellReservedForOperatorUse := reserved; //@sic R5s100850 sic@ } else { v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].cellReservedForOperatorUse := notReserved; //@sic R5s100850 sic@ } if (v_SysinfoType==sysinfoAndBR) { //UE supports category M1 or enhanced coverage TC v_NewCellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity := p_PlmnList[i]; v_NewCellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.cellAccessRelatedInfo.plmn_IdentityList[i].cellReservedForOperatorUse := notReserved; //@sic R5s100850 sic@ } } if (v_CellInfo.Frequency.FrequencyBandIndicator == 6 or (px_FDD_OperationBand==6 and ((v_SysinfoCombination == c4) or (v_SysinfoCombination == c9) or (v_SysinfoCombination == c10) or (v_SysinfoCombination == c10a) or (v_SysinfoCombination == c11 or p_UtranConfigured)))) { // For Band 6: set MCC to the Japan MCC value v_MCC_JapanBand6 := f_EUTRA_ConvertMCC_Nas2Asn(px_eJapanMCC_Band6); v_NewCellInfo.NAS_Parameters.Guti_Parameters.PLMN_Identity.mcc := v_MCC_JapanBand6; for (i:=0; i<v_NoOfPlmns; i:=i+1) { v_NewCellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity.mcc := v_MCC_JapanBand6; if (v_SysinfoType==sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_NewCellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.cellAccessRelatedInfo.plmn_IdentityList[i].plmn_Identity.mcc := v_MCC_JapanBand6; } } } f_EUTRA_CellInfo_Set(p_CellId, v_NewCellInfo); }

## Change 4

| Function name | f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC |
| --- | --- |
| Reason for change | The SIB2 in step 3 is not implemented in TTCN as per Table 13.5.1c.3.3-3 in the prose. |
| Summary of change | Modified the SIB2 information to align with prose. |
| TTCN module | LTE_A_R12\13\ AccessControl_R12.ttcn |
| MCC160 comment | Accepted |

Before Change:

function f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC { // MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; <!SKIPPED CODE!> // @siclog "Step 1" siclog@ //Make the UE attempt an MTSI MO Speech call f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @siclog "Step 2" siclog@ // Check: Does the UE transmit a RRCConnectionRequest message within 10 s? if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerNoRRCSetupReq)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, " Step 2"); } // @siclog "Step 3" siclog@ //SS adjusts SIB2 of Cell1 and transmits a Paging message. The systemInfoValueTag in the SystemInformationBlockType1 is increased. f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false); f_EUTRA_ModifySysinfo(eutra_Cell1); // @siclog "Step 4" siclog@ // Wait for 15 s (Note 1) to allow the new system information to take effect f_Delay(15.0); // @siclog "Step 5" siclog@ //Make the UE attempt an MTSI MO Speech call f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); <!SKIPPED CODE!> }

After Change:

function f_TC_13_5_1c_EUTRA() runs on EUTRA_PTC { // MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; <!SKIPPED CODE!> // @siclog "Step 1" siclog@ //Make the UE attempt an MTSI MO Speech call f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @siclog "Step 2" siclog@ // Check: Does the UE transmit a RRCConnectionRequest message within 10 s? if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell1, v_TimerNoRRCSetupReq)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, " Step 2"); } // @siclog "Step 3" siclog@ //SS adjusts SIB2 of Cell1 and transmits a Paging message. The systemInfoValueTag in the SystemInformationBlockType1 is increased. f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false,-,-,-,-,-,-,-,-,true); f_EUTRA_ModifySysinfo(eutra_Cell1); // @siclog "Step 4" siclog@ // Wait for 15 s (Note 1) to allow the new system information to take effect f_Delay(15.0); // @siclog "Step 5" siclog@ //Make the UE attempt an MTSI MO Speech call f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); <!SKIPPED CODE!> }

## Change 5

| Function name | f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 |
| --- | --- |
| Reason for change | Please refer Change 2.4 |
| Summary of change | Please refer Change 2.4 |
| TTCN module | Common\EUTRA\EUTRA_CellInfo.ttcn |
| MCC160 comment | Accepted |

Before Change:

function f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12(EUTRA_CellId_Type p_CellId, integer p_PlnmId, boolean p_AcBarring := false, template (omit) AC_BarringConfig p_BarringForMMTEL_Voice:= omit, template (omit) AC_BarringConfig p_BarringForMMTEL_Video:= omit, template (omit) AC_BarringConfig p_BarringForMO_Signalling:= omit, template (omit) AC_BarringConfig p_BarringForMO_Data:= omit, template (omit) AC_BarringConfig p_BarringForCSFB := omit, boolean p_SkipForVoice := false, boolean p_SkipForVideo := false, boolean p_SkipForSMS := false) runs on EUTRA_PTC { var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0] := cs_AC_BarringPerPLMN_r12 (p_PlnmId, p_AcBarring, p_BarringForMO_Signalling, p_BarringForMO_Data, p_BarringForMMTEL_Voice, p_BarringForMMTEL_Video, p_BarringForCSFB); if (p_SkipForVoice) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0].ac_BarringSkipForMMTELVoice_r12:= true_; } if (p_SkipForVideo) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0].ac_BarringSkipForMMTELVideo_r12:= true_; } if (p_SkipForSMS) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0].ac_BarringSkipForSMS_r12:= true_; } f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); f_EUTRA_SetSIs_ValueTag(p_CellId); }

After Change:

function f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12(EUTRA_CellId_Type p_CellId, integer p_PlnmId, boolean p_AcBarring := false, template (omit) AC_BarringConfig p_BarringForMMTEL_Voice:= omit, template (omit) AC_BarringConfig p_BarringForMMTEL_Video:= omit, template (omit) AC_BarringConfig p_BarringForMO_Signalling:= omit, template (omit) AC_BarringConfig p_BarringForMO_Data:= omit, template (omit) AC_BarringConfig p_BarringForCSFB := omit, boolean p_SkipForVoice := false, boolean p_SkipForVideo := false, boolean p_SkipForSMS := false, boolean p_SkipForBarring := false) runs on EUTRA_PTC { var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); if (not p_SkipForBarring) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0] := cs_AC_BarringPerPLMN_r12 (p_PlnmId, p_AcBarring, p_BarringForMO_Signalling, p_BarringForMO_Data, p_BarringForMMTEL_Voice, p_BarringForMMTEL_Video, p_BarringForCSFB); } else{ v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0] := cs_AC_BarringPerPLMN_r12_SkipForBarring (p_PlnmId); } if (p_SkipForVoice) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0].ac_BarringSkipForMMTELVoice_r12:= true_; } if (p_SkipForVideo) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0].ac_BarringSkipForMMTELVideo_r12:= true_; } if (p_SkipForSMS) { v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.ac_BarringPerPLMN_List_r12[0].ac_BarringSkipForSMS_r12:= true_; } f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); f_EUTRA_SetSIs_ValueTag(p_CellId); }

## Change 6

| Function name | cs_AC_BarringPerPLMN_r12_SkipForBarring |
| --- | --- |
| Reason for change | Please refer Change 2.4 |
| Summary of change | New template added to set the plmn-IdentityIndex-r12 as per Table 13.5.1c.3.3-3 |
| TTCN module | Common\EUTRA\EUTRA_SysInfo_Templates.ttcn |
| MCC160 comment | Accepted but see MCC160 Implementation |

template (value) AC_BarringPerPLMN_r12 cs_AC_BarringPerPLMN_r12_SkipForBarring(integer p_PlnmId ) := { plmn_IdentityIndex_r12 := p_PlnmId };

MCC160 Implementation

template (value) AC_BarringPerPLMN_r12 cs_AC_BarringPerPLMN_r12_SkipForBarring(integer p_PlnmId ) :=

{// @sic R5s250052 sic@

plmn_IdentityIndex_r12              := p_PlnmId,

ac_BarringInfo_r12  := omit,

ac_BarringSkipForMMTELVoice_r12     := omit,

ac_BarringSkipForMMTELVideo_r12     := omit,

ac_BarringSkipForSMS_r12            := omit,

ac_BarringForCSFB_r12               := omit,

ssac_BarringForMMTEL_Voice_r12      := omit,

ssac_BarringForMMTEL_Video_r12      := omit

};

# Branches executed

The test case was executed on on EUTRA band 1 using ciphering algorithm eea1 and integrity algorithm eia1

# Execution Log Files

## Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE passed this test case on Platform Keysight S8704A Protocol Conformance Toolset solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_13_5_1c_LOG.html

- PICS/PIXIT parameter file:

TC_13_5_1c_PIXIT.xml

# References

| [1] | R5s250053: Supporting information for addition of SSAC ACB per PLMN test case 13.5.1c |
| --- | --- |