3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250074

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3629 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At Table 11.4.11.3.2-2: Parallel behaviour Step 2Ba1, the routing table is set for QFI 2 for PDU Sessions the UE will establish when it attaches to 5GC. This needs to be reflected in the PDU Session Establishment Accept during the PDU connection for Emergency services, which currently signals QFI 1 incorrectly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Step 29a1 PDU Session Establishment for QFI 2 as per cl.4.9.11 in TS38.508-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase incorrectly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

3	References	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_11_4_11_NR5GC_TestBody() |
| --- | --- |
| Reason for change | At Table 11.4.11.3.2-2: Parallel behaviour Step 2Ba1, the routing table is set for QFI 2 for PDU Sessions the UE will establish when it attaches to 5GC. This needs to be reflected in the PDU Session Establishment Accept during the PDU connection for Emergency services, which currently signals QFI 1 incorrectly. |
| Summary of change | Update Step 29a1 PDU Session Establishment for QFI 2 as per cl.4.9.11 in TS38.508-1 |
| TTCN module | Emergency_IRAT_NR5GC_NR.ttcn |
| MCC160 Comments | Accepted in principle but always assigned QFI 2 for the Emergency DNN, in f_Get_NG_PDUSessionEstablishmentAccept |

Before Change:

function fl_TC_11_4_11_NR5GC_TestBody() runs on NR5GC_PTC { var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var template (present) NG_Request_Type v_RequestType := (cr_NG_Request_Type('010'B), cr_NG_Request_Type('100'B)); var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var O1_Type v_EmergencySessionId; var template (value) NG_PDU_SessionId v_SessionId; var integer i; //@siclog "Step 17" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @sic R5s220883 sic@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 18-28" siclog@ f_NR5GC_IRAT_MobilityRegistration (nr_Cell1, v_NAS_SecurityParams, omit, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), f_LastVisited_TAI(nr_Cell1), -, -, noRrcConnectionRelease, -, cds_NG_NetworkFeatureSupport_NoN26, -, cr_PDU_SessionStatusAny ifpresent, // @sic R5-231417 sic@ SUPPORTED); // @sic R5-237320 sic@ // Tell IMS PTCs the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // This function stores all received PDN Connectivity msgs, so just expect one more... //@siclog "Step 29a1" siclog@ f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, -, -, -, v_PduSessionInfoList); // @sic R5s210149, R5s220883, R5s221342 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(Emergency_PDN))) { // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); } } // ...then check one of received msgs is for the emergency bearer // @siclog "Step 1 Parallel Behaviour" siclog@ for (i := 1; i <= f_NR5GC_MsgInDefault_GetNumOfNASMessageReq(); i := i+1){ // @sic R5s220174 sic@ v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i); if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('100'B), omit, omit)) { // Emergency bearer v_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId; if (match (v_EmergencySessionId, v_SessionId.sessionId)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1"); } // transfer of existing bearer } else if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -,-, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('010'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Request Type neither existing nor emergency PDU session"); } } // @siclog "Step 30" siclog@ f_Delay(3.0); // tell IMS stop waiting for Re-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); // @siclog "Step 31-33" siclog@ f_NR5GC_PDUSessionRelease (nr_Cell1, v_EmergencySessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); // @siclog "Step 34" siclog@ f_NR_RRCRelease(nr_Cell1); }

After Change:

function fl_TC_11_4_11_NR5GC_TestBody() runs on NR5GC_PTC { var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var template (present) NG_Request_Type v_RequestType := (cr_NG_Request_Type('010'B), cr_NG_Request_Type('100'B)); var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var O1_Type v_EmergencySessionId; var template (value) NG_PDU_SessionId v_SessionId; var integer i; //@siclog "Step 17" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @sic R5s220883 sic@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 18-28" siclog@ f_NR5GC_IRAT_MobilityRegistration (nr_Cell1, v_NAS_SecurityParams, omit, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), f_LastVisited_TAI(nr_Cell1), -, -, noRrcConnectionRelease, -, cds_NG_NetworkFeatureSupport_NoN26, -, cr_PDU_SessionStatusAny ifpresent, // @sic R5-231417 sic@ SUPPORTED); // @sic R5-237320 sic@ // Tell IMS PTCs the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // This function stores all received PDN Connectivity msgs, so just expect one more... //@siclog "Step 29a1" siclog@ f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, -, -, -, -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, -, -, "2", v_PduSessionInfoList); // @sic R5s210149, R5s220883, R5s221342 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(Emergency_PDN))) { // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); } } // ...then check one of received msgs is for the emergency bearer // @siclog "Step 1 Parallel Behaviour" siclog@ for (i := 1; i <= f_NR5GC_MsgInDefault_GetNumOfNASMessageReq(); i := i+1){ // @sic R5s220174 sic@ v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i); if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('100'B), omit, omit)) { // Emergency bearer v_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId; if (match (v_EmergencySessionId, v_SessionId.sessionId)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1"); } // transfer of existing bearer } else if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -,-, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('010'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Request Type neither existing nor emergency PDU session"); } } // @siclog "Step 30" siclog@ f_Delay(3.0); // tell IMS stop waiting for Re-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Stop Waiting For Reregister")); // @siclog "Step 31-33" siclog@ f_NR5GC_PDUSessionRelease (nr_Cell1, v_EmergencySessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); // @siclog "Step 34" siclog@ f_NR_RRCRelease(nr_Cell1); }

#### MCC160 Implementation

function f_Get_NG_PDUSessionEstablishmentAccept(GSM_MobilityInfo_Type p_GSM_MobilityInfo,

GMM_MobilityInfo_Type p_GMM_MobilityInfo,

template (omit) DNN p_DNN,       // @sic R5-205939 sic@

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) S_NSSAI_Type p_S_NSSAI := omit, // @sic R5s190919, R5-211168 sic@

template (omit) AlwaysOnPDUSessionInd p_AlwaysOnPDUSessionInd := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

template (omit) charstring p_BuildQosFlow := omit,

boolean p_InterworkWithEPS := true,

template (omit) NGSM_NetworkFeatureSupport p_NGSM_NtwkFeatSupport := omit, // @sic R5s201387 Baseline Moving sic@

template (omit) ServingPLMNRateControl p_ServingPLMNRateCtrl := omit,

template (omit) ATSSSContainer p_ATSSSContainer := omit,

template (omit) ControlPlaneInd p_ControlPlaneInd := omit,

template (omit) IPHeaderCompressionConfig p_IPHeaderCompConfig := omit,

template (omit) EthernetHeaderCompressConfig p_EthernetHeaderCompConfig := omit,

template (omit) PDN_AddressInfo_Type p_ExtraPDN_Address := omit,

template (omit) ServiceLvlAAContainer p_ServiceLvlAA := omit,

template (omit) ReceivedMBSContainer p_ReceivedMBS := omit,

template (omit) N3QAI p_N3QAI := omit,

template (omit) ProtocolDesc p_ProtocolDesc := omit) return template (value) NG_NAS_DL_Pdu_Type

{

var template (value) NG_NAS_DL_Message_Type v_GSMMsg;

var template (value) PDU_SessionType v_PDU_SessionType;

var B3_Type v_PDN_TypeToBeUsed;

var ListOfPDN_AddressInfo_Type v_PDN_Addresses := {};

var template (omit) PDU_Address v_PDU_Address := omit;

var template (omit) AlwaysOnPDUSessionInd v_AlwaysOn := omit;

var template (value) QoS_Rules v_QoS_Rules := f_BuildDefaultQoSRules ("1"); // @sic R5-202585 sic@

var template (value) QoSFlowDescr v_QoSFlowDescr;

var template (omit) ExtdProtocolConfigOptions v_ExtdPCO;

var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit;

var template (value) MappedEPSContext v_MappedEPSContext;

var bitstring v_EPSParam; // If used

var O1_Type v_EPSBearerId := '00'O; // If used

var boolean v_InterworkWithEPS := false;

var template (value) QoS_Flow v_QoS_Flow;

var octetstring v_ParamOctets;

var template (value) S_NSSAI_Type v_S_NSSAI;

if (isvalue(p_PDU_SessionType)) {

v_PDU_SessionType := valueof(p_PDU_SessionType);

} else {

v_PDU_SessionType := cs_PDU_SessionType (p_GSM_MobilityInfo.SessionType);

}

if (isvalue(p_GMM_MobilityInfo.GMMCap) and p_InterworkWithEPS) { // @sic R5s190464 R5s191172, R5s201367 sic@

if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { // @sic R5-195328 sic@

v_InterworkWithEPS := true;

if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // @sic R5-204400 sic@

v_EPSBearerId := hex2oct(int2hex(valueof(p_GSM_MobilityInfo.EPS_Bearer), 1) & '0'H);

} else {

FatalError(__FILE__, __LINE__, "EPS Supported but no bearer id calculated");

}

if (p_GSM_MobilityInfo.PDUType == IMS_DNN) {    // @sic R5-205939 sic@

v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_2));

} else {

v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_1));

}

v_ParamOctets := bit2oct(v_EPSParam); // @sic R5s210302 sic@

// This is only correct if the mappedEPSContext contains EPSQoS

v_MappedEPSContext := cs_OneMappedEPSContext(v_EPSBearerId, lengthof(v_ParamOctets), cs_EPSParameter_QoS(v_ParamOctets)); // @sic R5s210302 sic@

v_MappedEPSBearerContexts := cs_OneMappedEPSBearerContextList (v_MappedEPSContext);

}

}

if (isvalue(p_QoS_Rules)) {

v_QoS_Rules := valueof(p_QoS_Rules);

} else {

select(p_GSM_MobilityInfo.PDUType) {  // @sic R5-221467 sic@

case (IMS_DNN, Emergency_PDN) {    // @sic R5-205939 sic@

v_QoS_Rules := f_BuildDefaultQoSRules ("2"); // @sic R5-202585 sic@

}

case (URLLC_DNN) { // @sic R5-221467 sic@

v_QoS_Rules := f_BuildDefaultQoSRules ("8");

}

// case MBS_DNN - to be implemented

}

} // else use QoS 1 already defined above

if (isvalue(p_QoSFlowDescr)) { // @sic R5-194896 sic@

v_QoSFlowDescr := valueof(p_QoSFlowDescr);

} else {

if (isvalue(p_BuildQosFlow)) { // @sic R5-202550 sic@

// need to build a specific QoS flow

v_QoS_Flow := f_BuildDefaultQosFlow(valueof(p_BuildQosFlow), v_InterworkWithEPS, v_EPSBearerId);

} else {

select(p_GSM_MobilityInfo.PDUType) {  // @sic R5-221467 sic@

case (IMS_DNN) {    // @sic R5-205939 sic@

v_QoS_Flow := cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearerId); // @sic R5-204400, R5s201323 sic@

}

case (URLLC_DNN) { // @sic R5-221467 sic@

v_QoS_Flow := f_BuildDefaultQosFlow ("7");

} case else {

v_QoS_Flow := cs_QoS_Flow1(v_InterworkWithEPS, v_EPSBearerId); // @sic R5-204400, R5s201323 sic@

}

}

}

v_QoSFlowDescr := cs_QoSFlowDescr ({v_QoS_Flow}); // @sic R5s220471 sic@

}

if (isvalue(p_S_NSSAI)) { // @sic R5-211168 sic@

v_S_NSSAI := p_S_NSSAI; // compiler warning may be ignored due to check

} else {

// 38.508-1 table 4.8.4-1 specifies SST_eMMB for both Config#1 and #2, but there is currently no other option

// This to be updated when another Config is specified. FFS

select (p_GSM_MobilityInfo.PDUType) {  // @sic R5s221179 sic@

case (URLLC_DNN, Ethernet_DNN) {      // @sic R5-227473 sic@

v_S_NSSAI := cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI_SST2_URLLC);

}

case (MIOT_DNN) {

v_S_NSSAI := cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI_SST3_MIoT);

}

case (V2X_DNN) {

v_S_NSSAI := cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI_SST4_V2X);

}

case else {

v_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI;

}

}

}

if (isvalue(p_AlwaysOnPDUSessionInd)) {

v_AlwaysOn := p_AlwaysOnPDUSessionInd;

} else if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) {

v_AlwaysOn := cs_AlwaysOnNotAllowed;

if (match(p_GSM_MobilityInfo.PDUType, (URLLC_DNN, Ethernet_DNN))) { // @sic R5s221179, R5-227473 sic@

v_AlwaysOn := cs_AlwaysOnAllowed;

}

}

v_PDN_TypeToBeUsed := valueof(v_PDU_SessionType.typeValue);

select (v_PDN_TypeToBeUsed){

case (tsc_PdnType_IPv4, tsc_PdnType_IPv6, tsc_PdnType_IPv4v6) {

v_PDN_Addresses[0] := f_PDN_AddressInfo_Get (f_NR5GC_GetPdnIndex(p_GSM_MobilityInfo)); // @sic R5s200643 sic@

v_PDU_Address := f_NG_GetPDNAddress(f_CheckExtdPCOforIPallocationViaNas(p_GSM_MobilityInfo.PCO), v_PDN_TypeToBeUsed, v_PDN_Addresses[0]); //  @sic R5s200571, R5s200643 sic@

}

}

if (isvalue(p_ExtdPCO)) {

v_ExtdPCO := p_ExtdPCO;

} else {

if (isvalue(p_ExtraPDN_Address)) { // @sic R5-213400, R5-217830 sic@

v_PDN_Addresses[1] := valueof(p_ExtraPDN_Address);

}

v_ExtdPCO := f_NG_GetDefaultExtdProtocolConfigOptions (p_GSM_MobilityInfo.PCO, v_PDN_Addresses); // @sic R5s200643, R5-213400 sic@

}

...

# References