3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250097

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3641 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation after step 24 when the SS releases the rrc connection, the UL grant is stopped and not restarted. This results in UE not getting any UL grant in case UE follows optional Steps 26a1-26a13. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called the function f_NR_ULGrantConfiguration_Start after Step Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Change 1

| Template name | fl_TC_6_3_1_7_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation after step 24 when the SS releases the rrc connection, the UL grant is stopped and not restarted. This results in UE not getting any UL grant in case UE follows optional Steps 26a1-26a13. This needs to be addressed. |
| Summary of change | Called the function f_NR_ULGrantConfiguration_Start after Step Please see below. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def))); //@sic R5s240711 sic@ f_NR_RRCRelease_Local(nr_Cell11, f_NR_GetNextSendOccasion_DL(nr_Cell11)); //@sic R5s240711 sic@ deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ //@sic R5-247540 sic@ //@siclog "Step 25" siclog@ //The SS starts a timer T1=5s t_WaitTimer.start; alt { //@siclog "Step 26a1" siclog@ //The UE transmit a RRCSetupRequest message [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp { //@siclog "Step 26a2" siclog@ //The SS stops timer T1 t_WaitTimer.stop; //@siclog "Step 26a3-26a13" siclog@ //Steps 3 to 13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NR Cell 11. f_NR_RRCSetup_Def(nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell11, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)), TESTMode_OFF, Initial_NoSecurity); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def))); //@sic R5s240711 sic@ f_NR_RRCRelease_Local(nr_Cell11, f_NR_GetNextSendOccasion_DL(nr_Cell11)); //@sic R5s240711 sic@ deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ //@sic R5-247540 sic@ //@siclog "Step 25" siclog@ //The SS starts a timer T1=5s t_WaitTimer.start; alt { //@siclog "Step 26a1" siclog@ //The UE transmit a RRCSetupRequest message [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp { //@siclog "Step 26a2" siclog@ //The SS stops timer T1 t_WaitTimer.stop; f_NR_ULGrantConfiguration_Start(nr_Cell11); //WA#WI1396644 //@siclog "Step 26a3-26a13" siclog@ //Steps 3 to 13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NR Cell 11. f_NR_RRCSetup_Def(nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell11, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)), TESTMode_OFF, Initial_NoSecurity); <<SKIPPED CODE>>