3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250121

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4893 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE SSAC ACB per PLMN test case 13.5.4a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 13.5.4a to the LTE_A_R12 test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 13.5.4a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1 Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE	5

4.2	UniSoc N6P UOB	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 13.5.4a which is part of the LTE_A_R12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

## 1.1 Verification Test Summary

Test Case: 	13.5.4a

Test Group:                            /LTE_A_R12/13/AccessControl_R12.ttcn

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Rohde & Schwarz CMW500

UE used:	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm

Technologies’ 6th generation modem-to-antenna 5G solution UE,

Unisoc N6P UOB UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_13_5_4a_EUTRA() runs on EUTRA_PTC |
| --- | --- |
| Reason for change | - In current TTCN implementation in step1 the SIB2 is not implemenated as per Table 13.5.4a.3.3-2 in the Prose. - In existing TTCN IMS MO Speech call establishment in EUTRA is not implemented as per TS 36.508, Table 4.5A.6.3-. It contains unnecessary UE Cap exchange procedure. |
| Summary of change | - Modified the SIB2 information to align with prose. - Removed in the UE Capability exchange procedure at step 19. |
| TTCN module | LTE_A_R12\13\ AccessControl_R12.ttcn |
| MCC160 comment | Accepted |

Before Change:

function f_TC_13_5_4a_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SCM / 0% access probability skip for MTSI MO speech call / AC-Barring per PLMN var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; <<! SKIPPED CODE !>> // @siclog "Step 1" siclog@ f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, cs_AC_BarringConfig(p00, s512, '11111'B), -, -, -, -, true_); f_EUTRA_ModifySysinfo (eutra_Cell1); <<! SKIPPED CODE !>> // @siclog "Step 19" siclog@ f_EUTRA_508Check_ConnectedState (eutra_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change:

function f_TC_13_5_4a_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SCM / 0% access probability skip for MTSI MO speech call / AC-Barring per PLMN var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; <<! SKIPPED CODE !>> // @siclog "Step 1" siclog@ f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, -, -, -, cs_AC_BarringConfig(p00, s512, '11111'B), -, true_); f_EUTRA_ModifySysinfo (eutra_Cell1); <<! SKIPPED CODE !>> // @siclog "Step 19" siclog@ //f_EUTRA_508Check_ConnectedState (eutra_Cell1); WA#WI1396735 f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

# Branches executed

The test case was executed on on EUTRA band 1 using ciphering algorithm eea1 and integrity algorithm eia1 on Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE.

The test case was executed on on EUTRA band 41 using ciphering algorithm eea1 and integrity algorithm eia1 on Unisoc N6P UOB UE.

# Execution Log Files

## Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_13_5_4a_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

## UniSoc N6P UOB

The UniSoc N6P UOB UE passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_13_5_4a_Unisoc.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s250122: Supporting information for addition of LTE SSAC ACB per PLMN test case 13.5.4a |
| --- | --- |