3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250153

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0966 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_IMS_ECallOnly_TestCall function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Contents for Notify header in function f_IMS_ECallOnly_TestCall are defined and assigned to v_MessageHeader_Notify before SUBSCRIBE has been received and processed. Some contents of NOTIFY are derived from SUBSCRIBE message. This will provoke an error when calling function f_IMS_NotifyRequest_MessageHeader_A_1_6. This issue can be resolved asigning contents to v_MessageHeader_Notify after SUBSCRIBE message has been received. In addition to issue above, a coordination message to NR/LTE PTC must be sent to let know that the Test call has been released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Assigned contents to v_MessageHeader_Notify after SUBSCRIBE has been received. - Added coordination message to let know NR/LTE PTC that IMS call has been released. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2, 21.26 and 11.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Online, , 13th Dec 2024 - 31st Dec 2025	1

1	Overview	2

2	Corrections required:	2

2.1	Correction to function f_IMS_ECallOnly_TestCall	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required:

  - Correction to function f_IMS_ECallOnly_TestCall

| Function name | f_IMS_ECallOnly_TestCall |
| --- | --- |
| Reason for change | Contents are for Notify header are defined and assigned to v_MessageHeader_Notify before SUBSCRIBE has been received and processed. Some contents of NOTIFY are derived from SUBSCRIBE message. This will provoke an issue while calling function f_IMS_NotifyRequest_MessageHeader_A_1_6. In addition to issue above a coordination message to NR/LTE PTC must be sent to let know that the Test call has been released. |
| Summary of change | 1. Assigned contents to v_MessageHeader_Notify after SUBSCRIBE has been received. 2. Added coordination message to let know NR/LTE PTC that IMS call has been released. |
| TTCN module | /Common/IMS/IMS_eCall_Functions.ttcn |
| MCC160 Comment | 1. Accepted in principle: There is no need ro use ‘IMS_Client.check’ to receive the 200 OK for NOTIFY (see implementation shown below). 2. Alternative solution implemented – add the coordination message to the test body of 11_3_2, after calling f_IMS_ECallOnly_TestCall |

Before Change:

function f_IMS_ECallOnly_TestCall(ECall_MSD_Update_Type p_MSDUpdate := msdUpdateRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var SipUrl v_ContactUrl; var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var template (value) MessageHeader v_MessageHeader_Notify; var boolean v_WaitForSubscribe := true; var boolean v_WaitForInvite := true; var boolean v_WaitFor200OK := false; var default v_PublishHandler; v_IMS_REGISTER_REQ := f_IMS_Register(); // REGISTER, 401 Unauthorized, REGISTER, 200 OK for REGISTER v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active);//WA#WI=1401307 v_PublishHandler := activate(a_IMS_PublishHandling()); while (v_WaitForSubscribe or v_WaitForInvite or v_WaitFor200OK) { alt { [v_WaitForSubscribe] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // SUBSCRIBE, 200 OK for SUBSCRIBE IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), // NOTIFY cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_WaitForSubscribe := false; v_WaitFor200OK := true; } [v_WaitForInvite] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); // INVITE f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_WaitForInvite := false; } [v_WaitFor200OK] a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol) { // 200 OK for NOTIFY v_WaitFor200OK := false; } } } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // => ACTIVATE DEDICATED EPS BEARER f_IMS_IPCAN_WaitForTrigger(IPCAN); // ... bearer activated f_ECall_CallSetup_Steps2_3(v_InviteRequestWithSdp, { AMR_16000, AMR_8000 }); /* NOTE: LTE test case 11.3.2 and NR5GC test case 11.5.2 specify the steps 2-3 to happen after establishment of the dedicated bearers * (or session modification in NR5GC) => coordination messages are needed. * IMS test case 21.26 does not specify any EUTRA signalling, so that the use of coordination messages is unspecified */ if (p_MSDUpdate == msdUpdateRequired) { f_ECall_MSDUpdate_AnnexC47(TEST); } v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); deactivate(v_PublishHandler); return v_InviteRequestWithSdp; }

After Change:

function f_IMS_ECallOnly_TestCall(ECall_MSD_Update_Type p_MSDUpdate := msdUpdateRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var SipUrl v_ContactUrl; var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var template (value) MessageHeader v_MessageHeader_Notify; var template (value) MessageHeader v_MessageHeader_Response; var boolean v_WaitForSubscribe := true; var boolean v_WaitForInvite := true; var boolean v_WaitFor200OK := false; var default v_PublishHandler; v_IMS_REGISTER_REQ := f_IMS_Register(); // REGISTER, 401 Unauthorized, REGISTER, 200 OK for REGISTER v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); v_PublishHandler := activate(a_IMS_PublishHandling()); while (v_WaitForSubscribe or v_WaitForInvite or v_WaitFor200OK) { alt { [v_WaitForSubscribe] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // SUBSCRIBE, 200 OK for SUBSCRIBE v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Notify); IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), // NOTIFY cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_WaitForSubscribe := false; v_WaitFor200OK := true; } [v_WaitForInvite] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); // INVITE f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_WaitForInvite := false; } [v_WaitFor200OK] IMS_Client.check(receive(car_IMS_DATA_RSP(cr_IMS_RoutingInfo(v_Protocol), cr_Response(c_statusLine200,?)))) { // 200 OK for NOTIFY f_IMS_ReceiveResponse(v_MessageHeader_Notify, v_Protocol, cr_Response(c_statusLine200, v_MessageHeader_Response)); v_WaitFor200OK := false; } } } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // => ACTIVATE DEDICATED EPS BEARER f_IMS_IPCAN_WaitForTrigger(IPCAN); // ... bearer activated f_ECall_CallSetup_Steps2_3(v_InviteRequestWithSdp, { AMR_16000, AMR_8000 }); /* NOTE: LTE test case 11.3.2 and NR5GC test case 11.5.2 specify the steps 2-3 to happen after establishment of the dedicated bearers * (or session modification in NR5GC) => coordination messages are needed. * IMS test case 21.26 does not specify any EUTRA signalling, so that the use of coordination messages is unspecified */ if (p_MSDUpdate == msdUpdateRequired) { f_ECall_MSDUpdate_AnnexC47(TEST); } v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); deactivate(v_PublishHandler); return v_InviteRequestWithSdp; }

MCC160 Implementation of f_IMS_ECallOnly_TestCall

function f_IMS_ECallOnly_TestCall(ECall_MSD_Update_Type p_MSDUpdate := msdUpdateRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var SipUrl v_ContactUrl; var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var template (value) MessageHeader v_MessageHeader_Notify; var boolean v_WaitForSubscribe := true; var boolean v_WaitForInvite := true; var boolean v_WaitFor200OK := false; var default v_PublishHandler; v_IMS_REGISTER_REQ := f_IMS_Register(); // REGISTER, 401 Unauthorized, REGISTER, 200 OK for REGISTER v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); v_PublishHandler := activate(a_IMS_PublishHandling()); while (v_WaitForSubscribe or v_WaitForInvite or v_WaitFor200OK) { alt { [v_WaitForSubscribe] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // SUBSCRIBE, 200 OK for SUBSCRIBE v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), // NOTIFY cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_WaitForSubscribe := false; v_WaitFor200OK := true; } [v_WaitForInvite] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); // INVITE f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_WaitForInvite := false; } [v_WaitFor200OK] a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol) { // 200 OK for NOTIFY v_WaitFor200OK := false; } } } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // => ACTIVATE DEDICATED EPS BEARER f_IMS_IPCAN_WaitForTrigger(IPCAN); // ... bearer activated f_ECall_CallSetup_Steps2_3(v_InviteRequestWithSdp, { AMR_16000, AMR_8000 }); /* NOTE: LTE test case 11.3.2 and NR5GC test case 11.5.2 specify the steps 2-3 to happen after establishment of the dedicated bearers * (or session modification in NR5GC) => coordination messages are needed. * IMS test case 21.26 does not specify any EUTRA signalling, so that the use of coordination messages is unspecified */ if (p_MSDUpdate == msdUpdateRequired) { f_ECall_MSDUpdate_AnnexC47(TEST); } v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); deactivate(v_PublishHandler); return v_InviteRequestWithSdp; }