3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250249

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0975 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_IPCAN_EmergencyCall |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-06-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A rework has been done in function f_EUTRA_IPCAN_EmergencyCall to handle different flavours of eCall test cases. Function will not work as expected as usage of input parameter p_ConnectionEstablished is wrong, it will skip RRC test steps previous to a PDN request when it should be the opposite; establish an RRC connection in case it is not established already. This can be solved setting a not p_ConnectionEstablished as condition to establish an RRC connection before estbalishing emergency PDN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a not to p_ConnectionEstablished condition in f_EUTRA_IPCAN_EmergencyCall function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | LTE eCall and emergency test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to f_EUTRA_IPCAN_EmergencyCall	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to f_EUTRA_IPCAN_EmergencyCall

| Function name | f_EUTRA_IPCAN_EmergencyCall |
| --- | --- |
| Reason for change | Function will not work as expected as usage of input parameter p_ConnectionEstablished is wrong, it will skip RRC test steps previous to a PDN request when it should be the opposite; establish an RRC connection in case it is not established already. |
| Summary of change | Added a not to p_ConnectionEstablished condition in f_EUTRA_IPCAN_EmergencyCall function. |
| TTCN module | IPCAN_EUTRA |
| MCC160 Comment | Accepted |

Before Change:

function f_EUTRA_IPCAN_EmergencyCall(EUTRA_CellId_Type p_CellId, EPS_BearerIdentity p_DefaultBearerId, boolean p_ConnectionEstablished, boolean p_VoiceBearerToBeEstablished) runs on EUTRA_PTC { /* @sic R5s140490 change 7 MCC160 implementation: p_ConnectionEstablished sic@ */ /* @sic R5-253122, R5-253123: p_VoiceBearerToBeEstablished to be set to false for IPCAN_EmergencyCall_Aborted sic@ */ var boolean v_StartIPfor2ndPDN := false; /* @sic R5s150412 sic@ */ var EPS_BearerIdentity v_DedicatedBearerId; var NAS_MSG_Indication_Type v_NasInd; // Table 4.5A.4.3-1 step 2 - 8 if (p_ConnectionEstablished) { f_EUTRA_508_EstablishIMSEmergencyCall_Steps2_8(p_CellId); } // Table 4.5A.4.3-1 step 9 v_NasInd := f_EUTRA_508_EstablishIMSEmergencyCall_Step9(p_CellId); // Table 4.5A.4.3-1 step 10 - 12 f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_12(p_CellId, p_DefaultBearerId, v_NasInd, v_StartIPfor2ndPDN); // Table 4.5A.4.3-1 step 13 - 15 if (p_VoiceBearerToBeEstablished) { v_DedicatedBearerId := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId); f_EUTRA_508_EstablishIMSEmergencyCall_Steps13_15(p_CellId, p_DefaultBearerId, v_DedicatedBearerId); } }

After Change:

function f_EUTRA_IPCAN_EmergencyCall(EUTRA_CellId_Type p_CellId, EPS_BearerIdentity p_DefaultBearerId, boolean p_ConnectionEstablished, boolean p_VoiceBearerToBeEstablished) runs on EUTRA_PTC { /* @sic R5s140490 change 7 MCC160 implementation: p_ConnectionEstablished sic@ */ /* @sic R5-253122, R5-253123: p_VoiceBearerToBeEstablished to be set to false for IPCAN_EmergencyCall_Aborted sic@ */ var boolean v_StartIPfor2ndPDN := false; /* @sic R5s150412 sic@ */ var EPS_BearerIdentity v_DedicatedBearerId; var NAS_MSG_Indication_Type v_NasInd; // Table 4.5A.4.3-1 step 2 - 8 if (not p_ConnectionEstablished) { f_EUTRA_508_EstablishIMSEmergencyCall_Steps2_8(p_CellId); } // Table 4.5A.4.3-1 step 9 v_NasInd := f_EUTRA_508_EstablishIMSEmergencyCall_Step9(p_CellId); // Table 4.5A.4.3-1 step 10 - 12 f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_12(p_CellId, p_DefaultBearerId, v_NasInd, v_StartIPfor2ndPDN); // Table 4.5A.4.3-1 step 13 - 15 if (p_VoiceBearerToBeEstablished) { v_DedicatedBearerId := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId); f_EUTRA_508_EstablishIMSEmergencyCall_Steps13_15(p_CellId, p_DefaultBearerId, v_DedicatedBearerId); } }