3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250288

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3719 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcases 9.1.8.1 and 9.1.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation for testcase 9.1.8.1, when the TC is run in SNPN mode, it is observed that there are no SNPN related MMI prompts. This needs to be implemented. In the current TTCN implementation of testcase 9.1.8.2, first the function f_NG_SMS_AT_Config_WithPowering_EnterState_0B (v_NGC_NASCellA); is called, which sets the cell to serving cell power. Because of this, when the function f_NR5GC_Preamble_SMSoverNASf_NR5GC_Preamble_Steps1_4fl_NR5GC_PreconfigUEf_UT_SwitchOnUE is called, it camps on the cell, while the user is still sending the AT commands for SNPN.This causes TC failure, and needs to be addressed |  |  |  |  |  |  |  |  |
| Summary of change: |  | For testcase 9.1.8.1, called the function fl_NR5GC_PreconfigUE(omit); after cell creation. This will ensure that the SNPN configuration commands are called. For testacse 9.1.8.2, between the calls to functions f_NG_SMS_AT_Config_WithPowering_EnterState_0B and f_NR5GC_Preamble_SMSoverNAS, called f_NR_SetCellPower(v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); This will ensure that when the user is configuring the UE with SNPN commnds, the UE does not find any suitable cell to camp on. Please see detailes below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.8.1.NR5GC, 9.1.8.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_9_1_8_1_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation for testcase 9.1.8.1, when the TC is run in SNPN mode, it is observed that there are no SNPN related MMI prompts. This needs to be implemented. |
| Summary of change | For testcase 9.1.8.1, called the function fl_NR5GC_PreconfigUE(omit); after cell creation. This will ensure that the SNPN configuration commands are called. |
| TTCN module | SMSoverNAS_NR5GC.ttcn |
| MCC160 Comments | Alternative solution implemented – added sending the SNPN data to f_NG_SMS_AT_Config_WithPowering |

Before Change:

function f_TC_9_1_8_1_NR5GC() runs on NR5GC_PTC { //SMS over NAS / MO and MT SMS over NAS - Idle mode var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1, NG_AllCellsOnSamePLMN); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Configure the SMS as per PIXIT using the appropriate AT commands - powering required //Provide the UE with a valid 5G NAS security context to make it send a REGISTRATION REQUEST message including also non-cleartext IEs. f_NG_SMS_AT_Config_WithPowering_EnterState_0B (v_NGC_NASCellA); //@sic R5-201089, R5s200311, R5s201075, R5s201247 sic@ f_NR_TestBody_Set(true); fl_TC_9_1_8_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); //@sic R5s200311 sic@ }

After Change:

function f_TC_9_1_8_1_NR5GC() runs on NR5GC_PTC { //SMS over NAS / MO and MT SMS over NAS - Idle mode var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1, NG_AllCellsOnSamePLMN); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); fl_NR5GC_PreconfigUE(omit); //WA#WI1435286 WA#9_1_8_1 and WA#9_1_8_2 //Configure the SMS as per PIXIT using the appropriate AT commands - powering required //Provide the UE with a valid 5G NAS security context to make it send a REGISTRATION REQUEST message including also non-cleartext IEs. f_NG_SMS_AT_Config_WithPowering_EnterState_0B (v_NGC_NASCellA); //@sic R5-201089, R5s200311, R5s201075, R5s201247 sic@ f_NR_TestBody_Set(true); fl_TC_9_1_8_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); //@sic R5s200311 sic@ }

## Change 2

| Template name | f_TC_9_1_8_2_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 9.1.8.2, first the function f_NG_SMS_AT_Config_WithPowering_EnterState_0B (v_NGC_NASCellA); is called, which sets the cell to serving cell power. Because of this, when the function f_NR5GC_Preamble_SMSoverNASf_NR5GC_Preamble_Steps1_4fl_NR5GC_PreconfigUEf_UT_SwitchOnUE is called, it camps on the cell, while the user is still sending the AT commands for SNPN.This causes TC failure, and needs to be addressed |
| Summary of change | For testacse 9.1.8.2, between the calls to functions f_NG_SMS_AT_Config_WithPowering_EnterState_0B and f_NR5GC_Preamble_SMSoverNAS, called f_NR_SetCellPower(v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); This will ensure that when the user is configuring the UE with SNPN commnds, the UE does not find any suitable cell to camp on. |
| TTCN module | SMSoverNAS_NR5GC.ttcn |
| MCC160 Comments | Alternative solution implemented – added sending the SNPN data to f_NG_SMS_AT_Config_WithPowering as per change 1 implies that preconfiguration and cell power setting are no longer required in f_NR5GC_Preamble_SMSoverNAS. |

Before Change:

function f_TC_9_1_8_2_NR5GC() runs on NR5GC_PTC { //@sic R5-202663 sic@ //SMS over NAS / Multiple MO and MT SMS over NAS - CONNECTED mode var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1, NG_AllCellsOnSamePLMN); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Configure the SMS as per PIXIT using the appropriate AT commands - powering required //Provide the UE with a valid 5G NAS security context to make it send a REGISTRATION REQUEST message including also non-cleartext IEs. f_NG_SMS_AT_Config_WithPowering_EnterState_0B (v_NGC_NASCellA); //@sic R5-201089, R5s200311, R5s201075, R5s201247, R5s201178 sic@ // Bring the UE into state 3N-A, registered with "SMS over NAS supported". f_NR5GC_Preamble_SMSoverNAS(v_NGC_NASCellA); f_NR_TestBody_Set(true); fl_TC_9_1_8_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change:

function f_TC_9_1_8_2_NR5GC() runs on NR5GC_PTC { //@sic R5-202663 sic@ //SMS over NAS / Multiple MO and MT SMS over NAS - CONNECTED mode var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1, NG_AllCellsOnSamePLMN); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Configure the SMS as per PIXIT using the appropriate AT commands - powering required //Provide the UE with a valid 5G NAS security context to make it send a REGISTRATION REQUEST message including also non-cleartext IEs. f_NG_SMS_AT_Config_WithPowering_EnterState_0B (v_NGC_NASCellA); //@sic R5-201089, R5s200311, R5s201075, R5s201247, R5s201178 sic@ f_NR_SetCellPower(v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //WA#WI1435286 WA#9_1_8_2 // Bring the UE into state 3N-A, registered with "SMS over NAS supported". f_NR5GC_Preamble_SMSoverNAS(v_NGC_NASCellA); f_NR_TestBody_Set(true); fl_TC_9_1_8_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

#### MCC160 Implementation

function f_NG_SMS_AT_Config_WithPowering () runs on NR5GC_PTC

{

var boolean v_ConfigSNPN := (pc_SNPN_only and not(f_GetTestcaseAttrib_NR_SNPN(testcasename())));

f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s211299 sic@

if (v_ConfigSNPN) {

fl_NR5GC_SendSNPNData();

}

f_NG_SMS_AT_Config ();

f_UT_SwitchOffUE(UT, false);

}

function f_NR5GC_Preamble_SMSoverNAS (NR_CellId_Type p_NR_CellId,

NR_TESTMODE_STATE_Type p_NR_TESTMODE_STATE := TESTMode_OFF) runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

// Switch on UE

f_NR5GC_SwitchOnAndResetIMS();

// RRC Connection Setup

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

-, -,

cr_NG_Update (-, '1'B) //SMS over NAS supported

)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Preamble");

}

//Steps 5 to 13 of the generic procedure for NR RRC IDLE specified in TS 38.508-1 subclause 4.5.2, Table 4.5.2.2-2: NR RRC_IDLE are performed.

f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo,

p_NR_CellId,

v_ReceivedMsg,

f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)),

p_NR_TESTMODE_STATE,

Initial_Secure);

//Step 14 of the generic procedure for NR RRC IDLE specified in TS 38.508-1 subclause 4.5.2, Table 4.5.2.2-2: NR RRC_IDLE are performed.

// SS transmits REGISTRATION ACCEPT message including 5GS registration result with SMS allowed bit set to "SMS over NAS allowed".

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,  // @sic R5-206426 sic@

f_NR5GC_GetMobileIdGUTI (p_NR_CellId),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)),

{bit2oct(f_NR_CellInfo_GetTAC(p_NR_CellId))}),

-, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -,

'1'B); // SMS over NAS allowed

SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId,

tsc_NR_RbId_SRB1,

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, p_NR_TESTMODE_STATE);

// Go to RRC_CONNECTED

f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_NR_TESTMODE_STATE);

}