3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250328

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4925 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EMM test case 9.2.3.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the test procedure PDP context is gets established before the TAU procedure gets triggered at Step 17 to move from UTRAN to LTE RAT. A UE supporting multi-EPS bearer can have 2 EPS bearer contexts (one transferred from UTRAN RAT and other in the LTE RAT) and will indicate them in EPS bearer context status IE in TAU message, this is not currently handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Passed cr_508_EPS_BearerContextStatusTAUReq as an additional parameter to the template cdr_TAU_Request_CheckAllAuthParams with 2 values for the ebi_5_7 parameter to handle 1 or 2 eps bearer contexts - Used the additional parameter p_EPS_BearerContextStatus for the assignment of epsBearerContextStatus in the cdr_TAU_Request_CheckAllAuthParams template |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-1 CR 5481 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_TC_9_2_3_3_1_Body |
| --- | --- |
| Reason for change | In the test procedure PDP context is gets established before the TAU procedure gets triggered at Step 17 to move from UTRAN to LTE RAT. A UE supporting multi-EPS bearer can have 2 EPS bearer contexts (one transferred from UTRAN RAT and other in the LTE RAT) and will indicate them in EPS bearer context status IE in TAU message, this is not currently handled. |
| Summary of change | Passed cr_508_EPS_BearerContextStatusTAUReq as an additional parameter to the template cdr_TAU_Request_CheckAllAuthParams with 2 values for the ebi_5_7 parameter to handle 1 or 2 eps bearer contexts |
| TTCN module | NAS_TrackingArea_UG.ttcn |
| MCC160 Comment | As there will be at least one eps bearer context, the template should not include ‘ifpresent’ Decision postponed until outcome of prose CR |

Before Change:

function fl_TC_9_2_3_3_1_Body() runs on EUTRA_PTC { var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus; var NAS_MSG_Indication_Type v_NasInd; // ========================================================================== // initialize component specific data structures // ========================================================================== // get all data which are required for a GUTI var GutiParameters_Type v_Guti1_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellB); var GutiParameters_Type v_Guti2_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_Guti1_Params.PLMN_Identity); var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL); // @sic R5s110176 sic@ var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType(); var template (present) EPS_UpdateTypeValue v_UpdateType := f_GetEPSTAUType(NORMAL); // @sic R5s110176 sic@ var EPS_UpdateTypeValue v_UpdateResult := f_GetEPSTAUType(NORMAL); // @sic R5s110176 sic@ ************************************** Skipping code**************************************************** //@siclog "Step 17" siclog@ //+ Check: does the UE send an RRCConnectionSetupComplete with the selectedPLMN-identity set to the value of the registered MME //(i.e. to the value in the GUTI stored on the USIM in EFEPSLOCI), and, the mmegi and mmec are set to the values derived from the GUTI stored on the USIM in EFEPSLOCI; [1,P] // Check: are the contents of the TRACKING AREA UPDATE REQUEST with the correct parameters? [2,3,P] v_TAUReq := cdr_TAU_Request_CheckAllAuthParams(v_UpdateType, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@ v_AdditionalUpdateType, cs_MobileIdentityGuti(omit, v_OldGUTI), // from UTRAN, cr_CiphKeySeqNum('8'H, v_RxdMsg.Authentication.AuthPS.KeySeq), // @sic R5s110726 sic@ cr_PTMSI_Signature(px_PTMSI_SigDef), f_GutiParameters2MobileIdentity('50'O, v_Guti1_Params), // New Value cr_TAI_Any, v_LAI); v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA, // @sic R5s120121 MCC160 implementation sic@ cr_508_RRCConnectionSetupComplete(v_RRC_TI, ?, cr_RegisteredMME(omit, v_GUTIFromUTRAN.MME_GroupId, v_GUTIFromUTRAN.MME_Code)), cr_NAS_Indication(tsc_SHT_IntegrityProtected, v_TAUReq)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ //+ The SS sends TRACKING AREA UPDATE ACCEPT // Note: the default message contents cause the allocation of a new GUTI and new TAI list ******************************************* Skipping Code *****************************************************

After Change:

function fl_TC_9_2_3_3_1_Body() runs on EUTRA_PTC { var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus; var NAS_MSG_Indication_Type v_NasInd; // ========================================================================== // initialize component specific data structures // ========================================================================== // get all data which are required for a GUTI var GutiParameters_Type v_Guti1_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellB); var GutiParameters_Type v_Guti2_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_Guti1_Params.PLMN_Identity); var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL); // @sic R5s110176 sic@ var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType(); var template (present) EPS_UpdateTypeValue v_UpdateType := f_GetEPSTAUType(NORMAL); // @sic R5s110176 sic@ var EPS_UpdateTypeValue v_UpdateResult := f_GetEPSTAUType(NORMAL); // @sic R5s110176 sic@ ************************************** Skipping code**************************************************** //@siclog "Step 17" siclog@ //+ Check: does the UE send an RRCConnectionSetupComplete with the selectedPLMN-identity set to the value of the registered MME //(i.e. to the value in the GUTI stored on the USIM in EFEPSLOCI), and, the mmegi and mmec are set to the values derived from the GUTI stored on the USIM in EFEPSLOCI; [1,P] // Check: are the contents of the TRACKING AREA UPDATE REQUEST with the correct parameters? [2,3,P] v_TAUReq := cdr_TAU_Request_CheckAllAuthParams(v_UpdateType, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@ v_AdditionalUpdateType, cs_MobileIdentityGuti(omit, v_OldGUTI), // from UTRAN, cr_CiphKeySeqNum('8'H, v_RxdMsg.Authentication.AuthPS.KeySeq), // @sic R5s110726 sic@ cr_PTMSI_Signature(px_PTMSI_SigDef), f_GutiParameters2MobileIdentity('50'O, v_Guti1_Params), // New Value cr_TAI_Any, v_LAI, -, cr_508_EPS_BearerContextStatusTAUReq(('001'B, '011'B)) ifpresent); v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA, // @sic R5s120121 MCC160 implementation sic@ cr_508_RRCConnectionSetupComplete(v_RRC_TI, ?, cr_RegisteredMME(omit, v_GUTIFromUTRAN.MME_GroupId, v_GUTIFromUTRAN.MME_Code)), cr_NAS_Indication(tsc_SHT_IntegrityProtected, v_TAUReq)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ //+ The SS sends TRACKING AREA UPDATE ACCEPT // Note: the default message contents cause the allocation of a new GUTI and new TAI list ******************************************* Skipping Code *****************************************************

## Change 2

| Function name | template (present) NAS_UL_Message_Type cdr_TAU_Request |
| --- | --- |
| Reason for change | See Reason for change in Change 1 |
| Summary of change | Used the additional parameter p_EPS_BearerContextStatus for the assignment of epsBearerContextStatus in the cdr_TAU_Request_CheckAllAuthParams template. |
| TTCN module | NAS_TrackingArea_UG.ttcn |
| MCC160 Comment | Accepted in principle but the default value should include ifpresent |

Before Change:

template (present) NAS_UL_Message_Type cdr_TAU_Request_CheckAllAuthParams(template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue, NAS_KsiValue p_KsiValue, template DRXparameter p_DRXparameter := omit, // @sic R5s120210 sic@ template AdditionalUpdateType p_AdditionalUpdateType, template (present) MobileIdentity p_OldGuti, template (present) CiphKeySeqNum p_CiphKeySeqNum, template PTMSI_Signature p_Sig, template MobileIdentity p_Guti, template TrackingAreaId p_LastTai, template LocAreaId p_LAI, template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap) // @sic R5s110801 sic@ modifies cr_508_TAU_Request := { /* @status APPROVED (LTE_IRAT) */ tRACKING_AREA_UPDATE_REQUEST := { oldGuti := p_OldGuti, gprsCipheringKeySeqNum := p_CiphKeySeqNum, oldPtmsiSignature := p_Sig, additionalGuti := p_Guti, nonce := cr_NonceUE_Any, ueNetworkCapability := cr_UENetworkCap_Any('58'O), lastVisitedRegisteredTai := p_LastTai, drxParameter := p_DRXparameter, ueRadioCapNeeded := p_UeRadioCap, epsBearerContextStatus := cr_508_EPS_BearerContextStatusTAUReq ifpresent, msNetworkCapability := cr_MS_NetworkCap_Any, oldLai := p_LAI, tmsiStatus := omit } };

After Change:

template (present) NAS_UL_Message_Type cdr_TAU_Request_CheckAllAuthParams(template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue, NAS_KsiValue p_KsiValue, template DRXparameter p_DRXparameter := omit, // @sic R5s120210 sic@ template AdditionalUpdateType p_AdditionalUpdateType, template (present) MobileIdentity p_OldGuti, template (present) CiphKeySeqNum p_CiphKeySeqNum, template PTMSI_Signature p_Sig, template MobileIdentity p_Guti, template TrackingAreaId p_LastTai, template LocAreaId p_LAI, template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap, template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_508_EPS_BearerContextStatusTAUReq) modifies cr_508_TAU_Request := { /* @status APPROVED (LTE_IRAT) */ tRACKING_AREA_UPDATE_REQUEST := { oldGuti := p_OldGuti, gprsCipheringKeySeqNum := p_CiphKeySeqNum, oldPtmsiSignature := p_Sig, additionalGuti := p_Guti, nonce := cr_NonceUE_Any, ueNetworkCapability := cr_UENetworkCap_Any('58'O), lastVisitedRegisteredTai := p_LastTai, drxParameter := p_DRXparameter, ueRadioCapNeeded := p_UeRadioCap, epsBearerContextStatus := p_EPS_BearerContextStatus, msNetworkCapability := cr_MS_NetworkCap_Any, oldLai := p_LAI, tmsiStatus := omit } };