3GPP TSG-RAN WG5 Meeting #61 	R5-134303r2

# San Francisco, USA, 11th – 15th November 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | CRNum | rev | - | Current version: | 10.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to INVITE message in case of Emergency NoRegistration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  |  | Date: |  | 2013-10-31 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | - Acc. 24.229 and section 5.11.2, it refers to draft-montemurro-gsma-imei-urn as shown below: the Contact header field includes an instance-id feature tag containing an IMEI URN as specified in draft-montemurro-gsma-imei-urn [153]; As per draft-montemurro-gsma-imei-urn-11 document, format of IMEI URN is given below: example: urn:gsma:imei:90420156-025763-0svn=42 Template: gsma-urn = "urn:gsma:" gsma-specifier *(":" gsma-specifier-defined-substring) *(";" gsma-specifier-param) <gsma-specifier-defined-substring> defined using ABNF( RFC 5234). Acc. RFC5234 and section 2.1, <> is not required for definition of gsma-specifier-defined-substring . In this document, <> are used for ‘of gsma-specifier-defined-substring ‘ which will be wrongly interpreted as part of the character string . - In case emergency call without emergency registration, record-route would be present of P-CSCF, E-PCSCF. As per 24.229, section 5.2.10.2 P-CSCF info needs to be included in record-route: the P-CSCF may rewrite the port number of its own Record-Route entry to an unprotected port where the P-CSCFwants to receive the subsequent incoming requests from the UE belonging to this dialog. And 24.229, section 5.11.2 point 9) mentions E-CSCF info needs to be included in the record-route: create a Record-Route header field containing its own SIP URI; 180 Ringing and 200 OK messages record route entries are modified and added in the reverse order. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed entry angle bracket ‘ < ‘ from gsma-specifier-defined-substring and added it this identifier under curly brackets ‘()’ - Record-Route entries for 180 Ringing, 200 OK in case of no emergency registration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1, A.2.6, A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | R5-134303->R5-133403r1-> R5-134303r2 |  |  |  |  |  |  |  |  |

## A.2.1	INVITE for MO Call Setup

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] RFC 5031 [97] TS 24.229 [10] 5.1.2A.1.3, 5.1.2A.1.5, 7.2A.10 |
| Method |  | INVITE |  |  |
| Request-URI | A4 | Px_CalleeUri px_CalleeURI may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting. |  |  |
| Request-URI | A5 | px_CalleeContactUri |  |  |
| Request-URI | A6, A7 | emergency service URN beginning as urn:service:sos |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] RFC 3581 [96] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1, A7 | IP address or FQDN and protected server port of the UE |  |  |
|  | A2 | IP address or FQDN, port (optional) and not checked |  |  |
|  | A6 | IP address and unprotected server port of the UE |  |  |
| response-port | A6 | rport (when using UDP) |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| Route |  | order of the parameters in this header must be like in this table |  | RFC 3261 [15] |
| route-param | A1 | <sip:px_pcscf: protected server port of SS ;lr>, <sip:px_scscf;lr> |  |  |
|  | A2 | <sip:px_pcscf: unprotected server port of SS (optional);lr>, <sip:px_scscf;lr |  |  |
|  | A6 | <sip:px_pcscf: unprotected server port of SS > |  |  |
|  | A7 | <sip:px_pcscf: protected server port of SS > |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A6 | Any SIP URI with display name as “Anonymous” |  |  |
|  | A7 | emergency public user identity (NOTE 3) |  |  |
|  | A4 | any SIP URI being subscribed and registered as listed in the XML body of the NOTIFY request; additionally when there is a P-Preferred-Identity header within the INVITE request the SIP URI shall match the URI within the P-Preferred-Identity header |  |  |
| tag | A4 | must be present, value not checked |  |  |
| addr-spec | A5 | local SIP URI of the UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| tag | A5 | local tag value corresponding to the SIP URI of the UE in the same dialog. (In the earlier requests within the same dialog this tag appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| To |  |  |  | RFC 3261 [15] RFC 5031 [97] |
| addr-spec | A6, A7 | emergency service URN beginning as urn:service:sos |  |  |
| addr-spec | A4 | px_CalleeUri |  |  |
| tag | A4 | not present |  |  |
| addr-spec | A5 | remote SIP URI of SS (i.e. the remote UE) as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| tag | A5 | remote tag value corresponding to the SIP URI of the SS in the same dialog. (In the earlier requests within the same dialog this tag appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid | A4 | value different to that received in REGISTER message |  |  |
| callid | A5 | value of Call-ID as in any previous request in the same dialog |  |  |
| Session-ID |  | (header exists when A9) |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | A4 | value different to that received in REGISTER message |  |  |
| sess-id | A5 | value of Session-ID as in any previous request in the same dialog |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A4 | must be present, value not checked |  |  |
| value | A5 | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |
| method |  | INVITE |  |  |
| Supported |  |  |  | RFC 3261 [15] draft-ietf-sipcore-location-conveyance [98] |
| option-tag |  | 100rel |  |  |
| option-tag | A8 | geolocation |  |  |
| Geolocation | A8 |  | Rel-9 | draft-ietf-sipcore-location-conveyance [98] |
| locationURI |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |
| inserted-param |  | inserted-by=“ value of sent-by within the Via header” |  |  |
| routing-param |  | routing-allowed=yes |  |  |
| Require |  | (header optional in A2 and not present in A6) |  | RFC 3261 [15] RFC 3312 [31] RFC 3329 [21] |
| option-tag | A1, A7 | sec-agree |  |  |
| Proxy-Require |  | (header optional in A2 and not present in A6) |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1, A7 | sec-agree |  |  |
| Security-Verify | A1, A7 | (not present in A2 or A6) |  | RFC 3329 [21] |
| sec-mechanism |  | same value as SecurityServer header sent by SS |  |  |
| Contact |  |  |  | RFC 3261 [15] RFC 3840 [63] draft-ietf-sip-gruu [61] RFC 5626 [109] |
| addr-spec | A1, A7 | SIP URI with IP address or FQDN and protected server port of UE or GRUU as returned by the SS in registration |  |  |
|  | A2 | SIP URI with IP address or FQDN and unprotected server port of UE or GRUU as returned by the SS in registration |  |  |
|  | A6 | SIP URI with IP address and unprotected server port of UE |  |  |
| feature-param | A6 | +sip.instance="<urn:gsma:imei:(gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in draft-montemurro-gsma-imei-urn, without optional parameters |  |  |
|  | A3 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2) |  |  |
| feature-param | A10 | video |  |  |
| feature-param | A12 | g.3gpp.srvcc-alerting |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp or multipart/mixed (when A8) |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info |  | (header optional when A2) |  | RFC 3455 [18] |
| access-net-spec |  | access network technology and, if applicable, the cell ID |  |  |
| Accept |  | (header optional when A5) | Rel-7 | RFC 3261 [15] |
| Media-range | A4 | application/sdp,application/3gpp-ims+xml (additional medias can be added in any order) |  |  |
| P-Preferred-Service |  |  |  | draft-drage-sipping-service-identification [68] |
| Service-ID | A3 | urn:urn-7:3gpp-service.ims.icsi.mmtel |  |  |
| P-Preferred-Identity |  |  |  | RFC 3325 [89] |
| PPreferredID-value | A7 | px_EmergencyPublicUserIdentity |  |  |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | A3 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2) |  |  |
| ac-value | A11 | video |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| Value |  | length of message-body |  |  |
| Message-body |  | The message body shall contain the following elements: a) SDP offer, contents as specified within the specific test cases referring to this common message. If condition A8 applies the SDP shall be one element within the multipart-MIME encapsulation; b) if condition A8 applies, the multipart-mime body shall also contain a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more 'geopriv' elements, each containing: - One 'location-info' element describing the location of the UE; and - One 'usage-rules' element describing the limitations of the usage of the location info. |  | RFC 4119 [99] |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A3 | UE supports MTSI (A.3A/50 3GPP TS 34.229-2 [5]) |
| A4 | INVITE creating a dialog |
| A5 | re-INVITE within a dialog |
| A6 | INVITE for creating an emergency session in case of no registration |
| A7 | INVITE for creating an emergency session within an emergency registration |
| A8 | UE uses Geolocation header to provide its geographical location for emergency session setup, has obtained its location and is setting up an emergency session |
| A9 | UE supports Session-ID (A.12/30 3GPP TS 34.229-2 [5]) |
| A10 | UE indicates video media feature tag in REGISTER and INVITE request (A.12/32 3GPP TS 34.229-2 [5]) |
| A11 | INVITE for creating a video call and UE supports video media feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A12 | INVITE for creating a voice call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE registered or the temporary user id (derived from IMSI) else.

## A.2.6	180 Ringing for INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 180 |  |  |
| Reason-Phrase |  | Ringing |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A7 | as defined for the common 183 response, see A.2.3 <sip:orig@px_ecscf;lr>, <sip:px_pcscf:unprotected server port of SS;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in INVITE message |  |  |
| Require |  |  |  | RFC 3261 [15] |
| option-tag | A3 | 100rel |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | same value as received in INVITE message |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | as defined for the common 183 response, see A.2.3 |  |  |
| P-Asserted-Identity | A4 | px_EmergencyTelURI |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec |  | as defined for the common 183 response, see A.2.3 |  |  |
| feature-param | A5 | g.3gpp.srvcc-alerting |  |  |
| Rseq |  |  |  | RFC 3262 [33] |
| response-num | A3 | previous RSeq number sent in the same direction incremented by one |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| Session-ID |  |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id |  | same value as received in INVITE message, if Session-ID header field exists in received INVITE message, otherwise, not present. |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in INVITE message |  |  |
| P-Access-Network-Info |  | (header missing when A1) |  |  |
| access-net-spec | A2 | access network technology and, if applicable, the cell ID |  |  |
| Feature-Caps |  |  |  |  |
| feature-param | A6 | g.3gpp.srvcc-alerting |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 180 sent by the SS |
| A2 | 180 sent by the UE |
| A3 | Response sent reliably (e.g. always when it contains an SDP body) |
| A4 | 180 sent by the SS when setting up an emergency call |
| A5 | 180 sent by the UE for a voice call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |
| A6 A7 | 180 sent by the SS for a voice call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) Response sent by SS for emergency call without emergency registration |

## A.3.1	200 OK for other requests than REGISTER or SUBSCRIBE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 200 |  |  |
| Reason-Phrase |  | OK |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in request |  |  |
| Record-Route |  | order of the parameters in this header must be like in this table |  |  |
| rec-route | A1,A2 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@px_scscf;lr>, <sip:px_pcscf: protected server port of SS ;lr> |  |  |
| rec-route rec-route | A3,A4 A7 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@px_scscf;lr>, <sip:px_pcscf: unprotected server port of SS (optional);lr> <sip:orig@px_ecscf;lr>, <sip:px_pcscf:unprotected server port of SS;lr> |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request or common to-tag (invite) added if missing from request |  |  |
| P-Asserted-Identity | A6 | px_EmergencyTelURI |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  |  |
| addr-spec | A1, A3 | px_CalleeContactUri |  |  |
| addr-spec | A2 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
| addr-spec | A4 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in request |  |  |
| Session-ID |  |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id |  | same value as received in request, if Session-ID header field exists in received request, otherwise, not present. |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in request |  |  |
| P-Access-Network-Info | A1, A2, A3 | NOTE: header optional when A4 |  |  |
| access-net-spec | A5 | access network technology and, if applicable, the cell ID |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | 0 |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE/UPDATE (IMS security ,A.6a/2 TS 34.229-2 [5])) |
| A2 | Response sent by UE for INVITE/UPDATE (IMS security ,A.6a/2 TS 34.229-2 [5])) |
| A3 | Response sent by SS for INVITE/UPDATE (GIBA, A.6a/1 TS 34.229-2 [5])) |
| A4 | Response sent by UE for INVITE/UPDATE (GIBA, A.6a/1 TS 34.229-2 [5])) |
| A5 | Any response sent by the UE within a dialog |
| A6 A7 | Response sent by SS for INVITE for emergency call Response sent by SS for INVITE for emergency call without emergency registration |