3GPP TSG-RAN WG5 Meeting #69 	R5-155555r1

# Anaheim, US, 16th – 20th Nov 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2290 | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New TC of Aggregate Power Control Tolerance for 2UL inter-band CA(6.3.5A.3.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, CATR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  |  | Date: |  | 2015-11-6 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The following test case (as described in the Work Plan ‘Further Rel-12 configurations for LTE Advanced Carrier Aggregation with 2UL (Rel-12)‘) is currently missing: 6.3.5A.3.2: Aggregate Power Control Tolerance for CA (inter-band DL CA and UL CA) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduced test case 6.3.5A.3.2: Aggregate Power Control Tolerance for CA (inter-band DL CA and UL CA) - Updated cover sheet |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5A.3.2 (new), F.1.1, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | 36.521-2 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-155555R5-155555r1 - Updated cover sheet and fixed typos |  |  |  |  |  |  |  |  |

< start of changes >

##### 6.3.5A.3.2	Aggregate power control tolerance for CA (inter-band DL CA and UL CA)

###### 6.3.5A.3.2.1	Test purpose

To verify the ability of a UE to maintain its power in non-contiguous transmission within 21 ms in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in TS 36.213 are constant on all active component carriers.

###### 6.3.5A.3.2.2	Test applicability

This test applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

###### 6.3.5A.3.2.3	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the aggregate power tolerance is specified on each component carrier exceed the minimum output power as defined in subclause 6.3.2A and the total power is limited by maximum output power as defined in subclause 6.2.2A. The requirements defined in Table 6.3.5A.3.2.3-1 shall apply on each component carrier with both component carriers active. The requirements can be tested by time aligning any transmission gaps on both the component carriers.

Table 6.3.5A.3.2.3-1: Aggregate power control tolerance

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ±2.5 dB |
| 0 dB | PUSCH | ±3.5 dB |
| NOTE: The UE transmission gap is 4 ms. TPC command is transmitted via PDCCH 4 subframes preceding each PUCCH/PUSCH transmission. |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.3.5A.3.1.

###### 6.3.5A.3.2.4	Test description

6.3.5A.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.3.2.4.1-1and table 6.3.5A.3.2.4.1-2. The details of the uplink reference measurement channel (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.5A.3.2.4.1-1: Test Configuration Table: PUCCH sub-test

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | Normal |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | C: Mid range |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg |
| Test Parameters for CA Configurations |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  | Uplink Configuration |  |
| PCC NRB | SCCs NRB | CC Mod | PCC & SCC RB allocation | FDD: PUCCH format = Format 1b with channel selection/Format 3 TDD: PUCCH format = Format 1b with channel selection/Format 3 |  |
| 6 | 25 | QPSK | 6+25 |  |  |
| 6 | 50 | QPSK | 6+50 |  |  |
| 25 | 15 | QPSK | 25+15 |  |  |
| 25 | 25 | QPSK | 25+25 |  |  |
| 25 | 50 | QPSK | 25+50 |  |  |
| 50 | 50 | QPSK | 50+50 |  |  |
| 50 | 100 | QPSK | 50+100 |  |  |
| 75 | 75 | QPSK | 75+75 |  |  |
| 100 | 75 | QPSK | 100+75 |  |  |
| 100 | 100 | QPSK | 100+100 |  |  |
| Note 1 :CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2 |  |  |  |  |  |

Table 6.3.5A.3.2.4.1-2: Test Configuration Table: PUSCH sub-test

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | C: Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2 Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , only the first of those is tested, according to the order on the Test Configuration Table list. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channel is set according to Table 6.3.5A.3.2.4.1-1 (PUCCH sub-test) and Table 6.3.5A.3.2.4.1-2 (PUSCH sub-test).

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.3.5A.3.2.4.3.

6.3.5A.3.2.4.2	Test procedure

Figure 6.3.5A.3.2.4.2-1: Test uplink transmission

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	The procedure is separated in two subtests to verify PUCCH and PUSCH aggregate power control tolerance respectively. The uplink transmission patterns are described in figure 6.3.5A.3.2.4.2-1.

5.	PUCCH sub test:

5.1	The SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.3.5A.3.2.4.1-1 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH on PCC. Send the appropriate TPC commands for PUCCH to the UE to ensure that the UE transmits PUCCH at 0dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz or at 0dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

5.2	Every 5 subframes transmit to the UE downlink PDSCH MAC padding bits as well as 0 dB TPC command for PUCCH via the PDCCH to make the UE transmit ACK/NACK on the PUCCH with transmission gap of 4 subframes. The downlink transmission is scheduled in the appropriate sub-frames to make the UE transmit PUCCH as described in figure 6.3.5A.3.2.4.2-1.

5.3	Measure the power of 5 consecutive PUCCH transmissions on PCC to verify the UE transmitted PUCCH power is maintained within 21 ms. The transient periods of 20us are excluded from the power measurement.

6.	PUSCH sub test:

6.1	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.3.5A.3.2.4.1-2 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH to the UE to ensure that the UE transmits PUSCH at 0dBm +/- 3.2.dB for carrier frequency f ≤ 3.0GHz or at 0dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.2	Every 5 subframes schedule the UE's PUSCH data transmission and transmit 0 dB TPC command for PUSCH via the PDCCH to make the UE transmit PUSCH with 4 subframes gap. The uplink transmission patterns are described in figure 6.3.5A.3.2.4.2-1.

6.3	Measure the power on both PCC and SCC of 5 consecutive PUSCH transmissions to verify the UE transmitted PUSCH power is maintained within 21 ms on each component carrier. The transient periods of 20us are excluded from the power measurement.

6.3.5A.3.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

###### 6.3.5A.3.2.5	Test requirement

For inter-band contiguous carrier aggregation, the aggregate power control tolerance per component carrier measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3.5A.3.2.5-1.

Table 6.3.5A.3.2.5-1: Aggregate power control tolerance for intra-band contiguous CA

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ±3.2 dB |
| 0 dB | PUSCH | ±4.2 dB |
| NOTE: The UE transmission gap is 4 ms. TPC command is transmitted via PDCCH 4 subframes preceding each PUCCH/PUSCH transmission. |  |  |

## <Unchanged sections skipped>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.2 UE Maximum Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2_1 Maximum Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 for each CC |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 for each CC |  |
| 6.2.2B UE Maximum Output Power for UL-MIMO | Same as 6.2.2 for each antenna |  |
| 6.2.2E UE Maximum Output Power for UE category 0 | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.3_1 Maximum Power Reduction (MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 for each antenna |  |
| 6.2.3_2 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH | Same as 6.2.3 |  |
| 6.2.4 UE Maximum Output Power with additional requirements | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 for each antenna |  |
| 6.2.5 Configured UE transmitted Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.5 for each CC |  |
| 6.2.5A.2 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.5 for each CC |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.5 for each CC |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | Same as 6.2.5 for each antenna |  |
| 6.3.2 Minimum Output Power | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 for each antenna |  |
| 6.3.3 Transmission ON/OFF Power | Transmission OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.2 UE Transmit OFF power for CA (inter-band DL CA and UL CA) | Same as 6.3.3A.1 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 for each antenna |  |
| 6.3.4.1 General ON/OFF time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4.2 PRACH and SRS time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4A.1.1 |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 for each antenna |  |
| 6.3.5.1 Power Control Absolute power tolerance | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5.2 Power Control Relative power tolerance | ±0.7 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | ±1.0 dB, f ≤ 3.0GHz |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | ±0.7 dB |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.2 for each CC |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 for each antenna |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 for each antenna |  |
| 6.3.5.3 Aggregate power control tolerance | ±0.7 dB |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | ±0.7 dB |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 for each antenna |  |
| 6.5.1 Frequency Error | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz DL Signal level: ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | TBD |  |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 for each antenna DL signal level same as 6.5.1 |  |
| 6.5.2.1 Error Vector Magnitude | PUSCH: ± 2.5% PUCCH: ± 2.5% PRACH: ± 2.5% |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 for each antenna |  |
| 6.5.2.1A PUSCH-EVM with exclusion period | PUSCH : ± 2.5% |  |
| 6.5.2.2 Carrier leakage | ±0.8dB |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | TBD |  |
| 6.5.2.3 In-band emissions for non allocated RB | ±0.8dB |  |
| 6.5.2.4 EVM equalizer Spectrum flatness | ±1.4dB |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | TBD |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 for each antenna |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 for each antenna |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 for each antenna |  |
| 6.6.1 Occupied bandwidth | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 for each CC |  |
| 6.6.1A.3 Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 for each antenna |  |
| 6.6.2.1 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 and 6.6.2.2 |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 for each antenna |  |
| 6.6.2.2 Additional Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.2A Additional Spectrum Emission Mask for CA | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | ±0.8 dB |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | ±0.8 dB |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 for each antenna |  |
| 6.6.2.4 Additional ACLR requirements | ±0.8 dB |  |
| 6.6.3.1 Transmitter Spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |
| 6.6.3.1_1 Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1, at each antenna used for transmission | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 6.6.3.2 Spurious emission band UE co-existence | ± 2.0 dB for results > -60 dBm, f ≤ 3.0GHz ±2.5 dB, 3.0GHz < f ≤ 4.2GHz ± 3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.2 Spurious emission band UE co-existence for CA (inter-band DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.3 Additional spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB NS-07 769 ≤ f ≤ 775 MHz: ± 1.5 dB |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 9kHz < f ≤ 4 GHz: ± 2.0 dB |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO | Same as 6.6.3.2 at each antenna used for transmission |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO | Same as 6.6.3.3, at each antenna used for transmission |  |
| 6.7 Transmit intermodulation | ± 2.6 dB, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] f ≤ 3.0GHz Wanted signal setting ± 0.7dB CW Interferer level ± 1.0dB Wanted signal meas ± 0.7dB Intermodulation product measurement error ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal setting ± 1.0dB CW Interferer level ± 1.3dB Wanted signal meas ± 1.0dB Intermodulation product measurement error ± 1.0dB |
| 6.7A.1 Transmit intermodulation | Same as 6.7, at each antenna used for transmission |  |
| 6.8B Time alignment error for UL-MIMO | ±25 ns |  |

## <Unchanged sections skipped>

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 6.2.2 UE Maximum Output Power | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-3 dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-4.0 dB |
| 6.2.2_1 UE Maximum Output Power for HPUE | f ≤ 3.0GHz Power class 1: 31dBm +2/-3dB | 0.7dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.2B UE Maximum Output Power for UL-MIMO | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2/-3 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-4 dB | Same as 6.2.2 | Formula: Upper limit +TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2.7/-3.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-5.0 dB Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.2E UE Maximum Output Power UE category 0 | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.3 Maximum Power Reduction | Power class 3 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 4.7dB QPSK: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 6.0dB |
| 6.2.3_1 Maximum Power Reduction for HPUE | Power class 1 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 1: QPSK: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 5.7dB |
| 6.2.3_2 UE Maximum Output Power for Multi-Cluster PUSCH | Same as 6.2.2 | Same as 6.2.2 | Same as 6.2.2 |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +2.7 / - 14.2dB 23dBm +3.0 / - 4.0dB (+2/-3 limit for B42) 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.5dB 23dBm +3.0 / - 14.5dB |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 | Same as 6.2.3 | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.4 UE Maximum Output Power with additional requirements | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB For network signalled value NS_03 , NS_04 (5MHz only), NS_05, to NS_06: A-MPR ≤ 1dB For network signalled value NS-04; Depending on the RB_start and RB allocation (10MHz, 15MHz and 20MHz): For 10MHz Region A with RB_start=0 – 12: A-MPR ≤ 3dB. Region B with RB_start=13 – 36 : A-MPR ≤ 2dB. Region C with RB_start=37 – 49 : A-MPR ≤ 3dB. For 15MHz Region A with RB_start=0 – 18: A-MPR ≤ 3dB. Region B with RB_start=19 – 55 : A-MPR ≤ 2dB. Region C with RB_start=56 – 74 : A-MPR ≤ 3dB. For 20MHz Region A with RB_start=0 – 24: A-MPR ≤ 3dB. Region B with RB_start=25 – 74 : A-MPR ≤ 2dB. Region C with RB_start=75 – 99 : A-MPR ≤ 3dB. For network signalled value NS-11; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz Fc < 2012.5 Region A with RB_start=0 – 4: A-MPR ≤ 15dB. Region B with RB_start=5 – 21: A-MPR ≤ 7dB (LCRB=7-50) or ≤ 10dB (LCRB=0-6 & ≥50). Region C with RB_start=22 – 56: A-MPR ≤ 0dB (LCRB=≤25) or ≤ 6dB (LCRB>25). Region D with RB_start=57 – 74: A-MPR ≤ 15dB. For 15MHz Fc = 2012.5 Region A with RB_start=0 – 12: A-MPR ≤ 10dB. Region B with RB_start=13 – 39: A-MPR ≤ 6dB (LCRB=≥ 30) or ≤ 0dB (LCRB=< 30). Region C with RB_start=40 – 65: A-MPR ≤ 2dB. Region D with RB_start=66 – 74: A-MPR ≤ 6.5dB. For 20MHz Region A with RB_start=0 – 12: A-MPR ≤ 15dB. Region B with RB_start=13 – 29: A-MPR ≤ 7dB (LCRB=10-60) or ≤ 10dB (LCRB=1-9 & > 60). Region C with RB_start=30 – 68: A-MPR ≤ 0dB (LCRB=1-24) or ≤ 7dB (LCRB= ≥ 25). Region D with RB_start=69 – 99: A-MPR ≤ 15dB. For network signalled value NS-20; Depending on the RB_start and RB allocation (5MHZ, 10MHz, 15MHz and 20MHz): For 5MHz Region A with RB_start=≤24: A-MPR ≤ 17dB. Region B with RB_start=0 – 3: A-MPR ≤ 1dB (LCRB=15-19) or ≤ 4dB (LCRB= ≥ 20). Region C with RB_start=4 – 6: A-MPR ≤ 2dB. Region D with RB_start=≤24: A-MPR ≤ 0dB. For 10MHz Fc = 2005 Region A with RB_start=0 – 25: A-MPR ≤ 16dB. Region B with RB_start=26 – 34: A-MPR ≤ 2dB (LCRB=8-15) or ≤ 5dB (LCRB= > 15). Region C with RB_start=35 – 49: A-MPR ≤ 6dB. For 10MHz Fc = 2015 Region A with RB_start=0 – 5: A-MPR ≤ 4dB. Region B with RB_start=6 – 10: A-MPR ≤ 2dB. For 15MHz Region A with RB_start=0 – 14: A-MPR ≤ 11dB (LCRB=1-9 & 40-5) or ≤ 6dB (LCRB= 10-39). Region B with RB_start=15 – 24: A-MPR ≤ 1dB (LCRB=24-29) or ≤ 7dB (LCRB=≥30). Region C with RB_start=25 – 39: A-MPR ≤ 5dB. Region D with RB_start=61 – 74: A-MPR ≤ 6dB. For 20MHz Region A with RB_start=0 – 21: A-MPR ≤ 17dB. Region B with RB_start=22 – 31: A-MPR ≤ 12dB (LCRB=1-9 & 31-75) or ≤ 6dB (LCRB=10-30). Region C with RB_start=32 – 38: A-MPR ≤ 9dB. Region D with RB_start=39 – 49: A-MPR ≤ 7dB. Region E with RB_start=50 – 69: A-MPR ≤ 5dB. Region F with RB_start=70 – 99: A-MPR ≤ 16dB. | 0.7 dB | Formula: Upper limit + TT, A: Lower limit – TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – MPR – TT, C: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – TT, D: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03: [A]: 2, 5, 10, 15, 20, 25 [B]: 1, 3, 7 [C]: 9, 14, 19, 24 [D]: 4, 6, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 Network signalled value NS_04 (5, 10, 15, 20MHz): [A] 3 [B] 10, 11, 19, 20, 28, 29 [C] 2, 6, 7, 13, 14, 15, 16, 23, 24, 25, 32 [D] 1, 4, 5, 8, 9, 12, 17, 18, 21, 22, 26, 27, 30, 31 Network signalled value NS_05: [A] 1, 3, 4, 8, 9, 14, 15, [B] 2, 5, 10, 11, 16, 17 [C] None [D] 6, 7, 12, 13, 18, 19 Network signalled value NS_06: [A]: 2, 5, 8, 11, 14, 17 [B]: 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 [C]: None [D]: None Network signalled value NS_07: [A]:3, 8, 12 [B]: 7, 9 [C]: 1, 2, 5, 13, 15 [D]: 4, 6, 10, 11, 14, 16 Network signalled value NS_08: [A]:1, 2, 4, 5, 12 [B]: 3, 6, 11, 13 [C]: None [D]: 7, 8, 9, 10, 14, 15, 16, 17 Network signalled value NS_11: [A]: 5c, 6b, 10c, 11c, 25 [B]: 8b, 12c, 13c, 14c, 17a, 17b, 21a, 21b, 28 [C]: 5a, 5b, 6a, 6c, 10a, 10b, 11, 11b, 15a, 15b, 16a, 16b, 24 [D]: 1a, 1b, 2a, 2b, 3a, 3b, 4a, 4b ,, 7a,7b,7c, 8a, 8c, 9a, 9b,9c, 12a, 12b, 13a, 13b, 14a, 14b, 18a, 18b, 19a, 19b, 20a, 20b, 22a, 22b, 23a, 23b, 26, 27, 29, 30 Network signalled value NS_12: [A]: 3, 8, 13 [B]: None [C]: 1, 4, 6, 11 [D]: 2, 5, 7, 9, 10, 12, 14, 15 Network signalled value NS_13: [A]: None [B]: 3, 4 [C]: 1 [D]: 2, 5 Network signalled value NS_14: [A]: None [B]: 2, 4, 7, 9 [C]: 1, 6 [D]: 3, 5, 8, 10 Network signalled value NS_15: [A]: 1, 7, 14 [B]: 3, 6, 10, 15, 19, 21, 22, 25 [C]: 12, 13 [D]: 2, 4, 5, 8, 9, 11, 16, 17, 18, 20, 23, 24, 26 Network signalled value NS_20: [A]: 1b, 1c, 1d, 6b, 7b [B]: 2d, 3d, 4d, 5d, 9b [C]: 1a, 6a, 7a, 11, 18, 19 [D]: 2a, 2b, 2c, 3a, 3b, 3c, 4a, 4b, 4c, 5a, 5b, 5c, 8a, 8b, 9a, 10a, 11b, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26 |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 1: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 1: For network signalled value NS_06: A-MPR 0dB | 0.7dB | Formula: Upper limit + TT, (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | Power class 3: For network signalled value CA_NS_01: [TBD] For network signalled value CA_NS_02: [TBD] For network signalled value CA_NS_03: [TBD] | Same as 6.2.4 | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 | Same as 6.2.4 | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2.5 Configured UE transmitted Output Power | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |
| 6.2.5A.2 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | f ≤ 3.0GHz 16 ≤ PCMAX < 20 ± 5.0 11≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – TT 16 ≤ PCMAX < 20 ± 5.7 11 ≤ PCMAX < 16 ± 6.7 -40 ≤ PCMAX < 11 ± 7.7 |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | f ≤ 3.0GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] 3.0GHz < f ≤ 4.2GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] | Same as 6.2.5 | Formula: Upper limit + TT, Lower limit – TT [16] ≤ PCMAX < [20] ± [5.7] [11] ≤ PCMAX < [16] ± [6.7] [-40] ≤ PCMAX < [11] ± [7.7] [16] ≤ PCMAX < [20] ± [6.0] [11] ≤ PCMAX < [16] ± [7.0] [-40] ≤ PCMAX < [11] ± [8.0] Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.2 Minimum Output Power | f ≤ 3.0GHz -40 dBm 3.0GHz < f ≤ 4.2GHz -40 dBm | 1.0 dB 1.3 dB | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–38.7 dBm |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 | Same as 6.3.2 | [Same as 6.3.2 |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 | Same as 6.3.2 | Same as 6.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.3 Transmission ON/OFF Power | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm | 1.5 dB 1.8 dB | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.3A.2 Transmit OFF Power for CA (inter-band DL CA and UL CA) | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 | Same as 6.3.3 | Same as 6.3.3 Uplink power measurement applies to each Tx antenna connector |
| 6.3.4.1 General ON/OFF time mask | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 | Same as 6.3.4.1 for each CC |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4.1 | Same as 6.3.4.1 | Same as 6.3.4.1 |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 | Same as 6.3.4.1 for each CC |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 | Same as 6.3.4.1 | Same as 6.3.4.1 Uplink OFF power measurement applies to each Tx antenna connector Uplink ON power measurement applies to [FFS] |
| 6.3.4.2 PRACH and SRS time mask | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |
| 6.3.5.1 Power Control Absolute power tolerance | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB 1.4 dB 1.4 dB | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | Same as 6.3.5.2 | Same as 6.3.5.2 | Same as 6.3.5.2 |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | TBD | Same as 6.3.5.1 | TBD |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 | Same as 6.3.5.1 | Same as 6.3.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.5.2 Power Control Relative power tolerance | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB | 0.7 dB | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |
| 6.3.5.3 Aggregate power control tolerance | Aggregate power control tolerance within 21 ms: PUCCH = ±2.5 dB PUSCH = ±3.5 dB | 0.7 dB | Formula: Upper limit + TT, Lower limit - TT PUCCH = ±3.2 dB PUSCH = ±4.2 dB |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | TBD | Same as 6.3.5.2 | TBD |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 | Same as 6.3.5.2 | Same as 6.3.5.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 | Same as 6.3.5.3 | Same as 6.3.5.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.1 Frequency Error | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens | 15 Hz 0.7 dB 1.0 dB | Formulae: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.1A.1 | Same as 6.5.1A.1 | Same as 6.5.1A.1 |
| 6.5.1A.2 Frequency Error for CA (inter-band DL CA and UL CA) | Same as 6.5.1A.1 per CC | Same as 6.5.1A.1 per CC | Same as 6.5.1A.1 per CC |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 | Same as 6.5.1 | Same as 6.5.1 |
| 6.5.2.1 Error Vector Magnitude | EVM limit: BPSK :17.5 % QPSK: 17.5 % 16QAM: 12.5 % | 0% | Formula: Minimum Requirement + TT |
| 6.5.2.1A PUSCH-EVM with exclusion period | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % | 0% | Formula: Minimum Requirement + TT |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.2.2 Carrier leakage | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB | Formula: Minimum Requirement + TT |
| 6.5.2.3 In-band emissions for non allocated RB | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) | 0.8dB | Formula: Minimum Requirement + TT |
| 6.5.2.4 EVM equalizer Spectrum flatness | Normal conditions. If (F-FUL_low ≥ [3MHz])&(FUL_high-F≥ [3MHz]) 4 dB else 8 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 5 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 7 dB Extreme conditions: If (F-FUL_low ≥ [5MHz])&(FUL_high-F≥ [5MHz]) 4 dB else 12 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 6 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 10 dB | 1.4dB | Formula: Minimum Requirement + TT |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | TBD | TBD | TBD |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | TBD | TBD | TBD |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD | TBD | TBD |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 | Same as 6.5.2.2 | Same as 6.5.2.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 | Same as 6.5.2.3 | Same as 6.5.2.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 | Same as 6.5.2.4 | Same as 6.5.2.4 |
| 6.6.1 Occupied bandwidth | For 1.4 MHz channel bandwidth: Occupied channel bandwidth = 1.4 MHz For 3.0 MHz channel bandwidth: Occupied channel bandwidth = 3.0 MHz For 5 MHz channel bandwidth: Occupied channel bandwidth = 5 MHz For 10 MHz channel bandwidth: Occupied channel bandwidth = 10 MHz For 15 MHz channel bandwidth: Occupied channel bandwidth = 15 MHz For 20 MHz channel bandwidth: Occupied channel bandwidth = 20 MHz | 0kHz | Formula: Minimum Requirement + TT |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | For 13 MHz channel bandwidth: Occupied channel bandwidth =[13] MHz For 15 MHz channel bandwidth: Occupied channel bandwidth =[15] MHz For 20 MHz channel bandwidth: Occupied channel bandwidth =[20] MHz For 35 MHz channel bandwidth: Occupied channel bandwidth =[35] MHz For 40 MHz channel bandwidth: Occupied channel bandwidth =[40] MHz | Same as 6.6.1 | Same as 6.6.1 |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 per CC | Same as 6.6.1 per CC | Same as 6.6.1 per CC |
| 6.6.1A.3 | Same as 6.6.1 for each CC | Same as 6.6.1 | Same as 6.6.1 for each CC |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 | Same as 6.6.1 | Same as 6.6.1 |
| 6.6.2.1 Spectrum Emission Mask | For 1.4 MHz BW: -10 dBm / 30kHz -25dBm to -10dBm / 1MHz For 3 MHz BW: -13 dBm / 30kHz -25dBm to -10dBm / 1MHz For 5 MHz BW: -15dBm / 30kHz -25dBm to -10dBm / 1MHz For 10 MHz BW: -18dBm / 30kHz -25dBm to -10dBm / 1MHz For 15 MHz BW: -20dBm / 30kHz -25dBm to -10dBm / 1MHz For 20 MHz BW: -21dBm / 30kHz -25dBm to -10dBm / 1MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | For 24.95 MHz BW: -22 dBm / 30kHz -25 dBm to - 10 dBm / 1 MHz For 29.9 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 30 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 34.85 MHz BW: -23.5dBm / 30kHz -25dBm to -10dBm / 1MHz For 39.8 MHz BW: -24dBm / 30kHz -25dBm to -10dBm / 1MHz | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 per CC | Same as 6.6.2.1 per CC | Same as 6.6.2.1 per CC |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 for each CC | Same as 6.6.2.1 | Same as 6.6.2.1 for each CC |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.2 Additional Spectrum Emission Mask | For 1.4 MHz BW: NS_03, NS_04 -10 dBm / 30 kHz -25 dBm to -13 dBm / 1MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1MHz For 3 MHz BW: NS_03, NS_04 -13 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100kHz -25 dBm to -13 dBm / 1 MHz For 5 MHz BW: NS_03, NS_04 -15 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -15 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1 MHz For 10 MHz BW: NS_03, NS_04, -18 dBm / 30 kHz -25 dBm to - 13dBm / 1 MHz NS_06 or NS_07 -18 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to - 13dBm / 1 MHz For 15 MHz BW: NS_03, NS_04 -20 dBm / 30kHz -25 dBm to -13 dBm / 1 MHz For 20 MHz BW: NS_03, NS_04 -21 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |
| 6.6.2.2A Additional Spectrum Emission Mask for CA | For network signalled value CA_NS_04: -21dBm/30kHz to -22.5dBm/kHz, BW-dependent, for 0-1MHz -13dBm/1MHz for 1MHz-5.5MHz -25dBm/1MHz beyond 5.5MHz | Same as 6.6.2.2 | Same as 6.6.2.2 |
| 6.6.2.2B Additional Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.2 | Same as 6.6.2.2 | Same as 6.6.2.2 |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30 dB UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 | 0 dB 0.8 dB 0.8 dB 0.8 dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2 dB UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 37 dB | 0dB 0.8dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 36.2 dB |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 E-UTRA ACLR: 30 dB | 0 dB 0.8 dB 0.8 dB 0.8 dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 E-UTRA ACLR: 29.2 dB |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.3 per component carrier | Same as 6.6.2.3 per component carrier | Same as 6.6.2.3 per component carrier |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.4 Additional ACLR requirements | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 43 dB for UTRA ACLR 2 | 0 dB 0.8 dB | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement – TT E-UTRA ACLR: 42.2 dB for UTRA ACLR 2 |
| 6.6.3.1 Transmitter Spurious emissions | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 19 GHz: -30dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |
| 6.6.3.1_1Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 | Same as 6.6.3.1 | Formula: Minimum Requirement + TT |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | TBD | Same as 6.6.3.1 | TBD |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1 | Same as 6.6.3.1 | Formula: Minimum Requirement + TT |
| 6.6.3.2 Spurious emission band UE co-existence | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | TBD | TBD | TBD |
| 6.6.3.3 Additional spurious emissions | NS_05 1884.5MHz f 1915.7MHz: -41dBm / 300kHz NS_07 769MHz ≤ f ≤ 775MHz -57dBm / 6.25kHz NS_08 860MHz ≤ f ≤ 895MHz -40dBm / 1MHz NS_09 1475.9MHz ≤ f ≤ 1510.9MHz -35dBm / 1MHz | 0 dB 1.5dB 0 dB 0 dB | Formula: Minimum Requirement + TT -41dBm / 300kHz -55.5 dBm / 6.25kHz -40dBm / 1MHz -35dBm / 1MHz |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | CA_NS_01 E-UTRA band 34: -50dBm / 1MHz 1884.5MHz f 1915.7MHz: -41dBm / 300kHz CA_NS_02 E-UTRA band 33: -50dBm / 1MHz E-UTRA band 34: -50dBm / 1MHz CA_NS_03 E-UTRA band 34: -50dBm / 1MHz E-UTRA band 39: -50dBm / 1MHz | 0 dB 0 dB 0 dB 0 dB 0 dB 0 dB | Formula: Minimum Requirement + TT -50 MHz / 1MHz -41 dBm / 300kHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO | Same as 6.6.3.2 | Same as 6.6.3.2 | Formula: Minimum Requirement + TT |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO | Same as 6.6.3.3 | Same as 6.6.3.3 | Formula: Minimum Requirement + TT |
| 6.7 Transmit intermodulation | Intermodulation Product 5MHz -29 dBc 10MHz -35 dBc CW Interferer level = -40 dBc | 0 dB | Formula: CW interferer Minimum Requirement– TT Intermod Products limits remain unchanged. CW interferer level = -40 dBc |
| 6.7A.1 Transmit intermodulation | Same as 6.7 | Same as 6.7 | Same as 6.7 |
| 6.7B Transmit intermodulation for UL-MIMO | Same as 6.7 | Same as 6.7 | Same as 6.7 |
| 6.8B Time alignment error for UL-MIMO | The Time Alignment Error (TAE) shall not exceed 130 ns | 25 ns | Formula: Minimum Requirement+ TT |

<end of changes >