3GPP TSG-RAN5 Meeting #72	R5-165161r1

# Gothenburg, Sweden, 22nd Aug 2016 - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2803 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NAICS RF TC 8.3.1.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NAICS-UEConTest |  |  |  |  |  | Date: |  |  | 2016-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | According to the workplan for NAICS, the TC 8.3.1.1.7 needs to be added to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new NAICS RF TC 8.3.1.1.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NAICS RF TC 8.3.1.1.7 will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1.7 (new), Annex F.1.4, Annex F.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1, CR R5-165197 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-165161 R5-165161r1 - Table 8.3.1.1.7.3-1 and Table 8.3.1.1.7.3-2 updated as per the proper tables from TS 36.101 - “+TT” notation added to some values in the test requirement tables. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 8.3.1.1.7	FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM10 serving cell configuration and TM9 interference model - Enhanced Performance Requirement Type B

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

- Many sections, including informations in AnnexF, are TBD

- Test Tolerances are undefined

###### 8.3.1.1.7.1	Test purpose

###### The purpose of this test is to verify closed loop rank one performance of the UE, on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission configured with TM10 in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.6.3.

###### 8.3.1.1.7.2	Test applicability

###### This test applies to all types of E-UTRA FDD UE release 12 and forward that support enhanced performance requirement Type B and PDSCH Transmission mode 10.

###### 8.3.1.1.7.3	Minimum conformance requirements

The requirements are specified in Table 8.3.1.1.7.3-2, with the addition of the parameters in Table 8.3.1.1.7.3-1 and the downlink physical channel setup according to Annex C.3.2.

The NAICS network assistance is provided when the serving cell TM10 is configured with QCL-type A and PCID based DM-RS scrambling.The neighbouring cell has transmission mode TM9 and NeighCellsInfo-r12 for interfering cell indicates presence of TM9. In 8.3.1.1.7.3-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.1.1.7.3-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM10 serving cell configuration and TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | 3 | 3 |
| PDSCH transmission mode |  |  |  | 10 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | Antenna ports 15, 16, 17, 18 | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 10 / 1 | 10 / 1 | 10 / 1 |
| CSI reference signal configuration |  |  |  | 5 | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 6 / 1000000000000000 | 6 / 0100000000000000 | 6 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to TS 36.211 [8] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of TS 36.331 [5]. |  |  |  |  |  |  |

Table 8.3.1.1.7.3-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM10 serving cell configuration and TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration |  |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.69 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x2 Low | 2x2 Low | 2x2 Low | 85 | 18.2 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The normative reference for these requirements is TS 36.101 [2] clause 8.3.1.1F.

###### 8.3.1.1.7.4	Test description

8.3.1.1.7.4.1	Initial conditions

TBD

8.3.1.1.7.4.2	Test procedure

TBD

8.3.1.1.7.4.3	Message contents

TBD

8.3.1.1.7.5	Test requirement

Table 8.3.1.1.7.5-1defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 for throughput test shall meet or exceed the specified value in Table 8.3.1.1.7.5-2 for the specified SNR including test tolerances.

Table 8.3.1.1.7.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM10 serving cell configuration and TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91+TT | 3.34+TT |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | 3 | 3 |
| PDSCH transmission mode |  |  |  | 10 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | Antenna ports 15, 16, 17, 18 | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 10 / 1 | 10 / 1 | 10 / 1 |
| CSI reference signal configuration |  |  |  | 5 | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 6 / 1000000000000000 | 6 / 0100000000000000 | 6 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to TS 36.211 [8] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of TS 36.331 [5]. |  |  |  |  |  |  |

Table 8.3.1.1.7.5-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM10 serving cell configuration and TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration |  |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.69 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x2 Low | 2x2 Low | 2x2 Low | 85 | 18.2 + TT | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## F.1.4	Measurement of performance requirements

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.1.1 Single PRB - Propagation Condition ETU70 | ± 0.8 dB | Overall system uncertainty for fading condition comprises three quantities: 1. Average Signal-to-noise ratio uncertainty 2. Signal-to noise ratio variation for single PRB 3. Fading profile power uncertainty Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx |
| 8.2.1.1.1_A.1 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_A.2 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  |
| 8.2.1.1.1_2 Multiple PRB - Propagation Condition EVA5 | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5 |  |
| 8.2.1.1.2 Single PRB | ± 0.8 dB | Same as 8.2.1.1.1 Single PRB |
| 8.2.1.2.1 - Propagation Condition EVA5 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.2.1 - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.2.1_1 - Propagation Condition EVA5 | Same as 8.2.1.2.1 Propagation EVA5 |  |
| 8.2.1.2.2 | ± 0.9 dB | Same as 8.2.1.2.1 Propagation Condition EVA5 |
| 8.2.1.2.2_1 | Same as 8.2.1.2.2 |  |
| 8.2.1.2.3_C.1 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc2 ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc2 ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.2.1.2.3_E.1 - Propagation Condition EVA5 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9dB Ês2/ Ês1 ± 1.1dB Ês3/ Ês1 ± 1.1dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0dB All symbols during non-ABS: Ês1/Noc3 ± 1.0dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.2.1.2.4 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.1.2.5 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.1.2.6 | Same as 8.2.1.2.5 | Same as 8.2.1.2.5 |
| 8.2.1.3.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.3.1_1 | Same as 8.2.1.3.1 | Same as 8.2.1.3.1 |
| 8.2.1.3.1_A.1 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.3.1_A.2 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.3.1A_A.1 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.3.1B | ± 0.9 dB | Same as 8.2.1.3.1 |
| 8.2.1.3.1C | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.1.3.2 | ± 0.9 dB | Same as 8.2.1.3.1 |
| 8.2.1.3.3_C.1 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc2 ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc2 ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.2.1.3.3_C.2 | Symbol #0 during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1..13 during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbol #0 during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1..13 during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc2 ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc2 ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.2.1.3.3_E.1 - Propagation Condition EVA5 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9dB Ês2/ Ês1 ± 1.1dB Ês3/ Ês1 ± 1.1dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0dB All symbols during non-ABS: Ês1/Noc3 ± 1.0dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.2.1.4.1 | ± 0.9 dB | Same as 8.2.1.3.1 |
| 8.2.1.4.1_1 | Same as 8.2.1.4.1 |  |
| 8.2.1.4.1_E.1 - Propagation Condition EPA5 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.2.1.4.1_H FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 | ± 0.9 dB | Same as 8.2.1.4.1 |
| 8.2.1.4.2 | ± 0.9 dB | Same as 8.2.1.3.1 |
| 8.2.1.4.2_1 | Same as 8.2.1.4.2 |  |
| 8.2.1.4.2_A.1 | Same as 8.2.1.4.2 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.4.2_A.2 | Same as 8.2.1.4.2 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.4.2_A.3 | Same as 8.2.1.4.2 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.4.2A | ± 0.9 dB | Same as 8.2.1.3.1 |
| 8.2.1.4.3 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.1.4.3A | Same as 8.2.1.4.2 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.4.4 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.1.7.1_A.1 | TBD | TBD |
| 8.2.2.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.2.2.1.1 Multiple PRBs - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.2.1.1 Single PRB - Propagation Condition ETU70 | ± 0.8 dB | Overall system uncertainty for fading condition comprises three quantities: 1. Average Signal-to-noise ratio uncertainty 2. Signal-to noise ratio variation for single PRB 3. Fading profile power uncertainty Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx |
| 8.2.2.1.1_1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  |
| 8.2.2.1.1_2 Multiple PRB - Propagation Condition EVA5 | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5 |  |
| 8.2.2.1.1_A.1 | Same as 8.2.2.1.1 Multiple PRBs for each CC | Same as 8.2.2.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.2.1.1_A.2 | Same as 8.2.2.1.1 Multiple PRBs for each CC | Same as 8.2.2.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.2.1.2 Single PRB | ± 0.8 dB | Same as 8.2.2.1.1 Single PRB |
| 8.2.2.2.1 - Propagation Condition EVA5 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |
| 8.2.2.2.1 - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.2.2.1_1 - Propagation Condition EVA5 | Same as 8.2.2.2.1 Propagation EVA5 |  |
| 8.2.2.2.2 | ± 0.9 dB | Same as 8.2.2.2.1 Propagation Condition EVA5 |
| 8.2.2.2.2_1 | Same as 8.2.2.2.2 |  |
| 8.2.2.2.3_C.1 | Same as 8.2.1.2.3_C.1 | Same as 8.2.1.2.3_C.1 |
| 8.2.2.2.3_E.1 - Propagation Condition EVA5 | Same as 8.2.1.2.3_E.1 | Same as 8.2.1.2.3_E.1 |
| 8.2.2.2.4 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.2.2.6 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.2.2.7 | Same as 8.2.2.2.6 | Same as 8.2.2.2.6 |
| 8.2.2.3.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |
| 8.2.2.3.1_1 | Same as 8.2.2.3.1 | Same as 8.2.2.3.1 |
| 8.2.2.3.1_A.1 | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.2.3.1_A.2 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3 DL CA) | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.2.3.1A_A.1 | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.2.3.1B | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.2.2.3.1C | Same as 8.2.1.3.1C | Same as 8.2.1.3.1C |
| 8.2.2.3.2 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.2.2.3.3_C.1 | Same as 8.2.1.3.3_C.1 | Same as 8.2.1.3.3_C.1 |
| 8.2.2.3.3_C.2 | Same as 8.2.1.3.3_C.2 | Same as 8.2.1.3.3_C.2 |
| 8.2.2.3.3_E.1 - Propagation Condition EVA5 | Same as 8.2.1.3.3_E.1 | Same as 8.2.1.3.3_E.1 |
| 8.2.2.4.1 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.2.2.4.1_1 | Same as 8.2.2.4.1 |  |
| 8.2.2.4.1_E.1 - Propagation Condition EPA5 | Same as 8.2.1.4.1_E.1 | Same as 8.2.1.4.1_E.1 |
| 8.2.2.4.1_H TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 | Same as 8.2.2.4.1 |  |
| 8.2.2.4.2 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.2.2.4.2_1 | Same as 8.2.2.4.2 |  |
| 8.2.2.4.2_A.1 | Same as 8.2.2.4.2 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.2.4.2_A.2 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | Same as 8.2.2.4.2 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.2.4.2A | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.2.2.4.3 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.2.4.4 | Same as 8.2.2.4.2 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.2.4.5 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.2.2.7.1_A.1 | TBD | TBD |
| 8.2.3.1.1.1 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.3.1.1.2 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.3.1.2.1 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.3.1.2.2 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.3.2.1.1 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.3.2.1.2 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.3.2.1A | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.3.2.2.1 | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.3.2.2.2 | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.3.2.2A | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.3.3.1.1 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.3.3.1.2 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.3.3.2.1 | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.2.3.3.2.2 | Same as 8.2.2.3.1 for each CC | Same as 8.2.2.3.1 Calculation applies for each CC |
| 8.3.1.1.1_D | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |
| 8.3.1.1.1_H FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 | Same as 8.3.1.1.1_D | Same as 8.3.1.1.1_D |
| 8.3.1.1.2_D | ± 0.9 dB | Same as 8.3.1.1.1_D |
| 8.3.1.1.3 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.3.1.1.7 | TBD | TBD |
| 8.3.1.2.1_D | ± 0.9 dB | Same as 8.3.1.1.1_D |
| 8.3.1.2.1_D_1 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.3.1.2.2 | ± 0.9 dB | Same as 8.3.1.1.1_D |
| 8.3.1.3.1_F | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |
| 8.3.1.3.2_F | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) | Same as 8.3.1.3.1_F |
| 8.3.1.3.3_F | Ês/Noc ± 0.9 dB for each TP | Same as 8.3.1.3.1_F |
| 8.3.2.1.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |
| 8.3.2.1.1_1 | ± 0.9 dB | Same as 8.3.2.1.1 |
| 8.3.2.1.2 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.1.2_D | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.1.2_H TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.1.3 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.1.3_D | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.1.4 | Ês/Noc ± 0.8 dB for each cell | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 8.3.2.2.1 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.2.1_D | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.2.1_D_1 | Ês/Noc ± 0.8 dB for each cell | Same as 8.3.1.2.1_D_1 |
| 8.3.2.2.2 | ± 0.9 dB | Same as 8.2.2.3.1 |
| 8.3.2.4.1_F | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB |
| 8.3.2.4.2_F | Ês/Noc ± 0.9 dB for each TP Time alignment error TP 2 relative to TP1: ±130 ns (±4Ts) | Same as 8.3.2.4.1_F |
| 8.3.2.4.3_F | Ês/Noc ± 0.9 dB for each TP | Same as 8.3.2.4.1_F |
| 8.4.1.1 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |
| 8.4.1.2.1 | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.4.1.2.1_1 | Same as 8.4.1.2.1 |  |
| 8.4.1.2.2 | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.4.1.2.2_1 | Same as 8.4.1.2.2 |  |
| 8.4.1.2.3_C.1 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.4.1.2.3_C.2 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.4.1.2.3_E.1 - Propagation Condition EVA5 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.4.1.2.3_E.2 - Propagation Condition EVA5 | Same as 8.4.1.2.3_E.1 | Same as 8.4.1.2.3_E.1 |
| 8.4.2.1 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |
| 8.4.2.2.1 | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.4.2.2.1_1 | Same as 8.4.2.2.1 |  |
| 8.4.2.2.2 | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.4.2.2.2_1 | Same as 8.4.2.2.2 |  |
| 8.4.2.2.3_C.1 | Same as 8.4.1.2.3_C.1 |  |
| 8.4.2.2.3_C.2 | Same as 8.4.1.2.3_C.2 |  |
| 8.4.2.2.3_E.1 - Propagation Condition EVA5 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.4.2.2.3_E.2 - Propagation Condition EVA5 | Same as 8.4.2.2.3_E.1 | Same as 8.4.2.2.3_E.1 |
| 8.5.1.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.5.1.2.1 | ± 1.1 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.6 dB |
| 8.5.1.2.1_1 | Same as 8.5.1.2.1 |  |
| 8.5.1.2.2 | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.5.1.2.2_1 | Same as 8.5.1.2.2 |  |
| 8.5.1.2.3_C.1 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.5.1.2.3_E.1 | Symbols #0, 4.. during ABS: Ês1/Noc2 ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB Symbols #1, 2.. during ABS: Ês1/Noc1 ± 1.0 dB All symbols during non-ABS: Ês1/Noc3 ± 1.0 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Symbols #0, 4.. means 0, 4, 7, 11 Symbols #1, 2.. means 1-3, 5, 6, 8-10, 12, 13 Symbols #0, 4.. during ABS: Ês1/Noc2 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 3 SNR uncertainty 2 + Cell 3 Fading profile power uncertainty 2) Symbols #1, 2.. during ABS: Ês1/Noc1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc1/Noc2 uncertainty 2) All symbols during non-ABS: Ês1/Noc3 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + Noc3/Noc2 uncertainty 2) Component uncertainties: SNR uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Noc1/Noc2 and Noc3/Noc2 uncertainty ±0.3 dB |
| 8.5.2.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.5.2.2.1 | ± 1.1 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.6 dB |
| 8.5.1.2.2_1 | Same as 8.5.2.2.1 |  |
| 8.5.2.2.2 | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.5.2.2.2_1 | Same as 8.5.2.2.2 |  |
| 8.5.2.2.3_C.1 | Same as 8.5.1.2.3_C.1 |  |
| 8.5.2.2.3_E.1 | Same as 8.5.1.2.3_E.1 |  |
| 8.7.1.1 | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 8.7.1.1_1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.1.1_A.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_A.2 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.1.1_H.2 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.3 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.2.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.2.1_1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.2.1_A.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_A.2 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.2.1_H.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.3.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.4.1 | Same as 8.7.3.1 | Same as 8.7.3.1 |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.8.1.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |
| 8.8.1.2 | Same as 8.8.1.1 | Same as 8.8.1.1 |
| 8.8.2.1 | Same as 8.8.1.1 | Same as 8.8.1.1 |
| 8.8.2.2 | Same as 8.8.1.1 | Same as 8.8.1.1 |
| 8.8.3.1 | Same as 8.8.1.1 | Same as 8.8.1.1 |
| 8.8.3.2 | Same as 8.8.1.1 | Same as 8.8.1.1 |
| 8.9.1.1.1 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.9.1.1.2 | Same as 8.9.1.1.1 | Same as 8.9.1.1.1 |
| 8.9.1.1.3 | Same as 8.9.1.1.1 | Same as 8.9.1.1.1 |
| 8.9.1.2.1 | Same as 8.9.1.1.1 | Same as 8.9.1.1.1 |
| 8.9.1.2.2 | Same as 8.9.1.1.1 | Same as 8.9.1.1.1 |
| 8.9.1.2.3 | Same as 8.9.1.1.1 | Same as 8.9.1.1.1 |
| 8.9.2.2.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.9.2.1.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |
| 8.10.4.1.1 | TBD | TBD |
| 8.10.4.1.2 | TBD | TBD |
| 8.10.4.2.1 | TBD | TBD |
| 8.10.4.2.2 | TBD | TBD |
| 10.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |
| 10.2 | ± 0.9 dB | Same as 10.1 |
| [Other tests FFS] |  |  |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |
| AWGN Bandwidth |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  | ±3 dB |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  | ±0.5 dB |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  | ±3 dB |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Fading profile power uncertainty |  | Test-specific |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |
| CA performance requirements only: Relative frequency error between carriers |  | 30Hz, measured over a 1ms period, and maximum carrier spacing 80MHz |
| enhanced performance requirement Type B only: Frequency error on the frequency offset to cell 1 |  | 30Hz, measured over a 1ms period |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN or N ocx parameters apply to all test cases except 8.7.1 and 8.7.2. The fading parameters apply to test cases using fading Note 3: In CA test cases using multiple component carriers (CCs), the uncertainties and related constraints apply for each CC. Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |

< Unchanged sections omitted >

## F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition HST | SNR as specified | 0.6dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1 Single PRB - Prop’n Condition ETU70 | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1_1 Multiple PRBs - Prop'n Condition EVA5 - Prop'n Condition ETU70 - Prop'n Condition ETU300 | SNRs as specified | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1_2 Multiple PRB - Prop'n Condition EVA5 | SNRs as specified | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1_A.1 | SNRs as specified | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1_A.2 | SNRs as specified | Same as 8.2.1.1.1 Multiple PRBs for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.2 Single PRB | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.2.1 - Prop’n Condition EVA5 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.2.1 - Prop’n Condition HST | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.2.1_1 - Prop'n Condition EVA5 | SNR as specified | Same as 8.2.1.2.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.2.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.2.2_1 | SNR as specified | Same as 8.2.1.2.2 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.2.3_C.1 | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +3.4dB Ês2 / Noc2: +6dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +4.3dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.1.2.3_E.1 - Prop’n Condition EVA5 | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Ês1 / Noc2: +3.4dB Ês2 / Noc2: +12dB Ês3 / Noc2: +10dB | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: +4.3dB Ês2 / Noc2: +11.8dB Ês3 / Noc2: +9.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |
| 8.2.1.2.4 | Noc: -98dBm/15kHz DIP 1: -2.23dB DIP 1: -8.06dB SINR: -1.10dB | 0dB +0.35dB +0.85dB +0.95dB | Noc: -98dBm/15kHz DIP 1: -1.88dB DIP 1: -7.21dB SINR: -0.15dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.1.2.5 | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.5 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.85 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.35 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.1.2.6 | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +8.4 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.54 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.94 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.1.3.1 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1_1 | SNR as specified | Same as 8.2.1.3.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1_A.1 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1_A.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1A_A.1 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1B | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.1C | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.9 dB Cell 2 Ês/Noc: +12.95 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.9 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +21.8 dB Cell 2 Ês/Noc: +13.75 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.1.3.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.3.3_C.1 | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +13.3dB Ês2 / Noc2: +6dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +14.2dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.2.1.3.3_C.2 | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +12.0dB Ês2 / Noc2: +6dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +12.9dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.2.1.3.3_E.1 - Prop’n Condition EVA5 | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Test 1 Ês1 / Noc2: + 13.9dB Ês2 / Noc2: + 9dB Ês3 / Noc2: + 7dB Test 2 Ês1 / Noc2: +22.6dB Ês2 / Noc2: +9dB +1dB Ês3 / Noc2: | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB +0.9dB -0.2dB -0.2dB | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: - 93.1 dBm/15kHz Ês1 / Noc2: + 14.8 dB Ês2 / Noc2: + 8.8dB Ês3 / Noc2: + 6.8 dB Ês1 / Noc2: +23.5dB Ês2 / Noc2: +8.8dB Ês3 / Noc2: +0.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |
| 8.2.1.4.1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.1_1 | SNR as specified | Same as 8.2.1.4.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.1_E.1 - Prop’n Condition EPA5 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +6.1 dB Ês2 / Noc2: +12 dB Ês3 / Noc2: +10 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +7.0 dB Ês2 / Noc2: +11.8 dB Ês3 / Noc2: +9.8 dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |
| 8.2.1.4.1_H FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 | SNR as specified | Same as 8.2.1.4.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.2 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.2_1 | SNR as specified | Same as 8.2.1.4.2 | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.2_A.1 | SNR as specified | Same as 8.2.1.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.2_A.2 | SNR as specified | Same as 8.2.1.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.2_A.3 | SNR as specified | Same as 8.2.1.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.2A | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.3 | Noc: -98dBm/15kHz DIP 1: -1.73dB DIP 1: -8.66dB SINR: 0.8dB | 0dB +0.29dB +0.89dB +0.99dB | Noc: -98dBm/15kHz DIP 1: -1.44dB DIP 1: -7.77dB SINR: 1.79dB T-put limit unchanged |
| 8.2.1.4.3A | SNR as specified | Same as 8.2.1.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.4.4 | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.0 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +10.1 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB | Test 1: 0 dB +1.85 dB +0.8 dB +0.8 dB Test 2: 0 dB +1.54 dB +0.8 dB +0.8 dB | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +18.85 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.64 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.1.7.1_A.1 | TBD | TBD | TBD |
| 8.2.2.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.1 Multiple PRBs - Prop’n Condition HST | SNR as specified | 0.6dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.1 Single PRB - Prop’n Condition ETU70 | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.1_1 Multiple PRBs - Prop'n Condition EVA5 - Prop'n Condition ETU70 - Prop'n Condition ETU300 | SNRs as specified | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.1_2 Multiple PRB - Prop'n Condition EVA5 | SNRs as specified | Same as 8.2.2.1.1 Multiple PRBs Propagation EVA5 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.1_A.1 | SNRs as specified | 8.2.2.1.1 Multiple PRBs for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.1_A.2 | SNRs as specified | 8.2.2.1.1 Multiple PRBs for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.1.2 Single PRB | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.2.1 - Prop’n Condition EVA5 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.2.1 - Prop’n Condition HST | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.2.1_1 - Prop'n Condition EVA5 | SNR as specified | Same as 8.2.2.2.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.2.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.2.2_1 | SNR as specified | Same as 8.2.2.2.2 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.2.3_C.1 | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +3.8dB Ês2 / Noc2: +6dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +4.7dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.2.3_E.1 - Prop'n Condition EVA5 | Noc1: [-98dBm/15kHz] Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Ês1 / Noc2: +3.5dB Ês2 / Noc2: +12dB Ês3 / Noc2: +10dB | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: +4.4dB Ês2 / Noc2: +11.8dB Ês3 / Noc2: +9.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |
| 8.2.2.2.4 | Noc: -98dBm/15kHz DIP 1: -1.73dB DIP 2: -8.66dB SINR: -1.4dB | 0 dB +0.29dB +0.89dB +0.99dB | Noc: -98dBm/15kHz DIP1: -1.44dB DIP2: -7.77 SINR: -0.41dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.2.6 | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.3 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.85 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.15 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.2.7 | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +8.1 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.54 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.64 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.3.1 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1_1 | SNR as specified | Same as 8.2.2.3.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1_2 | SNR as specified | Same as 8.2.2.3.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1_A.1 | SNRs as specified | Same as 8.2.2.3.1 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1_A.2 | SNRs as specified | Same as 8.2.2.3.1 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1A_A.1 | SNRs as specified | Same as 8.2.2.3.1 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1B | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.1C | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.6 dB Cell 2 Ês/Noc: +12.95 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.9 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +21.5 dB Cell 2 Ês/Noc: +13.75 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.3.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.3.3_C.1 | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +14.0dB Ês2 / Noc2: +6dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +14.9dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.2.2.3.3_C.2 | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +12.2dB Ês2 / Noc2: +6dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +13.1dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.2.2.3.3_E.1 - Prop’n Condition EVA5 | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Test 1 Ês1 / Noc2: + 14.2 dB Ês2 / Noc2: + 9 dB Ês3 / Noc2: + 7 dB Test 2 Ês1 / Noc2: +22.7dB Ês2 / Noc2: +9dB Ês3 / Noc2: +1dB | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB +0.9dB -0.2dB -0.2dB | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: + 15.1 dB Ês2 / Noc2: + 8.8 dB Ês3 / Noc2: + 6.8 dB Ês1 / Noc2: +23.6dB Ês2 / Noc2: +8.8dB Ês3 / Noc2: +0.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |
| 8.2.2.4.1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.1_1 | SNR as specified | Same as 8.2.2.4.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.1_E.1 - Prop'n Condition EPA5 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +6.4 dB Ês2 / Noc2: +12 dB Ês3 / Noc2: +10 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +7.3 dB Ês2 / Noc2: +11.8 dB Ês3 / Noc2: +9.8 dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |
| 8.2.2.4.1_H TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for SCE-L1 | SNR as specified | Same as 8.2.2.4.1 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.2 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.2_1 | SNR as specified | Same as 8.2.2.4.2 | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.2_A.1 | SNR as specified | Same as 8.2.2.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.2_A.2 | SNR as specified | Same as 8.2.2.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.2A | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.3 | Noc: -98dBm/15kHz DIP 1: -1.73dB DIP 2: -8.66dB SINR: 1.1dB | 0 dB +0.29dB +0.89dB +0.99dB | Noc: -98dBm/15kHz DIP1: -1.44dB DIP2: -7.77 SINR: 2.09dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.4.4 | SNR as specified | Same as 8.2.2.4.2 for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.2.2.4.5 | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.1 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.5 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB | Test 1: 0 dB +1.85 dB +0.8 dB +0.8 dB Test 2: 0 dB +1.54 dB +0.8 dB +0.8 dB | Test 1: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.95 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged Test 2: Noc: -98dBm/15kHz Cell 1 Ês/Noc: +11.04 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.2.2.7.1_A.1 | TBD | TBD | TBD |
| 8.2.3.1.1.1 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.1.1.2 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.1.2.1 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.1.2.2 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.2.1.1 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.2.1.2 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.2.1A | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.2.2.1 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.2.2.2 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.2.2A | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.3.1.1 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.3.1.2 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.3.2.1 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3.3.2.2 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.1_D | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.1_H FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.2_D | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.3 | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.10dB | 0 dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -0.12dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.3.1.1.7 | TBD | TBD | TBD |
| 8.3.1.2.1_D | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.2.1_D_1 | Noc: -98dBm/15kHz Ês1 / Noc: +14.2 dB Ês2 / Noc: +7.25 dB | 0 dB +0.8 dB -0.35 dB | Noc: -98dBm/15kHz Ês1 / Noc: +15.0 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.3.1.2.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.3.1_F | SNR as specified Test 1: 2us Test 2: -0.5us | 0.9 dB -0.145us +0.142us | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset – uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |
| 8.3.1.3.2_F | SNR as specified Test 1: 2us Test 2: -0.5us | 0.9 dB -0.145us +0.142us | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset – uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |
| 8.3.1.3.3_F | SNR as specified Test 1: 2us Test 2: -0.5us | 0.9 dB -0.145us +0.142us | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset - uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |
| 8.3.2.1.1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.1_1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.2 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.2_D | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.2_H TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for SCE-L1 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.3 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.3_D | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.1.4 | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.0dB | 0 dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -0.02dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.3.2.2.1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.2.1_D | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.2.1_D_1 | Noc: -98dBm/15kHz Ês1 / Noc: +14.8 dB Ês2 / Noc: +7.25 dB | 0 dB +0.8 dB -0.35 dB | Noc: -98dBm/15kHz Ês1 / Noc: +15.6 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.3.2.2.2 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.2.4.1_F | SNR as specified Test 1: 2us Test 2: -0.5us | 0.9 dB -0.145us +0.142us | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset – uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |
| 8.3.2.4.2_F | SNR as specified Test 1: 2us Test 2: -0.5us | 0.9 dB -0.145us +0.142us | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset - uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |
| 8.3.2.4.3_F | SNR as specified Test 1: 2us Test 2: -0.5us | 0.9 dB -0.145us +0.142us | Formula: SNR + TT Test 1: 57Ts Test 2: -11Ts Timings: (Offset - uncertainty), then rounded to integer multiple of Ts T-put limit unchanged |
| 8.4.1.1 | SNR as specified | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.4.1.2.1 | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.4.1.2.1_1 | SNR as specified | Same as 8.4.1.2.1 | Formula: SNR + TT T-put limit unchanged |
| 8.4.1.2.2 | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.4.1.2.2_1 | SNR as specified | Same as 8.4.1.2.2 | Formula: SNR + TT T-put limit unchanged |
| 8.4.1.2.3_C.1 | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -3.9dB Ês2 / Noc2: +1.5dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.4.1.2.3_C.2 | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -4.2dB Ês2 / Noc2: +1.5dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3.3dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.4.1.2.3_E.1 - Prop'n Condition EVA5 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -2.2 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -1.3 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |
| 8.4.1.2.3_E.2 - Prop'n Condition EVA5 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -2 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -1.1 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |
| 8.4.2.1 | SNR as specified | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.4.2.2.1 | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.4.2.2.1_1 | SNR as specified | Same as 8.4.2.2.1 | Formula: SNR + TT T-put limit unchanged |
| 8.4.2.2.2 | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.4.2.2.2_1 | SNR as specified | Same as 8.4.2.2.2 | Formula: SNR + TT T-put limit unchanged |
| 8.4.2.2.3_C.1 | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -3.9dB Ês2 / Noc2: +1.5dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.4.2.2.3_C.2 | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: -4.1dB Ês2 / Noc2: +1.5dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: -3.2dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.4.2.2.3_E.1 - Prop'n Condition EVA5 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -.2 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -1.1 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |
| 8.4.2.2.3_E.2 - Prop'n Condition EVA5 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: -1.8 dB: Ês2 / Noc2 +5 dB Ês3 / Noc2: +3 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: -0.9 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-dsg limit unchanged |
| 8.5.1.1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.5.1.2.1 | SNR as specified | 1.1 dB | Formula: SNR + TT T-put limit unchanged |
| 8.5.1.2.1_1 | SNR as specified | Same as 8.5.1.2.1 | Formula: SNR + TT T-put limit unchanged |
| 8.5.1.2.2 | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.5.1.2.3_C.1 | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: +4.6dB Ês2 / Noc2: +1.5dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: +5.5dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.5.1.2.3_E.1 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +5 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +5.9 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-an limit unchanged |
| 8.5.1.2.2_1 | SNR as specified | Same as 8.5.1.2.2 | Formula: SNR + TT T-put limit unchanged |
| 8.5.2.1 | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.5.2.2.1 | SNR as specified | 1.1 dB | Formula: SNR + TT T-put limit unchanged |
| 8.5.2.2.1_1 | SNR as specified | Same as 8.5.2.2.1 | Formula: SNR + TT T-put limit unchanged |
| 8.5.2.2.2 | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.5.2.2.2_1 | SNR as specified | Same as 8.5.2.2.2 | Formula: SNR + TT T-put limit unchanged |
| 8.5.2.2.3_C.1 | Noc1: -100.5dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.3dBm/15kHz Ês1 / Noc2: +4.6dB Ês2 / Noc2: +1.5dB | -0.1dB 0dB -0.1dB +0.9dB -0.2dB | Noc1: -100.6dBm/15kHz Noc2: -98dBm/15kHz Noc3: -95.4dBm/15kHz Ês1 / Noc2: +5.5dB Ês2 / Noc2: +1.3dB Formula for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |
| 8.5.2.2.3_E.1 | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93 dBm/15kHz Ês1 / Noc2: +5.7 dB Ês2 / Noc2: +5 dB Ês3 / Noc2: +3 dB | 0 dB 0 dB -0.1 dB +0.9 dB -0.2 dB -0.2 dB | Noc1: -98 dBm/15kHz Noc2: -98 dBm/15kHz Noc3: -93.1 dBm/15kHz Ês1 / Noc2: +6.6 dB Ês2 / Noc2: +4.8 dB Ês3 / Noc2: +2.8 dB The analysis is recorded in 3GPP TR 36.904 [17] Pm-an limit unchanged |
| 8.7.1.1 FDD sustained data rate performance | Downlink power -85dBm/15kHz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_1 | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.1 | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.2 | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.1 | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.2 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.3 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1 TDD sustained data rate performance | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.2.1_1 | Same as 8.7.2.1 | Same as 8.7.2.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_A.1 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_A.2 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.1 | Same as 8.7.2.1 | Same as 8.7.2.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.2 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.3 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.3.1 FDD sustained data rate performance for EPDCCH scheduling | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.4.1 TDD sustained data rate performance for EPDCCH scheduling | Same as 8.7.2.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.2.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.2.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.8.1.1 FDD distributed EPDCCH performance | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.1.2 TDD distributed EPDCCH performance | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.2.1 FDD localized EPDCCH performance with TM9 | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.2.2 TDD localized EPDCCH performance with TM9 | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.3.1 FDD localized EPDCCH transmission with TM10 Type B quasi co-location type | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.3.2 TDD localized EPDCCH transmission with TM10 Type B quasi co-location type | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.9.1.1.1 FDD PDSCH Transmit Diversity 2x1 for UE category 0 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.2 FDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.3 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.1 TDD PDSCH Transmit Diversity for UE category 0 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.2 TDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.2.1.1 FDD PHICH transmit diversity for UE category 0 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.2.2.1 TDD PHICH Transmit Diversity for UE category 0 | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.4.1.1 | SNRs as specified | TBD | TBD |
| 8.10.4.1.2 | SNR as specified | TBD | TBD |
| 8.10.4.2.1 | SNRs as specified | TBD | TBD |
| 8.10.4.2.2 | SNRs as specified | TBD | TBD |
| 10.1 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 10.2 | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| [Other tests FFS] |  |  |  |

< End of changes >