3GPP TSG-RAN5 Meeting #77	R5-176778r1

# Reno, Nevada, United States, 27th Nov 2017 - 1st Dec 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4347 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Small update of CIoT TC 23.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sierra Wireless, S.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2017-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | R5-174488 agreed in Berlin was not implemented. It is reapplied here with: A step number correction before 17a6, One typo fixed in table 23.1.1.3.3-3 Values in table 23.1.1.3.3-4 are updated as they were missing in original CR R5-174488 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Approved CR reapplied with small eroor fixed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An approved CR would not be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Update of on top of R5-14488 are highlited |  |  |  |  |  |  |  |  |

### 23.1.1	CIoT / Control Plane MO and MT IP and non-IP Data Transfer / Serving PLMN Rate Control / APN Rate Control

23.1.1.1	Test Purpose (TP)

(1)

with { the UE in RRC-IDLE after attaching with Control Plane CIoT EPS optimisation for EPS services }

ensure that {

when { UE has user data to send via the control plane }

then { the UE initiates a Service request procedure for EPS services with Control Plane CIoT EPS optimization by sending a CONTROL PLANE SERVICE REQUEST message (rules for ciphering of the initial NAS message apply )

when { UE has already a PDN established }

then { the UE incorporates ESM DATA TRANSPORT message carrying the user data in the CONTROL PLANE SERVICE REQUEST message (rules for ciphering of the initial NAS message apply) }

}

(2)

with { the UE in RRC-IDLE after attaching with Control Plane CIoT EPS optimisation for EPS services }

ensure that {

when { UE, while in RRC-CONNECTED, has user data to send via the control plane }

then { the UE sends one or more ESM DATA TRANSPORT message(s) including the user data to be sent in the User data container IE }

}

(3)

with { the UE in RRC-IDLE after attaching with Control Plane CIoT EPS optimisation for EPS services }

ensure that {

when { UE performs Control Plane data transfer whenever this is needed }

then { the UE obeys the local serving PLMN rate control, and, if supported the APN Rate Control and the maximum length of user data container that can be sent in the ESM DATA TRANSPORT message (set by the MTU parameter) through EPS bearer context modification }

}

(4)

with { the UE in RRC-IDLE after attaching with Control Plane CIoT EPS optimisation for EPS services }

ensure that {

when { the Network has user data to send via the control plane, and, has executed a Paging for EPS services procedure }

then { the UE successfully completes RRC connection establishment together with Service request procedure for EPS services with Control Plane CIoT EPS optimization }

when { UE has a PDN established }

then { is able to receive user data sent via ESM DATA TRANSPORT message }

}

23.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 and TS 24.301, unless otherwise stated these are Rel-13 requirements.

[TS 36.331, clause 5.3.3]

1>	set the content of RRCConnectionSetupComplete message as follow

...

2>	if the UE supports CIoT EPS optimisation(s):

3>	include attachWithoutPDN-Connectivity if received from upper layers;

3>	include up-CIoT-EPS-Optimisation if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-EPS-Optimisation if received from upper layers;

[TS 24.301, clause 4.2]

A UE using EPS services with control plane CIoT EPS optimization can initiate transport of user data via the control plane. For this purpose a UE in EMM-IDLE mode can initiate the service request procedure and transmit the ESM DATA TRANSPORT message in an information element in the CONTROL PLANE SERVICE REQUEST message.

[TS 24.301, clause 5.3.15]

If the UE indicates support of one or more CIoT EPS optimizations and the network supports one or more CIoT EPS optimizations and decides to accept the attach or tracking area update request, the network indicates the supported CIoT EPS optimizations to the UE per TAI list when accepting the UE request. Network indication of support is interpreted by the UE as the acceptance to use the respective feature. After completion of the attach or tracking area updating procedure, the UE and the network can then use the accepted CIoT EPS optimizations for the transfer of user data (IP, non-IP and SMS).

Broadcast system information may provide information about support of CIoT EPS optimizations (see 3GPP TS 36.331 [22]). At reception of new broadcast system information, the lower layers deliver it to the EMM layer in the UE. The information provided by lower layers is per PLMN and used by the UE to determine whether certain CIoT EPS optimizations are supported in the cell.

The UE shall not attempt to use CIoT EPS optimizations which are indicated as not supported.

In WB-S1 mode, when the UE requests the lower layer to establish a RRC connection and the UE requests the use of EMM-REGISTERED without PDN connection, control plane CIoT EPS optimization or user plane CIoT EPS optimization, the UE shall pass an indication of the requested CIoT EPS optimizations to the lower layers.

[TS 24.301, clause 5.5.1]

If the UE supports NB-S1 mode or Non-IP PDN type, then the UE shall support the extended protocol configuration options IE.

If the UE supports the extended protocol configuration options IE, then the UE shall set the ePCO bit to "extended protocol configuration options supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports CIoT EPS optimizations, it shall indicate in the UE network capability IE of the ATTACH REQUEST message whether it supports EMM-REGISTERED without PDN connection.

If EMM-REGISTERED without PDN connection is not supported by the UE or the MME, or if the UE wants to request PDN connection with the attach procedure, the UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container IE.

[TS 24.301, clause 5.6.1.2.1]

The UE shall send a CONTROL PLANE SERVICE REQUEST message, start T3417 and enter the state EMM-SERVICE-REQUEST-INITIATED.

For case a in subclause 5.6.1.1, the Control plane service type of the CONTROL PLANE SERVICE REQUEST message shall indicate "mobile terminating request". The UE may include the ESM DATA TRANSPORT message. The UE shall not include any ESM message other than ESM DATA TRANSPORT message.

For case b in subclause 5.6.1.1,

-	if the UE has pending IP or non-IP user data that is to be sent via the control plane radio bearers, the Control plane service type of the CONTROL PLANE SERVICE REQUEST message shall indicate "mobile originating request". The UE shall include an ESM DATA TRANSPORT message in the ESM message container IE.

For cases b and m in subclause 5.6.1.1,

-	if the UE has pending IP or non-IP user data that is to be sent via the user plane radio bearers, the UE shall set the Control plane service type of the CONTROL PLANE SERVICE REQUEST message to "mobile originating request" and the "active" flag in the Control plane service type IE to 1. The UE shall not include any ESM message container or NAS message container IE in the CONTROL PLANE SERVICE REQUEST message.

[TS 24.301, clause 5.6.2.2.1.1]

Upon reception of a paging indication, if control plane CIoT EPS optimization is used by the UE, the UE shall stop the timer T3346, if running, and shall additionally:

-	initiate a service request procedure as specified in subclause 5.6.1.2.2 if the UE is in the EMM-IDLE mode without suspend indication; or

-	proceed the behaviour as specified in subclause 5.3.1.3 if the UE is in the EMM-IDLE mode with suspend indication.

NOTE 2:	If the UE is in the EMM-IDLE mode without suspend indication and has an uplink user data to be sent to the network using control plane CIoT EPS optimization when receiving the paging indication, the UE can piggyback the uplink user data during the service request procedure initiated to respond to the paging, as specified in subclause 5.6.1.2.2.

[TS 24.301, clause 6.2A]

The UE and the MME may support robust header compression (ROHC) framework (see IETF RFC 4495 [37]) for IP header compression if control plane CIoT EPS optimization is supported for PDN connections of IP PDN type. If IP header compression for control plane CIoT EPS optimization is supported, the ROHC profiles defined in 3GPP TS 36.323 [38] may be supported. The ROHC configuration is negotiated and established during the UE requested PDN connectivity procedure as specified in subclause 6.5.1. Both the UE and the MME indicate whether IP header compression for control plane CIoT EPS optimization is supported during attach and tracking area updating procedures (see subclauses 5.5.1 and 5.5.3). The ROHC configuration can be re-negotiated by using the UE requested bearer resource modification procedure or the EPS bearer context modification procedure as specified in subclauses 6.4.3 and 6.5.4.

[TS 24.301, clause 6.3.8]

Serving PLMN rate control is applicable for PDN connections established for control plane CIoT EPS optimization only.

[TS 24.301, clause 6.3.9]

APN rate control controls the maximum number of uplink user data messages sent by the UE in a time interval for the APN in accordance with 3GPP TS 23.401 [10]. The UE shall limit the rate at which it generates uplink user data messages to comply with the APN rate control policy. The NAS shall provide the indicated rates to upper layers for enforcement. The indicated rate in a NAS procedure applies to the APN the NAS procedure corresponds to, and the indicated rate is valid until a new value is indicated or the last PDN connection using this APN is released.

If an indication of additional exception reports is provided with the APN rate control parameters, the UE is allowed to send uplink exception reports even if the limit for the APN rate control has been reached.

NOTE:	The HPLMN can discard or delay user data that exceeds the limit provided for APN rate control.

[TS 24.301, clause 6.5.1.2]

When the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message, the UE shall not start timer T3482 and shall not include the APN.

NOTE 1:	If the UE needs to provide protocol configuration options which require ciphering or provide an APN, or both, during the attach procedure, the ESM information transfer flag is included in the PDN CONNECTIVITY REQUEST. The MME then at a later stage in the PDN connectivity procedure initiates the ESM information request procedure in which the UE can provide the MME with protocol configuration options or APN or both.

In the PDN type IE the UE shall either indicate the IP version capability of the IP stack associated with the UE or non IP as specified in subclause 6.2.2.

If the PDN type value of the PDN type IE is set to IPv4 or IPv6 or IPv4v6 and the UE indicates "Control plane CIoT EPS optimization supported" in the UE network capability IE of the ATTACH REQUEST message, the UE may include the Header compression configuration IE in the PDN CONNECTIVITY REQUEST message.

[TS 24.301, clause 6.6.4.2]

Upon receipt of a request to transfer user data via the control plane, if the UE is in EMM-CONNECTED mode, the UE initiates the procedure by sending the ESM DATA TRANSPORT message including the user data to be sent in the User data container IE. The length of the value part of the User data container IE should not exceed the link MTU size for the respective type of user data (IPv4, IPv6 or Non-IP).

NOTE:	The recommended maximum size for link MTU is 1358 octets to prevent fragmentation in the backbone network (see 3GPP TS 23.060 [74]). Depending on the network configuration, setting link MTU size to a value larger than 1358 octets could lead to inefficient core network implementation due to fragmentation.

If the UE is in EMM-IDLE mode, the UE initiates the procedure by sending the ESM DATA TRANSPORT message included in a CONTROL PLANE SERVICE REQUEST message.

23.1.1.3	Test description

23.1.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, default system information combination.

UE:

None.

Preamble:

-	UE is in State 3A-CP, Control Plane CIoT connection request, UE Test Mode Activated (Test Loop G) according to TS 36.508 [18].

##### 23.1.1.3.2	Test procedure sequence

Table 23.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | NOTE: Settings in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST, in preamble, to check PLMN rate control: - PLMN Rate control set to max of 10 messages per 6 minutes; - APN control not provided; - MTU parameters not provided. |  |  |  |  |
| 1 | The SS transmits a CLOSE UE TEST LOOP message to close the UE test loop mode for user data transfer (12 transmission; 60 sec delay). | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 3 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 3 octets. | <-- | RRC: ULInformationTransfer ESM DATA TRANSPORT | - | - |
| 4 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease | - | - |
| 5 | Wait for the time set in the CLOSE UE TEST LOOP to expire. | - | - | - | - |
| 6 | SS starts timer 6 min PLMN Rate control | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRC: RRCConnectionRequest | - | - |
| 8 | SS transmits an RRCConnectionSetup-message. | <-- | RRC: RRCConnectionSetup | - | - |
| 9 | Check: Does the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the CONTROL PLANE SERVICE REQUEST message and an ESM DATA TRANSPORT message containing the same user data sent by the SS in step 3? | --> | RRC: RRCConnectionSetupComplete NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 1 | P |
| 10 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: SERVICE ACCEPT |  |  |
| 11 | Check: Does the UE send an ESM DATA TRANSPORT message containing the same user data sent by the SS in step 3? | --> | RRC: ULInformationTransfer TC: ESM DATA TRANSPORT | 2 | P |
| - | EXCEPTION: Step 12 is repeated 8 times. Note: The number of messages is set so that together with the message sent in step 11 it respects the PLMN data rate set in the latest ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | - | - | - | - |
| 12 | Check: Does the UE send an ESM DATA TRANSPORT message containing the same user data sent by the SS in step 3? | --> | RRC: ULInformationTransfer TC: ESM DATA TRANSPORT | 2,3,4 | P |
| 13 | Wait until 6 min timer Expires | - | - | - | - |
| 14 | SS starts timer 6 min PLMN Rate control | - | - | - | - |
| - | EXCEPTION: Step 15 is repeated 2 times. Note: The number of messages is set so that it respects the PLMN data rate set in the latest ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | - | - | - | - |
| 15 | Check: Does the UE send an ESM DATA TRANSPORT message containing the same user data sent by the SS in step 3? | --> | RRC: ULInformationTransfer TC: ESM DATA TRANSPORT | 2, 3 | P |
| 16 | SS stops timer 6 min PLMN Rate control | - | - | - | - |
| - | EXCEPTION: Steps 17a1 to 17a8 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports APN control. | - | - | - | - |
| 17a1 | IF pc_APN_RateControl THEN The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message of to modify APN, MTU and PLMN rate controls. NOTE: Settings in MODIFY EPS BEARER CONTEXT REQUEST to check APN rate control - APN control set for max 1 message per minute, - MTU parameters are not provided. - PLMN Rate control not provided (ie previous value still apply) | <-- | RRC: DLInformationTransfer NAS: MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 17a2 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer NAS: MODIFY EPS BEARER CONTEXT ACCEPT | 4 | P |
| 17a3 | The SS transmits a CLOSE UE TEST LOOP message to close the UE test loop mode for user data transfer (2 transmission; 0 sec delay). | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 17a4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 17a5 | SS transmits an ESM DATA TRANSPORT message containing downlink user data. | <-- | RRC: DLInformationTransfer ESM DATA TRANSPORT | - | - |
| - | EXCEPTION: Steps 17a6-17a8 are repeated 2 times. Note: The number of repetition is set so that it respects the APN rate controls set in step 17a1. Max of 1 message every minute | - | - | - | - |
| 17a6 | SS starts timer 1 min APN Rate control | - | - | - | - |
| 17a7 | Check: Does the UE send an ESM DATA TRANSPORT message respecting the APN set in step 17a1: Max size of message 256 Octets? | --> | RRC: ULInformationTransfer TC: ESM DATA TRANSPORT | 2,3,4 | P |
| 17a8 | Wait until timer 1 min APN Rate control expires. | - | - | - | - |
| - | EXCEPTION: Steps 18a1-18a6 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports MTU parameters. | - | - | - | - |
| 18a1 | IF pc_NonIP_Link_MTU_Parameter OR pc_IPv4_Link_MTU_Parameter THEN The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message to modify APN, MTU and PLMN rate controls. NOTE: Settings in MODIFY EPS BEARER CONTEXT REQUEST to check MTU parameters - APN control set to unrestricted; - MTU parameters container size limits set to max 128 Octet. - PLMN Rate control not provided. | <-- | RRC: DLInformationTransfer NAS: MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 18a2 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer NAS: MODIFY EPS BEARER CONTEXT ACCEPT | 3 | P |
| 18a3 | The SS transmits a CLOSE UE TEST LOOP message to close the UE test loop mode for user data transfer (1 transmission; 0 sec delay). | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 18a4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 18a5 | SS transmits an ESM DATA TRANSPORT message containing 240 Octets of downlink user data. Note: The size of the user data which is expected to be looped back is deliberately chosen to allow testing of MTU parameters set in step 18a1. | <-- | RRC: DLInformationTransfer ESM DATA TRANSPORT | 4 | P |
| - | EXCEPTION: Step 18a6 is repeated until the sum of the Octets of uplink user data sent in each ESM DATA TRANSPORT message =>240 Octets (the complete downlink user data provided in step 18a5 is looped back). | - | - | - | - |
| 18a6 | Check: Does the UE send an ESM DATA TRANSPORT message respecting each containing user data container of max of 128 Octets (the MTU set in step 18a1)? | --> | RRC: ULInformationTransfer TC: ESM DATA TRANSPORT | 2,3,4 | P |
| 19 | The SS transmits DEACTIVATE TEST MODE message to deactivate the test mode G. | <-- | RRC: DLInformationTransfer TC: DEACTIVATE TEST MODE | - | - |
| 20 | The UE transmits an DEACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: DEACTIVATE TEST MODE COMPLETE | - | - |
| 21 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease | - | - |

23.1.1.3.3	Specific message contents

Table 23.1.1.3.3-1: Message CLOSE UE TEST LOOP (step 1, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7A-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | '00000110'B | UE test loop mode G setup |  |
| Operation mode and repetitions |  |  |  |
| M0 | 0 | data is returned in uplink at the EMM entity |  |
| R6..R0 | '0001100'B | 12 The received DL message in uplink shall be looped back 12 times. |  |
| Uplink data delay | '00111100'B | T_delay_modeG timer=60 sec 0..255 seconds (binary coded, T7 is most significant bit and T0 least significant bit) |  |

Table 23.1.1.3.3-2: Message ESM DATA TRANSPORT (steps 3 and 11, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7.3-12A. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| User data container | '11110000 11110000 11110000'B | 3 Octets of user data - The value is arbitrary chosen |  |
| Release assistance indication | Not present |  |  |

Table 23.1.1.3.3-3: Message ESM DATA TRANSPORT (steps 12, 15, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7.3-12A. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| User data container | '11110000 11110000 11110000'B | 3 Octets of user data - The value is arbitrary chosen |  |
| Release assistance indication | Not present Or Present with DDX='00'B | The messages sent in step 12 and the first message sent in step 15. |  |
|  | Not present Or Present (with DDX='00'B Or DDX='01'B) | The second message sent in step 15. |  |

Table 23.1.1.3.3-4: Message MODIFY EPS BEARER CONTEXT REQUEST (step 17a1, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], Table 4.7.3-18. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | The same value as the value set in the latest ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent prior to this message |  |  |
| Protocol configuration options | Not present |  |  |
| Extended Protocol configuration options |  |  |  |
| Container ID n+X+3 | ‘0016’H | APN rate control support parameters | pc_APN_RateControl |
| Length of container ID n+X+3 contents | 4 |  |  |
| Container ID n+X+3 contents |  | The container identifier contents field contains parameters for APN rate control functionality |  |
| Octet 1 AER+Uplink time unit | '0001'B | - Additional exception reports at maximum rate reached are not allowed - minute (interval) |  |
| Octets 2-4 | '0000001'O | - Max 1 message per minute |  |

Table 23.1.1.3.3-5: Message CLOSE UE TEST LOOP (step 17a3, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7A-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | '00000110'B | UE test loop mode G setup |  |
| Operation mode and repetitions |  |  |  |
| M0 | 0 | data is returned in uplink at the EMM entity |  |
| R6..R0 | '0000010'B | 2 The received DL message in uplink shall be looped back 2 times |  |
| Uplink data delay | '00000000'B | T_delay_modeG timer=0 sec 0..255 seconds (binary coded, T7 is most significant bit and T0 least significant bit) |  |

Table 23.1.1.3.3-6: Message ESM DATA TRANSPORT (step 17a5, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7.3-12A. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| User data container | 10 Octets, not all zeroes | Randomly chosen |  |
| Release assistance indication | Not present |  |  |

Table 23.1.1.3.3-7: Message ESM DATA TRANSPORT (step 17a7, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7.3-12A. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| User data container | The same as the one provided in the relevant DL ESM DATA TRANSPORT message. |  |  |
| Release assistance indication | Not present Or Present with DDX='00'BNot checked | The messages before the last. |  |
|  | Not present Or Present (with DDX='00'B Or DDX='01'B) | The last message |  |

Table 23.1.1.3.3-8: Message MODIFY EPS BEARER CONTEXT REQUEST (step 18a1, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], Table 4.7.3-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | The same value as the value set in the latest ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent prior to this message |  |  |
| Protocol configuration options | Not present |  |  |
| Extended Protocol configuration options |  |  |  |
| Container ID n+X+1 | ‘0010’H | IPv4 Link MTU | pc_IPv4_Link_MTU_Parameter |
| Length of container ID n+X+1 contents | 2 |  |  |
| Container ID n+X+1 contents | '00000000 10000000'B | - 128 octets maximum length of user data container that can be sent in the ESM DATA TRANSPORT message |  |
| Container ID n+X+2 | ‘0015’H | Non-IP Link MTU | pc_NonIP_Link_MTU_Parameter |
| Length of container ID n+X+2 contents | 2 |  |  |
| Container ID n+X+2 contents | '00000000 10000000'B | - 128 octets maximum length of user data container that can be sent in the ESM DATA TRANSPORT message |  |
| Container ID n+X+3 | ‘0016’H | APN rate control support parameters | pc_APN_RateControl |
| Length of container ID n+X+3 contents | 4 |  |  |
| Container ID n+X+3 contents |  | The container identifier contents field contains parameters for APN rate control functionality |  |
| Octet 1 AER+Uplink time unit | '0000'B | - Additional exception reports at maximum rate reached are not allowed - unrestricted (interval) |  |
| Octets 2-4 | 11111111 11111111 11111111'B | - unrestricted Maximum uplink rate (octet 2 to octet 4) is a binary coded representation of the maximum number of messages the UE is restricted to send per time unit. The time unit is indicated in the uplink time unit. If the uplink time unit is set to "unrestricted", the maximum uplink data volume the UE can send is not restricted. |  |

Table 23.1.1.3.3-9: Message CLOSE UE TEST LOOP (steps 18a3, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7A-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | '00000110'B | UE test loop mode G setup |  |
| Operation mode and repetitions |  |  |  |
| M0 | 0 | CP data loopback mode |  |
| R6..R0 | '0000001'B | 1 The received DL message in uplink shall be looped back 1 time (once) |  |
| Uplink data delay | '00000000'B | T_delay_modeG timer=0 sec 0..255 seconds (binary coded, T7 is most significant bit and T0 least significant bit) |  |

Table 23.1.1.3.3-10: Message ESM DATA TRANSPORT (step 18a5, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7.3-12A. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| User data container | 240 Octets, not all zeroes | 240 Octets of user data - The number of Octets is chosen in relation to the MTU parameters set in the TC and shall be obeyed; the value of each octet is not of importance |  |
| Release assistance indication | Not checked |  |  |

Table 23.1.1.3.3-11: Message ESM DATA TRANSPORT (step 18a6, Table 23.1.1.3.2-1)

| Derivation path: TS 36.508 [18], table 4.7.3-12A. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| User data container | Max 128 Octets of data | - The number of Octets is chosen in relation to the MTU parameters set in the TC and shall be obeyed; the value of each octet is not checked. |  |
| Release assistance indication | Not present Or Present with DDX='00'BNot checked | The messages before the last. |  |
|  | Not present Or Present (with DDX='00'B Or DDX='01'B) | The last message |  |